LX30WB

·	Processor:

·	Single socket for 80486 CPUs.

·	Supports Intel 486 SX, DX, DX2, and DX4 processors and AMD 486 DX, DX2, and DX4 processors

·	P24T (Intel Pentium OverDrive) processors require Interposer kit from Intel

·	3.3V processors require a Power Module

·	Expansion:

·	Three 16 bit ISA slots – two 32 bit VLB slots – one 8 bit ISA slot

·	Chipset:

·	Micronics X30 chipset ASIC

·	CMD 640A Local Bus IDE

·	Winbond Super I/O

·	Memory (parity and non-parity):

·	Three 32/36-bit 72-pin SIMM sockets, acoomodating 60ns or 70ns SIMMs.

·	Supports 1M x 32/36 (4 MB), 2M x 32/36 (8 MB), 4M x 32/36 (16 MB), and 8M x 32/36 (32 MB).

·	Maximum memory is 96 MB.

·	Parity and non-parity memory cannot be mixed.

·	Cache:

·	0/128/256K Level 2 cache.

·	PCI IDE:

·	Two IDE connectors on-board

·	Primary (VL) IDE supports Mode 3 and LBA.

·	Secondary (ISA) IDE supports Mode 0 and LBA.

·	Floppy:

·	Supports 360K, 720K, 1.2, 1.44  floppy drives.

·	Serial / Parallel ports:

·	Two on-board 16550 compatible serial ports.

·	One on-board parallel port with ECP and EPP support.

·	BIOS:

·	Phoenix BIOS on 512k Flash chip.

·	Keyboard:

·	Standard AT keyboard connector

·	PS/2 header support

·	Form factor:

·	Baby AT footprint (8.5” x 9.85”)

LX30WB Jumper Settings

CPU speed:

| Jumper | Function | Setting |
| --- | --- | --- |
| W2 | S Series CPU P24D Other (default) | 1-2 2-3 open |
| W4 | P24D Other (default) | closed open |
| W6 | 486 DX4/DX2/DX/SX, P24D, S Series (default) P24T M7 | 4-5 1-2 2-3 |
| W7 | 486SX Other (default) | 2-3 1-2 , 3-4 |
| W8 | 486DX4/Dx2/DX/SX, M7, S Series (default) P24T, P24D | 2-3 1-2 |
| W9 | 486DX4/DX2/DX/SX, P24D, S Series (default) M7 | 1-2 2-3 |
| W10 | 486DX4/DX2/DX/SX, M7, S Series (default) P24D, P24T | 1-2 2-3 |
| W11 | 486DX4/DX2/DX/SX, P24T, S Series (default) P24D M7 | 3-4 1-2 , 3-4 2-3 |
| W12 | 486DX4/DX2/DX/SX, P24D, S Series (default) M7 | 2-3 1-2 |
| W13 | DX4 (3x CLK) DX4 (2.5x CLK) DX4 (2x CLK) | None 1-2 3-4 |
| W14 | 486DX4/DX2/DX, P24D, M7 (default) 486SX P24T | 3-4 none 2-3 |
| W17 | 486DX4/DX2/DX/SX, S Series (default) P24T, P24D M7 | 2-3 1-2 1-2 |
| W18 | 486DX4/DX2/DX/SX, S Series P24T, P24D M7 | 1-2 1-2 2-3 |

CPU speed:

|  | 25 MHz | 33 MHz | 40 MHz | 50 MHz |
| --- | --- | --- | --- | --- |
| W15 | open | close | open | close |
| W16 | open | close | close | open |

Cache Size Selection:

|  | 128K | 256K |
| --- | --- | --- |
| W25 | 1-2 | 2-3 |
| W26 | 1-2 | 2-3 |
| W27 | 1-2 | 2-3 |
| W28 | 1-2 | 2-3 |

VL IDE Interface:

|  | Function | Setting |
| --- | --- | --- |
| # W33 | Enable VL IDE (default) Disable VL IDE | open close |
| # W34 | VL IDE at IRQ 14 (default) VL IDE at IRQ 15 | 1-2 2-3 |

ISA IDE Interface:

|  | Function | Setting |
| --- | --- | --- |
| # W37 | Enable ISA IDE (default) Disable ISA IDE | 2-3 1-2 |
| W41 | ISA IDE at IRQ 14 ISA IDE at IRQ 15 (default) | 2-3 1-2 |

Video Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W32 | Color (default) Mono | closed open |

BIOS Reset Jumper:

| Jumper | Function | Setting |
| --- | --- | --- |
| W20 | Normal operation (default) Clear CMOS RAM settings | 1-2 2-3 |