JP3:External Debug display connector.

JP4:

| JP4 |  |
| --- | --- |
| Open | External Debug Display |
| Short | On Board Debug Display |

S1:Clock Ratio Table

In case the CPU you use is belonged old style - Multiplier type, you may refer to following set up. All the new frequency-fixed CPU are Auto-Detect so that

don’t need to set up S1.

S1

| 1-2 | 3-4 | 5-6 | 7-8 | Ratio |
| --- | --- | --- | --- | --- |
| On | On | On | On | X2 |
| On | On | On | Off | X2.5 |
| On | On | Off | On | X3 |
| On | On | Off | Off | X3.5 |
| On | Off | On | On | X4 |
| On | Off | On | Off | X4.5 |
| On | Off | Off | On | X5 |
| On | Off | Off | Off | X5.5 |
| Off | On | On | On | X6 |
| Off | On | On | Off | X6.5 |
| Off | On | Off | On | X7 |
| Off | On | Off | Off | X7.5 |
| Off | Off | On | On | X8 |
| Off | Off | On | Off | X8.5 |
| Off | Off | Off | On | X9 |

J15:Clear CMOS

| J15 |  |
| --- | --- |
| 1-2 | Normal Operation(Default) |
| 2-3 | For Clearing CMOS Data |

CPU Host Clock Setting:

| JP1 | JP2 | Speed |
| --- | --- | --- |
| On | On | 66MHz |
| On | Off | 100MHz |
| Off | On | 105MHz |
| Off | Off | 133MHz |

J12: USB Header