6ABX2

MAIN BOARD

ATX  FORM  FACTOR

USER'S   MANUAL

( VER : 1.0 )

COPYRIGHT

THIS MANUAL AND THE INFORMATION CONTAINED HEREIN ARE PROTECTED BY COPYRIGHT. ALL RIGHTS RESERVED.

WARNING AND DISCLAIMER

THIS MANUAL  IS DESIGNED TO PROVIDE INFORMATION  ABOUT  THE PENTIUM® II SYSTEM  BOARD.  EVERY EFFORT HAS BEEN MADE TO MAKE THIS MANUAL AS ACCURATE AS POSSIBLE, BUT NO WARRANTY OR FITNESS IS IMPLIED.    ALL THE INFORMATIONS ARE PROVIDED ON AN 'AS IS' BASIS. THE AUTHOR AND HIS CORRESPONDING PUBLISHING COMPANY SHALL  HAVE NEITHER LIABILITY NOR RESPONSIBILITY TO  ANY  PERSON  OR ENTITY WITH  RESPECT TO ANY LOSS OR DAMAGES ARISING  FROM THE INFORMATION  CONTAINED IN THIS MANUAL  OR FROM THE USE OF THE SYSTEM BOARD THAT ACCOMPANIES IT. INFORMATION CONTAINED IN THIS MANUAL IS SUBJECT TO CHANGE WITHOUT NOTICE. THE MANUFACTURER OF THE SYSTEM BOARD WILL NOT BE HELD RESPONSIBLE FOR TECHNICAL OR EDITORIAL  OMISSIONS  MADE HEREIN, NOR FOR THE INCIDENTAL OR CONSEQUENTIAL DAMAGES RESULTING FROM  ITS FURNISHING, PERFORMANCE, FUNCTIONALITY OR USE. SUBSEQUENT CHANGES TO THIS MANUAL WILL BE INCORPORATED INTO THE NEXT EDITION. WE WELCOME ANY SUGGESTION REGARDING THIS MANUAL OR OUR COMPUTER PRODUCTS.

TRADEMARKS

ALL TRADEMARKS ARE THE PROPERTY OF THEIR RESPECTIVE OWNERS.

TABLE OF CONTENTS

CHAPTER 1. INTRODUCTION							                                          1

1-1       OVERVIEW 				                                                               		 1

1-2       UNPACKING 					                  	                                            2

1-3       SPECIFICATIONS					                                                       2

1-4	       QUICK REFERENCE FOR CD SOFTWARE DRIVERS          3

CHAPTER 2. INSTALLATION			 	                                           4

2-1       	LAYOUT REFERENCE				                                              4

2-2       	JUMPER SETTINGS					                                                   5

2-3       DIMM INSTALLATION PROCEDURE	S                  7

2-4       	MEMORY SIZE                         	                                  8

2-5      	CPU RM KIT ASSEMBLING PROCEDURES           9

CHAPTER 3. BIOS SETUP	             	                                    11

3-1      AWARD BIOS CMOS SETUP		                                   11

3-2      STANDARD CMOS SETUP			 	                                     12

3-3      BIOS FEATURES SETUP				                                          13

3-4      CHIPSET FEATURES SETUP 			                                  16

3-5      POWER MANAGEMENT SETUP			                            18

3-6      PNP/PCI CONFIGURATION SETU P		                       20

3-7      INTEGRATED PERIPHERALS		 	                                21

3-8      SUPERVISOR/USER PASSWORD			                           23

3-9      IDE HDD AUTO DETECTION			                                  24

3-10    LOAD SETUP DEFAULTS				                                        26

3-11    SAVE & EXIT SETUP				                                                26

3-12    EXIT WITHOUT SAVING				                                         26

3-13    I/O & MEMORY MAP				                                               27

3-14    TIME & DMA CHANNELS MAP			                             29

3-15    INTERRUPT MAP					                                                     29

3-16    RTC & CMOS RAM MAP				                                        30

CHAPTER 1: INTRODUCTION

1-1 OVERVIEW :

THE I440 BX MAIN BOARD IS DESIGNED WITH  INTEL® 82440BX AGPSET WHICH PROVIDES AN INTEGRATED IDE CONTROLLER WITH TWO HIGH PERFORMANCE IDE INTERFACES FOR UP TO FOUR IDE DEVICES ( HARD DEVICES , CD-ROM DEVICES , ETC ), AND USB ( UNIVERSAL SERIAL BUS ) FEATURES ENHANCE THE OVERALL PE7RFORMANCE AND EXTENSION FOR THIS BOARD.

IT SUPPORTS  INTEL® PENTIUM® II CPUS FAMILY RUNNING AT 233-450 MHZ  SPEED, OR EVEN HIGHER SPEED, WHICH IS IDEAL FOR MS-DOS, WINDOWS, WINDOWS 95, WINDOW NT, NOVELL, OS/2, UNIX…

THE PERFORMANCE, SPEED AND EXTENSIBILITY OF I440BX MAIN BOARD MAKE IT THE PERFECT CHOICE FOR BUILDING A LAN SERVER, A HIGH-END  WORKSTATION OR A MULTI-USER SYSTEM.

1-2 UNPACKING :

THE MAIN BOARD PACKAGE CONTAINS:

* I440BX MAIN BOARD

* MANUAL

* CABLES

* RETENTION MODULES

IF ANY OF THESE ITEMS IS MISSING OR DAMAGED,CONTACT THE DEALER FROM WHOM YOU PURCHASED. LEAVE THE I440BX MAIN BOARD IN ITS ORIGINAL PACKING UNTIL YOU ARE READY TO INSTALL IT.

1-3 SPECIFICATIONS

: 233 - 450 MHZ   INTEL®  PENTIUM® II CPU

CPU

: 3 OF 168-PIN DIMM UP TO 384MB , 3.3V

MEMORY

EXP.SLOT

:  ISA SLOTS X3,  PCI SLOTS X4,  A.G.P. SLOT  X1

CHIPSET

: INTEL® I440 BX CHIPSET

INTEL® 82443BX PCI AND A.G.P CONTROLLER

INTEL® 82371EB I/O BRIDGE

BIOS

: AWARD® FULL PNP (PLUG & PLAY) BIOS

: ON BOARD 2 X PCI IDE DEVICES , 1 X FDC , 2 X

I/O FUNCTION

SERIAL PORTS(16550 FAST COM),1X PARALLEL

PORT DEVICE /EPP/ECP,  USB  CONNECTOR , IR

CONNECTOR

BOARD SIZE

: 30.5 CM X 21 CM.

: COMPLIED WITH APM (ADVANCED POWER

GREEN FUNCTION

MANAGEMENT).

SPECIAL FUNCTION

:  LM78  SUPPORTS HARDWARE

MONITORING  IS  OPTIONAL.

POWER CONNECTOR

:  ATX POWER CONNECTOR

SB-LINK

: CREATIVE® “SB-LINK”

1-4  QUICK REFERENCE FOR CD SOFTWARE DRIVERS

THIS CD CONTAINS DRIVERS AS BELOW:

- MAIN BOARDS: I440BX®, I440EX®, I440LX®, I430TX®, VIA®  VPX, VP3 MAIN BOARDS

- AGP CARDS:  S- 6326 AND T985

- SOLO-1: ESS-SOLO-1 SOUND DRIVER

- GL518SM: CPU VOLTAGE /TEMPERATURE AND FAN SPEED DETECTION SOFTWARE

E.   PCCILLIN: ANTI- VIRUS PROTECTION SOFTWARE

PLEASE READ “INDEX” BEFORE INSTALLING REQUIRED DRIVERS.  “INDEX” OFFERS ALL THE INFORMATION ON THE BELOW FILES.

CHAPTER 2. INSTALLATION

CPUFA1

1

JBAT1

1

2-1 LAYOUT REFERENCE

JRMS1

# SLOT 1

DIMM1

DIMM2

DIMM3

AGP

SLOT

USB1

KB1

I/O

1  6

3  4

BIOS

1

JMDM1

INTEL®

FW82371EB

SYSFA1

J4

IDE2

IDE1

JWOL1

JPC1

1

ISA1

ISA3

ISA2

PCI 3

PCI 2

PCI 1

PCI 4

SPEAKER

RESET

TB-LED

### IDE LED

COM1  COM2

PRINTER

IR1

82443BX

SB-LINK

ATX

UPPER LAYER:  PS2 MOUSE

LOWER LAYER: KEYBOARD

JRMS1

## SUSPEND SW

### KEYLOCK

2-2 JUMPER SETTINGS

- SETTING “CPU SPEED“

SW1: CPU SPEED SELECTOR

| CPU SPEED | CLOCK SPEED | RATIO | SW1 |
| --- | --- | --- | --- |
| 1 2 3 4 1 2 3 4 PENTIUM® II 233MHZ 1 2 3 4 1 2 3 4 ## ON 1 2 3 4 | 66MHZ | 3.5X |  |
| PENTIUM® II 350MHZ | 100MHZ |  |  |
| PENTIUM® II 266MHZ # ON | 66MHZ | 4X |  |
| PENTIUM® II 400MHZ | 100MHZ |  |  |
| PENTIUM® II 300MHZ ON | 66MHZ | 4.5X |  |
| *PENTIUM® II 450 MHZ | 100MHZ |  |  |
| PENTIUM® II 333 MHZ ON | 66MHZ | 5X |  |
| *PENTIUM® II 500 MHZ | 100MHZ |  |  |
| PENTIUM® II 366 MHZ ON | 66MHZ | 5.5X |  |
| *PENTIUM® II 550 MHZ | 100MHZ |  |  |

*PENTIUM®  II  450MHZ / 500 MHZ / 550 MHZ ARE FOR REFERENCE ONLY AS THEY

ARE NOT AVAILABLE WHEN THIS MANUAL IS PUBLISHED.

2. JBAT1: RTC - BATTERY SELECTOR

|  | NORMAL | CLEAR CMOS |
| --- | --- | --- |
| RTC | 1-2 (NORMAL) | 2-3(DEFAULT) |

CUSTOMERS NEED TO CLEAR CMOS, THEN RECONFIGURE

IT IF FORGET PASSWORD FOR BIOS SETUP.

3. JRMS1: ATX POWER SWITCH

THE SYSTEM POWER IS CONTROLLED BY A MOMENTARY SWITCH (WHEN "POWER SWITCH TYPE" IS SET TO MOMENTARY) CONNECTED TO THIS LEAD.  PUSHING THE BUTTON ONCE WILL TURN ON THE SYSTEM AND PUSHING ANOTHER TIME WILL TURN OFF THE SYSTEM.

THE SYSTEM POWER LED SHOWS THE STATUS OF THE SYSTEM'S POWER.     THIS CONNECTION DOES NOT HAVE A FUNCTION WHEN A STANDARD POWER  SUPPLY IS USED.

4. IR1 :  IR (INFRARED) CONNECTOR

| IR CONNECTOR PIN OUT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PIN 1 RX | PIN 2 GND | PIN 3 TX | PIN 4 +5V | PIN 5 RXH | PIN 6 VCC | PIN 7 GND |

NOTE  : IR1 USES THE SAME I/O PORT AS COM2.  THERE IS NO ANY

HARDWARE JUMPER SETTING FOR IRCON/COM2 ON THIS

MAIN BOARD BUT CUSTOMERS NEED TO SET PROPER BIOS

SETTING FOR "IRDA1.0","ASKIR" OR "STANDARD"(DEFAULT)

UNDER  "INFRA RED (IR) FUNCTION"  OF  "INTEGRATED

PERIPHERALS."

5. OTHER JUMPER SETTINGS AND CONNECTORS :

IDE1                   :  PRIMARY IDE CONNECTOR

IDE2                   :  SECONDARY IDE CONNECTOR

J4                        :  FLOPPY DISK CONNECTOR

COM1                :  SERIAL PORT 1 CONNECTOR.

COM2                :  SERIAL PORT 2 CONNECTOR.

KB1                    :  PS/2 KEYBOARD/MOUSE CONNECTOR.

USB1                  : USB (UNIVERSAL SERIAL BUS) CONNECTOR

JMDM1       	      : INTERNAL MODEM RING ON CONNECTOR

JPC1                  : CASE CONNECTOR

CPUFA1            : CPU FAN CONNECTOR

SYSFAN            : SYSTEM, (CASE) FAN CONNECTOR

J5                        : SB-LINK CONNECTOR

JWOL1                     : WAKE ON LAN CONNECTOR

2-3  DIMM MEMORY INSTALLATION PROCEDURES

INSERT THE MODULE AS SHOWN.   DUE TO DIFFERENT NUMBER OF PINS ON EITHER SIDE OF THE BREAKS, THE MODULE WILL ONLY FIT IN THE ORIENTATION AS SHOWN.    DRAM SIMM MODULES HAVE THE SAME PIN CONTACT ON BOTH SIDES.   SDRAM DIMM MODULES HAVE DIFFERENT PIN CONTACTSS ON EACH SIDE AND THEREFORE HAVE A HIGHER PIN DENSITY.

NOTE: DIMM SUPPORTS 3.3V SDRAM ONLY.

20 pins

60 pins

84 pins

DIMM

2-4 MEMORY SIZE

THE MOTHER BOARD SUPPORTS  168-PIN DIMMS OF  8MB, 16MB, 32MB, 64MB, 128MB TO FORM A MEMORY SIZE BETWEEN 8MB TO 384MB.

NOTE: DIMM SUPPORTS 3.3V SDRAM ONLY.  SINCE THERE ARE VARIOU WAYS OF DIMM INSTALLATIONS, THE FOLLOWING IS ONLY FOR REFERENCES ONLY.  THERE ARE STILL OTHER WAYS TO PUT DIMMS ON DIFFERENT SOCKETS.

| TOTAL | DIMM1 DIMM2 DIMM3 |
| --- | --- |
| 8MBYTES | 8MB --- --- |
| 16MBYTES | 8MB 8MB --- |
| 24MBYTES | 8MB 8MB 8MB |
| 32MBYTES | 8MB 8MB 16MB |
| 32MBYTES | 16MB 16MB --- |
| 32MBYTES | 32MB --- --- |
| 40MBYTES | 16MB 16MB 8MB |
| 48MBYTES | 16MB 16MB 16MB |
| 64MBYTES | 64MB --- --- |
| 64MBYTES | 32MB 32MB --- |
| 96MBYTES | 32MB 32MB 32MB |
| 128MBYTES | 64MB 64MB --- |
| 128MBYTES | 128MB --- --- |
| 192MBYTES | 64MB 64MB 64MB |
| 256MBYTES | 128MB 128MB --- |
| 384MBYTES | 128MB 128MB 128MB |

2-5 CPU RM KIT ASSEMBLING PROCEDURE

THE ENCLOSED RM KIT IS THE LATEST MODEL SUITABLE FOR BOTH KLAMATH AND CELERON.  IT IS NOT NECESSARY  TO CHANGE DIFFERENT CPU RM  KITS FOR DIFFERENT CPU MODELS.

- CHECK IF THE FOLLOWING SET OF PIECE PARTS ARE INCLUDED IN YOUR PACKAGE.  4 SEPERATE PIECE PARTS IN TOTAL.

RETENTION MECHANISM (R.M.): 2 PCS

screw

Front side

Back side

Lock for Klamath™

Lock for Celeron™

RM. ATTACH MOUNT (RMAM): 2 PC

2. MAKE SURE POWER IS OFF DURING ASSEMBLY.

3. INSERT THE RMAMS  THROUGH THE BOTTOM OF THE MOTHERBOARD AND ATTACH THEM TO THE RETENTION MECHANISM.

# 6P2BX

5. FASTEN UP THE SCREWS ON THE RETENTION MECHANISM TO TIGHTEN UP RETENION MECHANISM AND RMAM.  CHECK IF ALL THE PIECE PARTS ARE FASTENED TIGHTLY.

- PUT THE CPU IN THE RM KIT. (PUSH THE CPU  INTO THE RM KIT HORIZONTALLY.)

DUE TO DIFFERENT PACKAGES OF CELERON AND KLAMATH, THERE ARE 2 LOCATIONS TO GET THEM LOCKED IN THE RM KIT.   BELOW IS NTOICE TO UNLOCK THE 2 CPUS.

Lock for Klamath

Lock for Celeron

***Klamath™: push the switch on the CPU, and CPU will be loosen from the lock of RM kit.

***Celeron™: use screw driver or any tool alike to draw this and hold.  At the same time,  softly pull one side of the CPU out.     Then, pull the other side and  CPU will be loosen this way.

CHAPTER 3. BIOS SETUP

3-1. AWARD BIOS CMOS SETUP

THE MENU DISPLAYS ALL THE MAJOR SELECTION ITEMS AND ALLOW USER TO SELECT ANY ONE OF SHOWN ITEM. THE SELECTION IS MADE BY MOVING CURSOR( PRESS ANY DIRECTION KEY ) TO THE ITEM AND PRESS <ENTER> KEY. AN ON-LINE HELP MESSAGE IS DISPLAYED AT THE BOTTOM OF THE SCREEN AS CURSOR IS MOVING TO VARIOUS ITEMS WHICH PROVIDES USER BETTER UNDERSTANDING OF EACH FUNCTION. WHEN A SELECTION IS MADE, THE MENU OF SELECTED ITEM WILL APPEAR.  SO THE USER CAN MODIFY ASSOCIATED CONFIGURATION PARAMETERS.

3-2. STANDARD CMOS SETUP

CHOOSE "STANDARD CMOS SETUP" IN THE CMOS SETUP UTILITY MENU. THE STANDARD CMOS SETUP ALLOWS USER TO CONFIGURE SYSTEM SETTING SUCH AS CURRENT DATE AND TIME, TYPE OF HARD DISK DRIVE INSTALLED IN THE SYSTEM, FLOPPY DRIVE TYPE, AND THE TYPE OF DISPLAY MONITOR. MEMORY SIZE IS AUTO DETECTED BY THE BIOS AND DISPLAYED FOR YOUR REFERENCE. WHEN A FIELD IS HIGHLIGHTED (DIRECTION KEYS TO MOVE CURSOR AND <ENTER> KEY TO SELECT). THE ENTRIES IN THE FIELD WILL BE CHANGED BY PRESSING <PAGEDOWN> OR <PAGEUP> KEY OR USER CAN ENTER NEW DATA DIRECTLY FROM THE KEYBOARD.

NOTE: IF HARD DISK PRIMARY MASTER/SLAVE AND SECONDARY   MASTER/SLAVE WERE USED AUTO, THEN THE HARD DISK  SIZE AND MODEL WILL BE AUTO DETECTED ON DISPLAY  DURING POST.

NOTE: THE "HALT ON:"FIELD IS TO DETERMINE WHEN TO HALT THE  SYSTEM BY THE BIOS IF ERROR OCCURRED DURING POST.

3-3. BIOS FEATURES SETUP

SELECT THE "BIOS FEATURES SETUP" OPTION IN THE CMOS SETUP UTILITY MENU ALLOWS USER TO CHANGE SYSTEM RELATED PARAMETERS IN THE DISPLAYED MENU. THIS MENU SHOWS ALL OF THE MANUFACTURER'S DEFAULT VALUES OF I440BX MAIN BOARD. AGAIN, USER CAN MOVE THE CURSOR BY PRESSING DIRECTION KEYS AND <PAGEDOWN> OR <PAGEUP> KEY TO MODIFY THE PARAMETERS, PRESSING [F1] KEY TO DISPLAY HELP MESSAGE OF THE SELECTED ITEM. THIS SETUP PROGRAM ALSO PROVIDE 2 CONVINENT WAYS TO LOAD THE DEFAULT PARAMETER DATA FROM BIOS [F6] OR CMOS [F7] AREA IF SHOWN DATA IS CORRUPTED. THIS PROVIDES THE SYSTEM A CAPABILITY TO RECOVER FROM ANY POSSIBLE ERROR.

ANTI-VIRUS PROTECTION :

THE OPTION IS AN AUTO ANTI-VIRUS PROTECTION, WHICH AUTOMATICALLY SEARCH FOR DISK VIRUS.

ENABLED: THE SYSTEMS WILL START AUTO ANTI-VIRUS PROTECTION.

DISABLED: THE FUNCTION WILL NOT WORK.

CPU EXTERNAL CACHE:

THIS OPTION SELECTS THE TYPE OF CACHING ALGORITHM USED BY BIOS AND THE COMPUTER FOR L2 (EXTERNAL) SECONDARY CACHE MEMORY.  THE SETTINGS ARE ENABLED OR DISABLED.

ENABLED:  ENABLE CACHE

DISABLED: DISABLE CACHE

EXTERNAL CACHE:

THIS OPTION SELECTS THE TYPE OF CACHING ALGORITHM USED BY BIOS AND THE COMPUTER FOR L2 (EXTERNAL) SECONDARY CACHE MEMORY.  THE SETTINGS ARE ENABLED OR DISABLED.

ENABLED:  ENABLE CACHE

QUICK POWER ON SELF TEST:

THIS CATEGORY SPEEDS UP POWER ON SELF TEST. (POST)  AFTER YOU POWER ON THE COMPUTER, IF IT IS SET TO ENABLE, BIOS WILL SHORTEN  OR SKIP SOME CHECK ITEMS DURING POST.

ENABLE :  ENABLE QUICK POST

DISABLED: NORMAL POST

BOOT SEQUENCE:

THIS CATEGORY DETERMINES WHICH DRIVE COMPUTER SEARCHES FIRST  FOR THE DOS (DISK OPERATING SYSTEM). DEFAULT VALUE IS A,C.

A,C: SYSTEM WILL FIRST SEARCH FOR FLOPPY DISK DRIVE THEN

HARD DISK DRIVE.

C,A: SYSTEM WILL FIRST SEARCH FOR HARD DISK DRIVE THEN

FLOPPY DISK DRIVE.

SWAP FLOPPY DRIVE:

THE SWAP FLOPPY DRIVE. DEFAULT VALUE IS DISABLED.  ENABLED:  FLOPPY A&B WILL BE SWAPPED UNDER THE DOS  DISABLED: FLOPPY A&B WILL BE NOT SWAPPED.

BOOT UP FLOPPY SEEK:

DURING POST, BIOS WILL DETERMINE IF THE FLOPPY DISK  DRIVE INSTALLED IS 40 OR 80 TRACKS. 360K TYPE IS 40 TRACKS WHILE 720K, 1.2M  AND 1.44M ARE ALL 80 TRACKS. THE DEFAULT VALUE IS ENABLED.

BOOT UP NUMLOCK STATUS:

THE DEFAULT VALUE IS ON.

ON:  KEYPAD IS NUMBER KEYS.

OFF: KEYPAD IS ARROW KEYS.

BOOT UP SYSTEM SPEED:

IT SELECTS THE DEFAULT SYSTEM SPEED-THE SPEED THAT THE  SYSTEM WILL RUN AT IMMEDIATELY AFTER POWER UP.

HIGH: SET THE SPEED TO HIGH.

LOW:  SET THE SPEED TO LOW.

VIDEO BIOS SHADOW:

IT DETERMINES WHETHER VIDEO BIOS WILL BE COPIED TO RAM,  HOWEVER, IT IS OPTIONAL FROM CHIPSET DESIGN. VIDEO SHADOW WILL INCREASE THE  VIDEO SPEED.

ENABLED :  VIDEO SHADOW IS ENABLED

DISABLED: VIDEO SHADOW IS DISABLED

C8000-CBFFF SHADOW:

CC000-CFFFF SHADOW:

D0000-D3FFF SHADOW:

D4000-D7FFF SHADOW:

D8000-DBFFF SHADOW:

DC000-DFFFF SHADOW:

THESE CATEGORIES DETERMINE WHETHER OPTIONAL ROM WILL BE COPIED TO RAM BY 16K  BYTE OR 32K BYTE PER/UNIT AND THE SIZE DEPENDS ON CHIPSET.

ENABLED :  OPTIONAL SHADOW IS ENABLED.

DISABLED: OPTIONAL SHADOW IS DISABLED.

3-4. CHIPSET FEATURES SETUP

AUTO CONFIGURATION  [THE BIOS WILL AUTOMATICALLY DETECT THE CPU SPEED AND  WILL AUTO-CONFIGURATE THE BUS FREQUENCY, DRAM SPEED, CACHE AND READ/WRITE CYCLE.]

DRAM RAS# PRECHARGE TIME [THE DRAM PRECHARGE TIME BY RAS.]

: 4

: 3 (DEFAULT)

RAS TO CAS DELAY  [CONTROL THE DRAM PAGE MISS AND ROW MISS LEADOFF	 	 TIMING.]

: 2

: 3 (DEFAULT)

DRAM READ BURST (B/E/F)  [THE TIMING USED DEPENDS ON THE TYPE OF DRAM  ON  A PER-BASIS. THE DRAM READ BURST TIMING                                                           ARE CONTROLLED BY REGISTER.]

: X2222

: X3333

: X4444 (DEFAULT)

DRAM WRITE BURST (B/E/F)  [SLOWER RATE MAY BE REQUIRED IN CERTAIN SYSTEM DESIGNS TO SUPPORT LAYOUT WITH LONGER TRACE LENGTH OR  SLOWER DRAM. THE DRAM WRITE BURST TIMING ARE CONTROLLED BY REGISTER.]

: X2222

: X3333

: X4444 (DEFAULT)

SYSTEM BIOS CACHEABLE  [DEFINE WHETHER SYSTEM BIOS AREA CACHEABLE OR NOT.]

:ENABLED

:DISABLED  (DEFAULT)

VIDEO BIOS CACHEABLE  [DEFINE WHETHER VIDEO BIOS AREA CACHEABLE OR NOT.]

:ENABLED

:DISABLED (DEFAULT)

8 BIT I/O RECOVERY TIME:

THIS FIELD DEFINES THE RECOVERY TIME FROM 1 TO 8  FOR 8-BIT I/O.

16 BIT I/O RECOVERY TIME:

TO DEFINE THE RECOVERY TIME FROM 1 TO 4 FOR 16-BIT  I/O.

MEMORY HOLE AT 15M-16M  [THIS FIELD ENABLE A MEMORY HOLE IN MAIN MEMORY  SPACE. CPU CYCLES MATCHING AN ENABLED HOLD ARE  PASSED ON TO PCI. NOTE THAT A SELECTED CAN NOT BE

CHANGED WHILE THE L2 CACHE IS ENABLED.]

:ENABLED

:DISABLED (DEFAULT)

3-5. POWER MANAGEMENT SETUP

User Define

POWER MANAGEMENT:

DISABLED        :  GLOBAL POWER MANAGEMENT WILL BE DISABLED.

USER DEFINE :USERS CAN CONFIGURE THEIR OWN POWER

MANAGEMENT.

MIN.SAVING  :PRE-DEFINE TIMER VALUE ARE USED SUCH THAT ALL

TIMERS ARE IN THEIR MAX . VALUE

MAX.SAVING :  PRE-DEFINE TIMER VALUES ARE USED SUCH THAT

ALL TIMERS ARE IN  THEIR MIN . VALUE.

PM CONTROL BY APM:

NO : SYSTEM BIOS WILL IGNORE APM.

YES : SYSTEM BIOS WILL WAIT FOR APM'S PROMPT

BEFORE IT ENTER ANY PM MODE, E.G. DOZE, STANDBY

OR SUSPEND.

NOTE 1.  IF APM IS INSTALLED, AND THERE IS A TASK RUNNING,

EVEN IF THE TIMER IS TIME OUT, THE APM WILL NOT

PROMPT  THE BIOS TO PUT THE SYSTEM INTO ANY POWER

SAVING MODE!

NOTE 2. IF APM IS NOT INSTALLED, THIS OPTION HAS NO EFFECT.

VIDEO OFF METHOD       :

BLANK SCREEN                 :  THE SYSTEM BIOS WILL ONLY BLANKS

OFF THE SCREEN WHEN DISABLED.

V/H  SYNC+BLANK            : BIOS WILL ALSO TURN OFF THE V/H SYNC

SIGNAL FROM VGA CARD TO MONITOR.

DPMS                                     : DISPLAY POWER MANAGEMENT BY VGA

CARD  SUPPORT.

DOZE MODE	                        : DISABLED , 1 MIN --- 1 HOUR

STANDBY MODE	                : DISABLED , 1 MIN --- 1 HOUR

SUSPEND MODE	                 : DISABLED , 1 MIN --- 1 HOUR

HDD POWER DOWN          : DISABLED , 1 MIN ---15 MIN

WAKE-UP EVENT	               : TO IRQ3, IRQ4 , IRQ8 , IRQ12 CHECK POINT.

ANY ACTIVITY. THE SYSTEM WILL WAKE

UP.

POWER DOWN ACTIVITIES: TO COM PORTS, LPT PORTS AND DRIVE

PORTS	   IRQ3.......IRQ15 CHECK POINT THEN

INTO GREEN FUNCTION.

3-6. PNP / PCI CONFIGURATION SETUP

(1-3) AVAILABLE IRQ    : ( NA, 3, 4, 5, 7, 9, 10, 11, 12, 13, 14, 15)

PCI IRQ ACTIVED BY    : LEVEL

PCI IDE IRQ MAP TO      : PCI-AUTO (PCI-SLOT 1, 2, 3)

PRIMARY IDE INT#       : A (B, C, D)

SECONDARY IDE INT#   : B (C, D, A)

PCI SLOTS ROUTING METHOD:

PCI 1: A, B, C, D

PCI 2: B, C, D, A

PCI 3: C, D, A, B

3-7. INTEGRATED PERIPHERALS

IDE HDD BLOCK MODE [THIS FEATURE ENHANCES HARD DISK PERFORMANCE BY MAKING  MULTI SECTOR TRANSFER, INSTEAD OF ONE SECTOR PER TRANSFER, MOST OF IDE DRIVERS, EXCEPT VERY EARLY DESIGNS ,CAN USE THIS FEATURE.]

:ENABLED (DEFAULT)

:DISABLED

IDE PRIMARY MASTER PIO	[DETECT YOUR PRIMARY MASTER HARD  DISK DEVICE.]

:AUTO (DEFAULT)

:MODE 0,1,2,3,4

IDE PRIMARY SLAVE PIO	[DETECT YOUR PRIMARY SLAVE HARD DISK DEVICE.]

:AUTO (DEFAULT)

:MODE 0,1,2,3,4

IDE SECONDARY MASTER PIO[DETECT YOUR SECONDARY MASTER HARD DISK DEVICE.]

:AUTO (DEFAULT)

:MODE 0,1,2,3,4

IDE SECONDARY SLAVE PIO [DETECT YOUR SECONDARY SLAVE HARD DISK DEVICE.]

: AUTO (DEFAULT)

: MODE 0,1,2,3,4

ON-CHIP PRIMARY PCI IDE [SELECT USE CHIP SUPPORT PRIMARY PCI IDE.]

: ENABLED (DEFAULT)

: DISABLED

ON-CHIP SECONDARY PCI IDE [SELECT USE CHIP SUPPORT SECONDARY PCI IDE.]

: ENABLED (DEFAULT)

: DISABLED

PCI SLOT IDE 2ND CHANNEL [USE EXTERNAL IDE. AS ISA IDE OR PCI IDE.]

: ENABLED (DEFAULT)

: DISABLED

ON-BOARD FDD CONTROLLER : ENABLED (DEFAULT)

: DISABLED

ON-BOARD SERIAL PORT 1	 : COM1  (DEFAULT)

: COM2

: COM3

: COM4

: DISABLED

ON-BOARD SERIAL PORT 2	 : COM1

: COM2  (DEFAULT)

: COM3

: COM4

: DISABLED

ON-BOARD PARALLEL PORT	 : 378H  (DEFAULT)

: 278H

: 3BCH

: DISABLED

ON-BOARD PARALLEL MODE	 : SPP(DEFAULT)

: EPP

: ECP

: ECP+EPP

3-8. SUPERVISOR/USER PASSWORD

THE " SUPERVISOR/USER PASSWORD SETTING " UTILITY SETS THE PASSWORD.  THE MAINBOARD MAY BE SHIPPED WITH THE DEFAULT PASSWORD "AWARD_SW" , OR WITH THE PASSWORD DISABLED.  IF YOU WANT TO CHANGE THE PASSWORD, YOU MUST FIRST ENTER THE CURRENT PASSWORD (" AWARD_SW " IN THIS CASE).     THEN AT THE PROMPT, TYPE YOUR NEW PASSWORD.   THE PASSWORD IS CASE SENSITIVE AND YOU CAN USE UP TO 8 ALPHANUMERIC CHARACTERS.  PRESS <ENTER> AFTER THE PASSWORD .  AT THE NEXT PROMPT, CONFIRM THE NEW PASSWORD BY TYPING IT AND PRESSING <ENTER> AGAIN.  WHEN YOU USE THIS FEATURE, THE " SECURITY OPTION" LINE IN BIOS FEATURES SETUP WILL DETERMINE WHETHER THE PASSWORD WILL BE REQUIRED.      TO DISABLE THE PASSWORD, PRESS THE <ENTER> KEY INSTEAD OF ENTERING A NEW PASSWORD WHEN THE " ENTER PASSWORD" DIALOG BOX APPEARS.  A MESSAGE WILL APPEAR CONFIRMING THAT THE PASSWORD IS DISABLE.  YOU MAY RECEIVE YOUR MAINBOARD SET UP THIS WAY.

THERE ARE TWO KINDS OF PASSWORD FUNCTIONS IN THE SETUP MENU : ONE IS SUPERVISOR PASSWORD, AND THE OTHER IS USER PASSWORD.

THE DIFFERENCES BETWEEN THEM ARE:

SUPERVISOR PASSWORD:      THE SUPERVISOR PASSWORD FUNCTION ALLOWS YOU THE  RIGHT TO CHANGE THE OPTIONS OF SETUP MENU ONCE  YOU ENTER THE SETUP MENU.

USER PASSWORD:     THE USER PASSWORD FUNCTION ONLY ALLOWS YOU TO ENTER THE SETUP MENU BUT DO NOT HAVE THE RIGHT TO CHANGE THE OPTIONS  OF THE SETUP MENU EXCEPT USER PASSWORD, SAVE & EXIT SETUP,   AND EXIT WITHOUT SAVING.

3-9 .IDE HDD AUTO DETECTION

THE "IDE HDD AUTO DETECTION" UTILITY IS A VERY USEFUL TOOL ESPECIALLY WHEN YOU DO NOT KNOW WHICH KIND OF HARD DISK TYPE YOU ARE USING.  YOU CAN USE THIS UTILITY TO DETECT THE CORRECT DISK TYPE INSTALLED IN THE SYSTEM AUTOMATICALLY OR YOU CAN SET HARD DISK TYPE TO AUTO IN THE STANDARD CMOS SETUP. YOU DON'T NEED THE "IDE HDD AUTO DETECTION" UTILITY. THE BIOS WILL AUTO-DETECT THE HARD DISK SIZE AND MODEL ON DISPLAY DURING POST.

NOTE:  HDD MODES

THE AWARD BIOS SUPPORTS 3 HDD MODES: NORMAL, LBA & LARGE

NORMAL MODE

GENERIC ACCESS MODE IN WHICH NEITHER THE BIOS NOR THE IDE CONTROLLER WILL MAKE ANY TRANSFORMATIONS DURING ACCESSING.

THE MAXIMUM NUMBER OF CYLINDERS, HEAD &  SECTORS FOR NORMAL MODE ARE 1024, 16 & 63.

NO. CYLINDER         (1024)

X   NO. HEAD                  (  16)

X   NO. SECTOR              (  63)

X   NO. PER SECTOR      ( 512)

528 MEGABYTES

IF USER SET THIS HDD TO NORMAL MODE, THE MAXIMUM ACCESSIBLE HDD SIZE WILL BE 528 MEGABYTES EVEN THOUGH ITS PHYSICAL SIZE MAY BE GREATER THAN THAT!

LBA (LOGICAL BLOCK ADDRESSING) MODE

A NEW HDD ACCESSING METHOD TO OVERCOME THE 528 MEGABYTE BOTTLENECK. THE NUMBER OF CYLINDERS, HEADS & SECTORS SHOWN IN SETUP MAY NOT BE THE NUMBER PHYSICALLY CONTAINED IN THE HDD.

DURING HDD ACCESSING, THE IDE CONTROLLER WILL TRANSFORM THE LOGICAL ADDRESS DESCRIBED BY SECTOR, HEAD & CYLINDER INTO ITS OWN PHYSICAL ADDRESS INSIDE THE HDD.

THE MAXIMUM HDD SIZE SUPPORTED BY LBA MODE IS 8.4 GIGABYTES WHICH IS OBTAINED BY THE FOLLOWING FORMULA:

NO. CYLINDER                 (1024)

X     NO. HEAD                          ( 255)

X     NO. SECTOR                      (  63)

X     NO. BYTES PER SECTOR    ( 512)                                                                                           8.4 GIGABYTES

LARGE MODE

EXTENDED HDD ACCESS MODE SUPPORTED BY AWARD SOFTWARE.

SOME IDE HDDS CONTAIN MORE THAN 1024 CYLINDER WITHOUT LBA SUPPORT (IN SOME CASES, USER DO NOT WANT LBA).

THE AWARD BIOS PROVIDES ANOTHER ALTERNATIVE TO SUPPORT THESE KINDS OF LARGE MODE:

CYLS.      HEAD    SECTOR      MODE

1120            16             59            NORMAL

560            32            59            LARGE

BIOS TRICKS DOS (OR OTHER OS) THAT THE NUMBER OF CYLINDERS IS LESS THAN 1024 BY DIVIDING IT BY 2. AT THE SAME TIME, THE NUMBER OF HEADS IS MULTIPLIED BY 2. A REVERSE TRANSFORMATION PROCESS WILL BE MADE INSIDE INT 12H IN ORDER TO ACCESS THE RIGHT HDD ADDRESS THE RIGHT HDD ADDRESS!

MAXIMUM HDD SIZE:

NO. CYLINDER                    (1024)

X       NO. HEAD                             (  32)

X       NO. SECTOR                         (  63)

X       NO. BYTES PER SECTOR    ( 512)

1 GIGABYTES

NOTE: TO SUPPORT LBA OR LARGE MODE OF HDDS, THERE MUST

BE SOME SOFTWARES INVOLVED. ALL THESE SOFTWARES

ARE LOCATED IN THE AWARD HDD SERVICE ROUTINE

(INT 13H).  IT MAY BE FAILED TO ACCESS A HDD WITH LBA

(LARGE) MODE SELECTED IF YOU ARE RUNNING UNDER AN

OPERATING SYSTEM WHICH REPLACES THE WHOLE INT 13H.

UNIX OPERATING SYSTEMS DO NOT SUPPORT EITHER LBA

OR LARGE AND MUST UTILITY THE STANDARD MODE. UNIX

CAN SUPPORT DRIVES LARGER THAN 528MB.

3-10 .LOAD SETUP DEFAULTS

"LOAD SETUP DEFAULTS" LOADS OPTIMIZED SETTINGS WHICH ARE STORED IN THE BIOS ROM.   THE AUTO-CONFIGURED SETTINGS ONLY AFFECT THE BIOS FEATURE SETUP AND CHIPSET FEATURES SETUP SCREENS.  THERE IS NO EFFECT ON THE STANDARD CMOS SETUP.  TO USE THIS FEATURE, HIGHLIGHT IT ON THE MAIN SCREEN AND PRESS THE <ENTER> KEY.  A LINE WILL APPEAR ON SCREEN ASKING IF YOU WANT TO LOAD THE SETUP DEFAULT VALUES. PRESS THE <Y> KEY AND THEN PRESS THE <ENTER> KEY .  THE SETUP DEFAULTS  WILL THEN LOAD.  PRESS <N> IF YOU DON'T WANT TO

3-11 SAVE & EXIT SETUP

THE "SAVE & EXIT SETUP" OPTION WILL BRING YOU BACK TO BOOT UP PROCEDURE WITH ALL THE CHANGES, YOU JUST MADE WHICH ARE RECORDED IN THE CMOS RAM.

3-12 EXIT WITHOUT SAVING

THE "EXIT WITHOUT SAVING" OPTION WILL BRING YOU BACK TO NORMAL BOOT UP PROCEDURE WITHOUT SAVING ANY DATA INTO CMOS RAM. ALL OF THE OLD DATA IN THE CMOS WILL NOT BE DESTROYED.

3-13 I/O & MEMORY MAP

MEMORY MAP

| ADDRESS RANGE | SIZE | DESCRIPTION |
| --- | --- | --- |
| 00000-7FFFF | 512K | CONVENTIONAL MEMORY |
| 80000-9FBFF | 127K | EXTENDED CONVENTIONAL MEMORY |
| 9FC00-9FFFF | 1K | EXTENDED BIOS DATA AREA IF PS/2 MOUSE IS INSTALLED |
| A0000-C7FFF | 160K | AVAILABLE FOR HI DOS MEMORY |
| C8000-DFFFF | 96K | AVAILABLE FOR HI DOS MEMORY AND ADAPTER ROMS |
| E0000-EEFFF | 60K | AVAILABLE FOR UMB |
| EF000-EFFFF | 4K | VIDEO SERVICE ROUTINE FOR MONOCHROME & CGA ADAPTER |
| F0000-F7FFF | 32K | BIOS CMOS SETUP UTILITY |
| F8000-FCFFF | 20K | BIOS RUNTIME SERVICE ROUTINE (2) |
| FD000-FDFFF | 4K | PLUG AND PLAY ESCD DATA AREA |
| FE000-FFFFF | 8K | BIOS RUNTIME SERVICE ROUTINE (1) |

I/O MAP

| 000-01F | DMA CONTROLLER (MASTER) |
| --- | --- |
| 020-021 | INTERRUPT CONTROLLER (MASTER) |
| 022-023 | CHIPSET CONTROL REGISTERS. I/O POSTS |
| 040-05F | TIMER CONTROL REGISTERS |
| 060-06F | KEYBOARD INTERFACE CONTROLLER (8042) |
| 070-07F | RTC PORTS & CMOS I/O PORTS |
| 080-09F | DMA REGISTER |
| 0A0-0BF | INTERRUPT CONTROLLER (SLAVE) |
| 0C0-0DF | DMA CONTROLLER (SLAVE) |
| 0F0-0FF | MATH COPROCESSOR |
| 1F0-1FB | HARD DISK CONTROLLER |
| 278-27F | PARALLEL PORT 2 |
| 2B0-2DF | GRAPHICS ADAPTER CONTROLLER |
| 2F8-2FF | SERIAL PORT 2 |
| 360-36F | NETWORK PORTS |
| 378-37F | PARALLEL PORT 1 |
| 3B0-3BF | MONOCHROME & PARALLEL PORT ADAPTER |
| 3C0-3CF | EGA ADAPTER |
| 3D0-CDF | CGA ADAPTER |
| 3F0-3F7 | FLOPPY DISK CONTROLLER |
| 3F8-3FF | SERIAL PORT-1 |

3-14 TIME & DMA CHANNELS MAP

TIME MAP:             TIMER CHANNEL 0 SYSTEM TIMER INTERRUPT                                                          TIMER CHANNEL 1 DRAM REFRESH REQUEST                                                             TIMER CHANNEL 2 SPEAKER TONE GENERATOR

DMA CHANNELS:  DMA CHANNEL 0 AVAILABLE

DMA CHANNEL 1 ONBOARD ECP (OPTION)                                                                  DMA CHANNEL 2 FLOPPY DISK (SMC CHIP)                                                                 DMA CHANNEL 3 ONBOARD ECP (DEFAULT)                                                              DMA CHANNEL 4 CASCADE FOR DMA CONTROLLER 1                                            DMA CHANNEL 5 AVAILABLE

DMA CHANNEL 6 AVAILABLE

DMA CHANNEL 7 AVAILABLE

3-15 INTERRUPT MAP

NIMI:     NON-MASKABLE INTERRUPT

IRQ(H/W): 0 SYSTEM TIMER INTERRUPT FROM TIMER 0

1 KEYBOARD OUTPUT BUFFER FULL

2 CASCADE FOR IRQ8-15

3 SERIAL PORT2

4 SERIAL PORT1

5 PARALLEL PORT 2

6 FLOPPY DISK (SMC CHIP)

7 PARALLEL PORT 1

8 RTC CLOCK

9 AVAILABLE

10 AVAILABLE

11 AVAILABLE

12 PS/2 MOUSE

13 MATH COPROCESSOR

14 ONBOARD HARD DISK (IDE1) CHANNEL

15 ONBOARD HARD DISK (IDE2) CHANNEL

3-16 RTC & CMOS RAM MAP

RTC & CMOS:00     SECONDS

01     SECOND ALARM

02     MINUTES

03     MINUTES ALARM

04     HOURS

05     HOURS ALARM

06     DAY OF WEEK

07     DAY OF MONTH

08     MONTH

09     YEAR

0A     STATUS REGISTER A

0B     STATUS REGISTER B

0C     STATUS REGISTER C

0D     STATUS REGISTER D

0E     DIAGNOSTIC STATUS BYTE

0F     SHUTDOWN BYTE

10     FLOPPY DISK DRIVE TYPE BYTE

12     HARD DISK TYPE BYTE

13     RESERVE

14     EQUIPMENT TYPE

15     BASE MEMORY LOW BYTE

16     BASE MEMORY HIGH BYTE

17     EXTENSION MEMORY LOW BYTE

18     EXTENSION MEMORY HIGH BYTE

19-2D

2E-2F

- RESERVED FOR EXTENSION MEMORY LOW BYTE

31   RESERVED FOR EXTENSION MEMORY HIGH BYTE                              32   DATE CENTURY BYTE

33   INFORMATION FLAG

34-3F  RESERVE

40-7F  RESERVED FOR CHIPSET SETTING DATA

---END---