# Introduction

## Specifications

| System Chipset | Intel 82430 VX chipset. |  |
| --- | --- | --- |
| CPU | One 321-pin socket 7 for Intel Pentium(P54C/CQS/ CS, P55C), AMD 5k86,K5,K6, Cyrix 6x86 (L,MX), IDT C6 processors, support 75/90/100/ 120/133/150/166/200/233MHz. |  |
| Memory | Expandable to 128 MB (4 banks) with four 72-pin SIMM sockets onboard (Support Fast Page Mode and EDO DRAM 6-2-2-2) and two 168-pin DIMM socket (support Syncchronous DRAM module 6-1-1-1). |  |
| Cache | 64-bit 256/512KB L2 Pipeline Burst SRAM onboard. |  |
| I/O | ALi 5113 or SMC669, two high speed 16550 compatible serial ports, one Multi-Mode. Parallel Port support SPP/EPP/ECP mode. Two onboard PCI IDE Ports (32 bit data transfer). Support two 360/720KB/1.2/1.44/2.88MB floppy disk devices. One PS/2 Mouse port. |  |
| BIOS | Award System BIOS installed in socket (Flash and PnP). |  |
| Expansion slots |  | Three PCI Master Slots and four 16-bit ISA Slots. |
| Dimension |  | 4-layer PCB, 2/3 baby size (220mm x 230mm). |
| Others |  | Support USB Bus. |

# Setup Guide

## Layout Diagram

## IV2 QUICK INSTALLATION GUIDE(SMC 669)

| 586 CPU | FS2 | FS1 | FS0 | CLK | BF1 | BF0 | RATE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pentium 75 MHz | 2-3 | 2-3 | 2-3 | 50 | 1-2 | 1-2 | 1.5 |
| Pentium 90 MHz | 2-3 | 2-3 | 1-2 | 60 | 1-2 | 1-2 | 1.5 |
| Pentium 100 MHz | 2-3 | 1-2 | 2-3 | 66 | 1-2 | 1-2 | 1.5 |
| Pentium 120 MHz | 2-3 | 2-3 | 1-2 | 60 | 1-2 | 2-3 | 2 |
| Pentium 133 MHz | 2-3 | 1-2 | 2-3 | 66 | 1-2 | 2-3 | 2 |
| Pentium (MMX) 166 MHz | 2-3 | 1-2 | 2-3 | 66 | 2-3 | 2-3 | 2.5 |
| Pentium (MMX) 200 MHz | 2-3 | 1-2 | 2-3 | 66 | 2-3 | 1-2 | 3 |
| Pentium (MMX) 233 MHz | 2-3 | 1-2 | 2-3 | 66 | 1-2 | 1-2 | 3.5 |
| AMD-5k86-P75-75MHz | 2-3 | 2-3 | 2-3 | 50 | 1-2 | 1-2 | 1.5 |
| AMD-5k86-P90-90MHz | 2-3 | 2-3 | 1-2 | 60 | 1-2 | 1-2 | 1.5 |
| AMD-K5-75MHz -PR75 | 2-3 | 2-3 | 2-3 | 50 | 1-2 | 1-2 | 1.5 |
| AMD-K5-90MHz -PR90 | 2-3 | 2-3 | 1-2 | 60 | 1-2 | 1-2 | 1.5 |
| AMD-K5-100MHz-PR100 | 2-3 | 1-2 | 2-3 | 66 | 1-2 | 1-2 | 1.5 |
| AMD-K5-90MHz-PR120 | 2-3 | 2-3 | 1-2 | 60 | 1-2 | 1-2 | 1.5 |
| AMD-K5-100MHz-PR133 | 2-3 | 1-2 | 2-3 | 66 | 1-2 | 1-2 | 1.5 |
| AMD-K5-115MHz-PR166 | 2-3 | 1-2 | 2-3 | 66 | 2-3 | 2-3 | 1.75 |
| AMD-K6(MMX)-166MHz | 2-3 | 1-2 | 2-3 | 66 | 2-3 | 2-3 | 2.5 |
| AMD-K6(MMX)-200MHz | 2-3 | 1-2 | 2-3 | 66 | 2-3 | 1-2 | 3 |
| Cyrix 6x86-100MHz-P120+ | 2-3 | 2-3 | 2-3 | 50 | 1-2 | 2-3 | 2 |
| Cyrix 6x86-110MHz-P133+ | 1-2 | 2-3 | 2-3 | 55 | 1-2 | 2-3 | 2 |
| Cyrix 6x86-120MHz-P150+ | 2-3 | 2-3 | 1-2 | 60 | 1-2 | 2-3 | 2 |
| Cyrix 6x86-133MHz-P166+ | 2-3 | 1-2 | 2-3 | 66 | 1-2 | 2-3 | 2 |
| Cyrix 6x86-150MHz-P200+ | 1-2 | 2-3 | 1-2 | 75 | 1-2 | 2-3 | 2 |
| Cyrix MX-150MHz-PR166 | 2-3 | 2-3 | 1-2 | 60 | 2-3 | 2-3 | 2.5 |
| Cyrix MX-166MHz-PR200 | 2-3 | 1-2 | 2-3 | 66 | 2-3 | 2-3 | 2.5 |
| Cyrix MX-200MHz-PR233 | 2-3 | 1-2 | 2-3 | 66 | 2-3 | 1-2 | 3 |

## CPU Voltage And Clear CMOS Data

JP5: Clear CMOS

- Turn off the system and short pins to clear CMOS. Then short pins before turning it on. You may damage the mainboard if you power on the system by shorting pins .

| - JP5 |  |
| --- | --- |
| - Open | - Normal operation(Default). |
| - Short | - for clearing CMOS Data. |

- 3.38V for 3.3~3.45V CPU, 3.52V for 3.45~3.52V CPU.

- Dual Voltage for Intel MMX(2.7~2.9V), Cyrix 6x86L(2.63~2.97V), 6x86 MX(2.63~2.97V), AMD K6(2.755~3.045V)CPU.

- For Dual Voltage CPU, JP10=Open.

| Single Voltage CPU |  | 3.38V | 3.45V | Dual Voltage CPU |  | 2.9V |
| --- | --- | --- | --- | --- | --- | --- |
| Intel P54C Cryrix M1 AMD-5k86 / K5 | JP11 |  |  | Intel P55C Cyrix 6x86L 6x86 MX AMD K6 | JP11 |  |
|  | JP12 |  |  |  | JP12 |  |

### Note for Intel CPUs

| 586 CPU | Type | V I/O | Vcore | CLK | RATE |
| --- | --- | --- | --- | --- | --- |
| Pentium-75 | P54C | 3.3 | None | 50 | 1.5 |
| Pentium-90 | P54C | 3.3 | None | 60 | 1.5 |
| Pentium-100 | P54C | 3.3 or 3.52 | None | 66 | 1.5 |
| Pentium-120 | P54C | 3.3 or 3.52 | None | 60 | 2 |
| Pentium-133 | P54C | 3.3 or 3.52 | None | 66 | 2 |
| Pentium-150 | P54C | 3.3 | None | 60 | 2.5 |
| Pentium-166 | P54C | 3.3 | None | 66 | 2.5 |
| Pentium-200 | P54C | 3.3 | None | 66 | 3 |
| Pentium-166(MMX) | P55C | 3.3 | 2.8 | 66 | 2.5 |
| Pentium-200(MMX) | P55C | 3.3 | 2.8 | 66 | 3 |
| Pentium-233(MMX) | P55C | 3.3 | 2.8 | 66 | 3.5 |
| Pentium-266(MMX) | P55C | 3.3 | 2.8 | 66 | 4 |

 The Intel CPUs have six multipliers 1.5/2/2.5/3/3.5/4.

### Note for Cyrix CPUs

| 586 CPU | Dual Voltage | V I/O | Vcore | CLK | Rate |
| --- | --- | --- | --- | --- | --- |
| Cyrix 6x86-P120+ | N | 3.3 or 3.52 | None | 50 | 2 |
| Cyrix 6x86-P133+ | N | 3.3 or 3.52 | None | 55 | 2 |
| Cyrix 6x86-P150+ | N | 3.3 or 3.52 | None | 60 | 2 |
| Cyrix 6x86-P166+ | N | 3.3 or 3.52 | None | 66 | 2 |
| Cyrix 6x86-P200+ | N | 3.3 or 3.52 | None | 75 | 2 |
| Cyrix 6x86L-P150+ | Y | 3.3 or 3.52 | 2.8 | 60 | 2 |
| Cyrix 6x86L-P166+ | Y | 3.3 or 3.52 | 2.8 | 66 | 2 |
| Cyrix 6x86L-P200+ | Y | 3.3 or 3.52 | 2.8 | 75 | 2 |
| Cyrix MX-PR166 | Y | 3.3 | 2.9 | 60 | 2.5 |
| Cyrix MX-PR200 | Y | 3.3 | 2.9 | 66 | 2.5 |
| Cyrix MX-PR233 | Y | 3.3 | 2.9 | 66 | 3 |
| Cyrix MX-PR266 | Y | 3.3 | 2.9 | 66 | 3.5 |

  The Cyrix CPUs have four multipliers 2/2.5/3/3.5.

  The Cyrix 6x86L is a dual voltage CPU without MMX functions. Its Vcore 
     range is 2.63~2.97V.

### Note  for AMD CPUs

| 586 CPU | V I/O | V core | CLK | Rate |
| --- | --- | --- | --- | --- |
| AMD-5K86-P75-75 MHz | 3.52 | None | 50 | 1.5 |
| AMD-5K86-P90-90 MHz | 3.52 | None | 60 | 1.5 |
| AMD-5K86-P100-100MHz MHz | 3.52 | None | 66 | 1.5 |
| AMD-K5-75MHz-PR75 | 3.52 | None | 50 | 1.5 |
| AMD-K5-90MHz-PR90 | 3.52 | None | 60 | 1.5 |
| AMD-K5-100MHz-PR100 | 3.52 | None | 66 | 1.5 |
| AMD-K5-90MHz-PR120 | 3.52 | None | 60 | 1.5 |
| AMD-K5-100MHz-PR133 | 3.52 | None | 66 | 1.5 |
| AMD-K5-116MHz-PR166 | 3.52 | None | 66 | 1.75 |
| AMD-K6(MMX)-166MHz | 3.3 | 2.9 | 66 | 2.5 |
| AMD-K6(MMX)-200MHz | 3.3 | 2.9 | 66 | 3 |
| AMD-K6(MMX)-233MHz | 3.3 | 3.2 | 66 | 3.5 |
| AMD-K6(MMX)-266MHz | 3.3 | 2.2 | 66 | 4 |
| AMD-K6(MMX)-300MHz | 3.3 or 3.45 | 2.2 | 66 | 4.5 |

  The AMD CPUs have nine multipliers 1.5/1.75/2.5/3/3.5/4/4.5/5/5.5.

AMD-K5-PR150  A B Q

Case Temperatures

Q=60’C; R=70’C

Operating Voltages

B=3.45~3.6V (Nominal); C=3.3~3.45 (Nominal);

F=3.1~3.45 (Nominal)

H= (V I/O)/3.3~3.45, (Vcore)/2.86~2.97

J= (V I/O)/3.3~3.45, (Vcore)/2.57~2.84

K= (V I/O)/3.3~3.45, (Vcore)/2.38~2.63

Package Type

A=296-pin SPGA

P-Rating (PR)

75/90/100/120/133/166

Family/Core

AMD-K5

* FOR REFERENCE  ONLY *

## CPU Frequencies

- When a CPU with 75MHz is applied, the PCI Bus CLK output becomes 37MHz. Under this circumstance, some VGA cards may not fit well. Then the system becomes unsteady, tends to hang up easily or even results in boot failure. Use another VGA card instead when any of the above mentioned conditions happens.

| Multipliper Factor for Intel CPU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 |  | 2 |  | 2.5 |  | 3 |  |

| Intel P54C/P55C | X1.5 | X2 | X2.5 | X3 |
| --- | --- | --- | --- | --- |
| Cyrix 6x86(L)/MX | None | X2 | X2.5 | X3 |
| AMD-5k86/K5/K6 | X1.5 | None | X1.75/X2.5 | X3 |

| 50 MHz |  | 55 MHz |  | 60 MHz |  | 66 MHz |  | 75 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PCI Bus Clk 25MHz |  | PCI Bus Clk 27MHz |  | PCI Bus Clk 30 MHz |  | PCI Bus Clk 33 MHz |  | PCI Bus Clk 37 MHz |  |

## DRAM, EDO RAM, Cache Installation Procedures:

- SIMM Socket output voltage is 5V, expandable to 128MB.

- Support 5V Fast Page Mode/ Extended Data Out RAM.

- The BIOS DRAM default setting is 70 ns. Change the BIOS “Chipset Feature Setup” default setting to 60ns for better performance, if the chipset is marked 60ns.

- Change nothing if EDO RAM is used. BIOS automatically detects the RAM type.

- With 586 CPUs, two FPM/EDO RAM Modules are required on SIMM sockets to compose a bank for the system to start.

- Sync SRAM (Pipeline Burst SRAM)

If Sync SRAM Modules are installed, they consists of Pipeline Burst 2 PCs 32K x 32-6 for 256KB or 2 PCs  64Kx32-6)

- Cannot mix up SIMM with DIMM since chips limitation.

## SDRAM Installation  Procedures:

- A 168-pin DIMM can support up to 128MB EDO/SDRAM.

- Default setting: 3.3V.

- First, verify the working voltage of the EDO/SDRAM module in either DIMM socket (DIMM1 or DIMM2-table free).

- You can set up the BIOS “Chipset Feature Setup” to the best working condition basing on the type of EDO/SDRAM you are using.

- The Dual Inline Memory Module (DIMM) must be 3.3 Volt and Unbuffered Synchronus DRAM (SDRAM) 8MB, 16MB, 32MB or 64MB. The following illustration shows the type of DIMM Module.

168-PIN  SDRAM  DIMM Notch Key Definitions

- Cannot mix up SIMM with DIMM since chips limitation.

## Other Jumper Settings

- SMI SW:

- Short to enter sleep mode. A keystroke or mouse movement (mouse driver exits) will instantly “wake up” the system.

- Power LED & Keylock:

- Keyboard lock switch and Power LED connector.

- Speaker:

- Connect to the system’s speaker for beeping.

- Reset:

- Short to restart system.

- HDD LED:

- LED ON when onboard PCI IDE hard disk activates.

- JP4(Flash BIOS Voltage):

| - JP4 |  |
| --- | --- |
| - 1-2 Short | - For 5V Flash BIOS. |
| - 2-3 Short | - For 12V Flash BIOS |