# Introduction

## Specifications

| System Chipset | VIA VT82C580 Apollo VPX chipset. |  |
| --- | --- | --- |
| CPU | One 321-pin socket 7 for Intel Pentium(P54C/CQS/ CS, P55C), AMD 5k86,K5,K6, Cyrix 6x86 (L,MX), IDT C6 processors, support 75/90/100/110 120/133/150/166/180/200/225/233/266/300MHz |  |
| Memory | Expandable to 512MB (2 banks) with two 72-pin SIMM sockets onboard (Support Fast Page Mode and EDO DRAM 5-2-2-2) and two 168-pin DIMM socket (support Syncchronous DRAM module 6-1-1-1). |  |
| Cache | 64-bit 256/ 512KB L2 Pipeline Burst SRAM onboard. |  |
| I/O | SMC 37C669, two high speed 16550 compatible serial ports, one Multi-Mode. Parallel Port support SPP/EPP/ECP standard mode. Two onboard PCI IDE Ports (32 bit data transfer). Support two 360/720KB/1.2/1.44/2.88MB floppy disk devices. One PS/2 Mouse port. |  |
| BIOS | Award System BIOS installed in socket (Flash and PnP). |  |
| Expansion slots |  | Three PCI Master Slots and three 16-bit ISA Slots. |
| Dimension |  | 4-layer PCB, size (220mm x 190mm). |
| Others |  | Support Ultra DMA/33, USB Bus, Keyboard Power On/Off, ATX Power supply. |

# Setup Guide

## Layout Diagram

## Switch Settings for CPUs

On = Short, Off = Open

| Intel, AMD, Cyrix | SW1 |  |  |  | SW1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 586 CPU | 1 | 2 | 3 | RATE | 4 | 5 | 6 | CLK |
| Pentium 75 MHz | Off | Off | Off | 1.5 | On | On | On | 50 |
| Pentium 90 MHz | Off | Off | Off | 1.5 | Off | On | On | 60 |
| Pentium 100 MHz | Off | Off | Off | 1.5 | On | Off | On | 66 |
| Pentium 120 MHz | Off | Off | On | 2 | Off | On | On | 60 |
| Pentium 133 MHz | Off | Off | On | 2 | On | Off | On | 66 |
| Pentium 150 MHz | Off | On | On | 2.5 | Off | On | On | 60 |
| Pentium (MMX) 166 MHz | Off | On | On | 2.5 | On | Off | On | 66 |
| Pentium (MMX) 200 MHz | Off | On | Off | 3 | On | Off | On | 66 |
| Pentium (MMX) 233 MHz | Off | Off | Off | 3.5 | On | Off | On | 66 |
| Pentium (MMX) 266 MHz | On | Off | On | 4 | On | Off | On | 66 |
| AMD-5k86-P75-75MHz | Off | Off | Off | 1.5 | On | On | On | 50 |
| AMD-5k86-P90-90MHz | Off | Off | Off | 1.5 | Off | On | On | 60 |
| AMD-K5-75MHz -PR75 | Off | Off | Off | 1.5 | On | On | On | 50 |
| AMD-K5-90MHz -PR90 | Off | Off | Off | 1.5 | Off | On | On | 60 |
| AMD-K5-100MHz-PR100 | Off | Off | Off | 1.5 | On | Off | On | 66 |
| AMD-K5-90MHz-PR120 | Off | Off | Off | 1.5 | Off | On | On | 60 |
| AMD-K5-100MHz-PR133 | Off | Off | Off | 1.5 | On | Off | On | 66 |
| AMD-K5-133MHz-PR166 | Off | On | On | 1.75 | On | Off | On | 66 |
| AMD-K6(MMX)-166MHz | Off | On | On | 2.5 | On | Off | On | 66 |
| AMD-K6(MMX)-200MHz | Off | On | Off | 3 | On | Off | On | 66 |
| AMD-K6(MMX)-233MHz | Off | Off | Off | 3.5 | On | Off | On | 66 |
| AMD-K6(MMX)-266MHz | On | Off | On | 4 | On | Off | On | 66 |
| AMD-K6(MMX)-300MHz | On | On | On | 4.5 | On | Off | On | 66 |
| Cyrix 6x86-100MHz-P120+ | Off | Off | On | 2 | On | On | On | 50 |
| Cyrix 6x86-110MHz-P133+ | Off | Off | On | 2 | Off | On | On | 55 |
| Cyrix 6x86-120MHz-P150+ | Off | Off | On | 2 | Off | On | On | 60 |
| Cyrix 6x86-133MHz-P166+ | Off | Off | On | 2 | On | Off | On | 66 |
| Cyrix 6x86-150MHz-P200+ | Off | Off | On | 2 | Off | On | Off | 75 |
| Cyrix MII-PR166 | Off | On | On | 2.5 | Off | On | On | 60 |
| Cyrix MII-PR200 | Off | On | On | 2.5 | On | Off | On | 66 |
| Cyrix MII-PR233 | Off | On | On | 2.5 | Off | On | Off | 75 |
| Cyrix MII-PR266 | Off | On | Off | 3 | On | Off | On | 66 |
| Cyrix MII-PR300 | Off | Off | Off | 3.5 | On | Off | On | 66 |

## CPU Voltage Settings

- VI/O default setting : 3.30V Vcore default setting : 2.80V

- JP1=Open, JP2=Short.

- Switch voltage is applied, making the temperature lower and voltage steadier.

- All the voltage specifications adopted here are the averages of the working voltage suggested by the CPU makers, to make any CPU applied work with the best performance.

- In Single voltage CPU VI/O=Vcore.

| JP4 = Single voltage CPU (Intel P54C, Cyrix 6x86, AMD 5k86/K5) |  |  |  |  |  | JP1 | JP2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.52V |  | 3.38V |  |  |  | On | Off |

- In Dual voltage CPU, you only need to set up Vcore. Just “Open” JP1 and “Short” JP2, VI/O will supply 3.3V automatically.

- Remember to make sure CPU voltage set up is 100% correct by referring to Page 11 and Page 12. Any voltage error setup happened in Dual Voltage CPU will cause system unstable or doesn’t work, or even worse is that it will burn out your CPU.

| JP4 = Dual voltage CPU(Intel P55C, Cryrix 6x86L, MX-6x86, AMD-K6) |  |  |  |  |  | JP1 | JP2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vcore3.20V |  | Vcore2.90V |  | Vcore2.80V |  | Off | On |
| Vcore2.20V |  |  |  |  |  |  |  |

## CPU Frequencies

- The CPU type default setting is Intel Pentium 166MHz=66 MHz＊2.5.

- When a CPU with 75MHz is applied, the PCI Bus CLK output becomes 37MHz. Under this circumstance, some VGA cards may not fit well. Then the system becomes unsteady, tends to hang up easily or even results in boot failure. Use another VGA card instead when any of the above-mentioned conditions happens.

| SW1(1/2/3) = Multipliper Factor for Intel / AMD / Cyrix CPU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 |  | 2 |  | 2.5 |  | 3 |  |
| 3.5 |  | 4 |  | 4.5 |  | 5 |  |

| Intel P54C/P55C | X1.5/X3.5 | X2 | X2.5 | X3 |
| --- | --- | --- | --- | --- |
| Cyrix 6x86(L)/MX | None/X3.5 | X2 | X2.5 | X3 |
| AMD-5k86/K5/K6 | X1.5/X3.5 | None | X1.75/X2.5 | X3 |

| SW1(4/5/6) = CPU Ex. Clock Select |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 MHz |  | 55 MHz |  | 60 MHz |  | 66 MHz |  | 75 MHz |  |
| PCI Bus Clk 25MHz |  | PCI Bus Clk 27MHz |  | PCI Bus Clk 30 MHz |  | PCI Bus Clk 33 MHz |  | PCI Bus Clk 37 MHz |  |

## DRAM, EDO RAM Installation Procedures:

- SIMM Socket output voltage is 5V, expandable to 512MB.

- Support 5V Fast Page Mode/ Extended Data Out RAM.

- The BIOS DRAM default setting is 70 ns. Change the BIOS “Chipset Feature Setup” default setting to 60ns for better performance, if the chipset is marked 60ns.

- Change nothing if EDO RAM is used. BIOS automatically detects the RAM type.

- With 586 CPUs, two FPM/EDO RAM Modules are required on SIMM sockets to compose a bank for the system to start.

- In VIA VPX chipset, SIMM and DIMM RAM are able to co-work.

- MEMO for Installing Syatem:

⊕ Concerning memory setup, you can find how to from “Chipset Frature Setup” under BIOS setup. However, to avoid system unstable or system hang, user without engineering background is not suggested to change BIOS set up.

⊕ If system boot failure, please clean SIMM socket (with clean oil) or polish Gold-Finger of DRAM with soft eraser, and try again.

## SDRAM, Cache Memory Installation Procedures:

- Default setting: 3.3V.(Only)

- First, verify the working voltage of the EDO/SDRAM module in either DIMM socket (DIMM1 or DIMM2-table free).

- You can set up the BIOS “Chipset Feature Setup” to the best working condition basing on the type of EDO/SDRAM you are using.

- The Dual Inline Memory Module (DIMM) must be 3.3 Volt and Unbuffered Synchronus DRAM (SDRAM) 8MB, 16MB, 32MB or 64MB. The following illustration shows the type of DIMM Module.

168-PIN  SDRAM  DIMM Notch Key Definitions

Other Jumper Settings

- Power LED & Keylock:

- Keyboard lock switch and Power LED connector.

- Speaker:

Connect to the system’s speaker for beeping.

- Reset:

Short to restart system.

- HDD LED:

LED ON when on board PCI IDE hard disk activates.

- SMI SW:

- Short to enter sleep mode. A keystroke or mouse movement (mouse driver exits) will instantly “wake up” the system.

POWER SW (FOR ATX POWER SUPPLY):

- The button should be a momentary switch that is normally open. Pushing the ATX Power Switch will immediately change the system status. Before or during “POST”, you need to hold the button for four seconds in order to turn off the system.

- JP5: Clear CMOS

- Turn off the system and short pins 2-3 (J3) to clear CMOS. Then short pins 1-2 before turning it on. You may damage the chipset if you power on the system by shorting pins 2-3.

| - JP5 |  |
| --- | --- |
| - 1-2 | - Normal operation(Default). |
| - 2-3 | - for clearing CMOS Data. |

- CPU Cooler Fan connector

- This is the connector for CPU cooler. Never use the jumper to short the connector. Serious damages caused this way will not be warrantied.