This publication, including all photographs, illustrations and software, is protected under international copyright laws, with all rights reserved. Neither this manual, nor any of the material contained herein, may be reproduced without the express written consent of the manufacturer.

The information in this document is subject to change without notice. The manufacturer makes no representations or warranties with respect to the contents hereof and specifically disclaims any implied warranties of merchantability or fitness for any particular purpose. Further, the manufacturer reserves the right to revise this publication and to make changes from time to time in the content hereof without obligation of the manufacturer to notify any person of such revision or changes.

### Trademarks

IBM, VGA, and PS/2 are registered trademarks of International Business Machines.

AMD, Duron and Athlon are registered trademarks of Advanced Micro Devices Inc.

Intel, Pentium/II/III, and MMX are registered trademarks of Intel Corporation.

Microsoft, MS-DOS and Windows 95/98/NT/2000 are registered trademarks of Microsoft Corporation.

PC-cillin and ChipAwayVirus are registered trademarks of Trend Micro Inc.

AMI is a registered trademark of American Megatrends Inc.

A3D is a registered trademark of Aureal Inc.

MediaRing Talk is a registered trademark of MediaRing Inc.

3Deep is a registered trademark of E-Color Inc.

ALi is a registered trademark of Acer Laboratories Inc.

Other names used in this publication may be trademarks and are acknowledged.

Copyright © 2001

All Rights Reserved

M817 Series, V1.3A

A1647X/April 2001

Notice for DDR SDRAM:

The table below lists the model chips of DDR that we have tested. We will test more DDR modules in the future. Users may get this information from our World Wide Web at www.pcchips.com.tw.

| Model Chip | Manufacture |
| --- | --- |
| K4H2808388-TCBO | SAMSUNG |
| HY5DU28822T-H | HYUNDAI |

Table of Contents

Trademarks	I

Chapter 1: Introduction	1

Key Features	2

Package Contents	5

Static Electricity Precautions	6

Pre-Installation Inspection	6

Chapter 2: Mainboard Installation	7

Mainboard Components	8

I/O Ports	8

Install A CPU	9

Install Memory	11

Setting Jumper Switches	12

Install the Mainboard	14

Install the Extension Brackets	15

Optional Extension Brackets	16

Install Other Devices	17

Expansion Slots	19

Chapter 3: BIOS Setup Utility	21

Introduction	21

Running the Setup Utility	22

Standard CMOS Setup Page	23

Advanced Setup Page	24

Power Management Setup Page	26

PCI / Plug and Play Setup Page	27

Load Optimal Settings	28

Load Best Performance Settings	28

Features Setup Page	28

CPU PnP Setup Page	30

Hardware Monitor Page	31

Change Password	31

Exit	32

Chapter 4: Software & Applications	33

Introduction	33

Installing Support Software	33

Auto-installing under Windows 98	36

# Chapter 1

## Introduction

This mainboard has a Socket-462 processor socket for an AMD K7 type CPUs. You can install any one of these processors on the mainboard.

The mainboard supports Socket-462 processor front-side bus speeds of 200MHz or 266MHz.

This mainboard uses the ALI M1647 chipset which supports a 4X AGP slot for highly graphics display, DDR interface and Ultra DMA 33/66/100 function, provides outstanding high system performance under all types of system operations. The mainboard has a built-in AC97 Codec, provides an AMR (Audio Modem Riser) slot to support Audio and Modem application, and has a built-in 10BaseT/100BaseTX Network Interface. In addition, the mainboard has an extended set of ATX I/O Ports including PS/2 keyboard and mouse ports, two USB ports, a parallel port, and two serial ports. Two extra USB ports can be added using the Extended USB Module that connects to the mainboard.

This mainboard has all the features you need to develop a powerful multimedia workstation that is network ready. The board is ATX size and has power connectors for an ATX power supply.

### Key Features

The key features of this mainboard include:

#### Socket-462 Processor Support

- Supports AMD Athlon/Duron processors

- Supports 200/266 MHz Front-Side Bus

#### Memory Support

- Two 168-pin DIMM slots for SDRAM memory modules

- Two 184-pin DIMM slots for DDR memory modules

- Support for 100/133 MHz memory bus

- Maximum installed memory is 1GB

Notice: You can NOT use SDRAM and DDR simultaneously.

#### Expansion Slots

- One AMR slot for a special audio/modem riser card

- One AGP4X slot for AGP 2.0-compliant interface

- Four 32-bit PCI slots for PCI 2.2-compliant bus interface

#### Onboard IDE channels

- Primary and Secondary PCI IDE channels

- Support for PIO (programmable input/output) modes

- Support for Multiword DMA modes

- Support for Bus Mastering and Ultra DMA 33/66/100 modes

#### Power Supply and Power Management

- ATX power supply connector

- ACPI and previous PMU support, suspend switch, keyboard power on/off

- Supports Wake on Modem, Wake on LAN and Wake on Alarm

#### AC97 Codec

- Compliant AC97 2.1 specification

- Supports 18-bit ADC (Analog Digital Converter) and DAC (Digital Analog Converter) as well as 18-bit stereo full-duplex codec

#### Onboard I/O Ports

- Provides PC99 Color Connectors for easy peripheral device connections

- Floppy disk drive connector with 1Mb/s transfer rate

- Two serial ports with 16550-compatible fast UART

- One parallel port with ECP and EPP support

- Two USB ports and optional two USB ports module

- Two PS/2 ports for keyboard and mouse

- One infrared port connector for optional module

#### Hardware Monitoring

- Built-in hardware monitoring for CPU & System temperatures, fan speeds and mainboard voltages

#### Built-in Ethernet LAN

- Built-in 10BaseT/100BaseTX Ethernet LAN

- LAN controller integrates Fast Ethernet MAC and PHY compliant with IEEE802.3u 100BASE-TX, 10BASE-T and ANSI X3.263 TP-PMD standards

- Compliant with ACPI 1.0 and the Network Device Class Power Management 1.0

- High Performance provided by 100Mbps clock generator and data recovery circuit for 100Mbps receiver

#### Onboard Flash ROM

- Supports Plug and Play configuration of peripheral devices and expansion cards

- Built-in virus protection using Trend’s ChipAwayVirus provides boot process virus protection.

#### Bundled Software

- PC-Cillin2000 provides automatic virus protection under Windows 95/98/NT/2000

- MediaRing Talk provides PC to PC or PC to Phone internet phone communication

- 3Deep delivers the precise imagery and displays accurate color in your monitor

- WinDVD2000 is a DVD playback application (optional)

#### Dimensions

- ATX form factor (30.5cm x 22cm)

### Package Contents

Attention: This mainboard series includes four different models. They are M817LMR (LAN/Modem Ready), M817LR (LAN Ready), M817MR (Modem Ready) and M817 (without Modem).

Please contact your local supplier for your purchase model.

Each model will support different specification, list as below:

| Model | Specification |
| --- | --- |
| M817LMR | Onboard LAN chip (U21), LAN header, LAN extension bracket and an AMR Fax/Modem card |
| M817LR | Onboard LAN chip (U21), LAN header and LAN extension bracket |
| M817MR | Provides an AMR Fax/Modem card |
| M817 | --- |

Your mainboard package ships with the following items:

- The mainboard

- This User’s Guide

- 1 UDMA/66 IDE cable

- 1 Floppy disk drive cable

- Support software on CD-ROM disk

#### Optional Accessories

You can purchase the following optional accessories for this mainboard.

- Extended USB module

### Static Electricity Precautions

Components on this mainboard can be damaged by static electricity. Take the following precautions when unpacking the mainboard and installing it in a system.

- Keep the mainboard and other components in their original static-proof packaging until you are ready to install them.

- During installation, wear a grounded wrist strap if possible. If you don’t have a wrist strap, discharge static electricity by touching the bare metal of the system chassis.

- Handle the mainboard carefully by the edges. Avoid touching the components unless it is absolutely necessary. During installation put the mainboard on top of the static-protection packaging it came in with the component side facing up.

### Pre-Installation Inspection

- Inspect the mainboard for damage to the components and connectors on the board.

- If you suspect that the mainboard has been damaged, do not connect power to the system. Contact your mainboard vendor and report the damage.

# Chapter 2

## Mainboard Installation

To install this mainboard in a system, follow the procedures in this chapter:

Identify the mainboard components

Install a CPU

Install one or more system memory modules

Verify that any jumpers or switches are set correctly

Install the mainboard in a system chassis (case)

Connect any extension brackets or cables to the mainboard connector headers

Install any other devices and make the appropriate connections to the mainboard connector headers.

Note:

- Before installing this mainboard, make sure jumper JP5 set to Normal setting. See this chapter for information on locating JP5 and the setting options.

- Never connect power to the system during installation. Doing so may damage the mainboard.

### Mainboard Components

Use the diagram below to identify the major components on the mainboard.

Note: Any jumper on your mainboard that do not appear in the illustration above is for testing only.

### I/O Ports

The illustration below shows a side view of the built-in I/O ports on the mainboard.

### Install A CPU

This mainboard has a Socket-462 CPU socket for AMD K7 processors.

To ensure reliability, ensure that your processor has a heatsink/cooling fan assembly.

Do not try to install a Socket-370/Socket-7 processor in the Socket-462. A Socket-370/Socket-7 processor such as the PPGA Celeron, FCPGA Pentium-III, Pentium-MMX, or the AMD K5/K6 does not fit in the Socket-462.

The following list notes the processors that are currently supported by this mainboard.

Athlon: 700 MHz~1.3 GHz, FSB: 200 MHz, 266 MHz

Duron: 600~950 MHz, FSB: 200 MHz

#### Installing a Socket-462 Processor

A processor installs into the ZIF (Zero Insertion Force) Socket-462 on the mainboard.

- Locate the Socket-462 and CPUFAN1. Pull the locking lever out slightly from the socket and raise it to the upright position.

Socket-462

Pin-1 Corner

CPUFAN1

- On the processor, identify the Pin-1 corner by its beveled edge.

- On the Socket-462, identify the Pin-1 corner. The Pin-1 corner is at the top of the locking lever when it locked.

- Match the Pin-1 corners and insert the processor into the socket. No force is required and the processor should drop into place freely.

- Swing the locking lever down and hook it under the catch on the side of the socket. This secures the CPU in the socket.

- All processors should be installed with a combination heatsink/cooling fan (the original fan is recommended, the others’ fan is not), connect the cable from the fan to the CPU fan power connector CPUFAN1.

See the Setting Jumper Switches section for detail information on CPU System Bus settings.

### Install Memory

The mainboard has two 168-pin/184-pin DIMM sockets for SDRAM/DDR SDRAM (Double Data Rate SDRAM) system memory modules. You must install at least one memory module in order to use the mainboard, and you can only use one of the both SDRAM and DDR SDRAM at the same time.

DDR SDRAM provides 800 MBps or 1 GBps data transfer depending on whether the bus is 100 MHz or 133 MHz. It doubles the rate to 1.6 GBps and 2.1 GBps by transferring data on both the rising and falling edges of the clock. DDR SDRAM uses additional power and ground lines and requires 184-pin 2.5V unbuffered DIMM module reather than the 168-pin 3.3V unbuffered DIMMs used by SDRAM.

DIMM1

DIMM2

DDR1

DDR2

For this mainboard, the maximum memory size is 1GB.

The edge connectors on the memory modules have cut outs, which coincide with spacers in the DIMM sockets so that memory module can only be installed in the correct orientation.

To install a module, push the retaining latches at either end of the socket outwards. Position the memory module correctly and insert it into the DIMM socket. Press the module down into the socket so that the retaining latches rotate up and secure the module in place by fitting into notches on the edge of the module. 
The table below lists the model chips of DDR that we have tested.

| Model Chip | Manufacture |
| --- | --- |
| K4H2808388-TCBO | SAMSUNG |
| HY5DU28822T-H | HYUNDAI |

See the next section for detail information on SDRAM Type and RAM Voltage settings.

### Setting Jumper Switches

Jumpers are sets of pins which can be connected together with jumper caps. The jumper caps change the way the mainboard operates by changing the electronic circuits on the mainboard. If a jumper cap connects two pins, we say the pins are SHORT. If a jumper cap is removed from two pins, the pins are OPEN.

JP5

1

1

JP3

JP6

1

JP2

JP1

JP2A

1

#### Jumper J5~J14: SDRAM Type Selector

The ten jumpers all consist of eight sets of 2-pin jumpers. Use these jumpers to select the SDRAM type for the system memory.

| Function | Jumper Setting |
| --- | --- |
| DDR DIMM socket | Short all Pins 1-2 |
| SDRAM DIMM socket | Open all Pins 1-2 |

#### Jumper JP1: RAM Voltage Selector

Use this 2-pin jumper to select the voltage for the installed SDRAM module.

| Function | Jumper Setting |
| --- | --- |
| 3.3V for SDRAM | Short Pins 1-2 |
| 2.5V for DDR | Open Pins 1-2 |

#### Jumper JP2: Codec Selector

Use this jumper to select the onboard audio codec or Audio Modem Riser (AMR) slot.

| Function | Jumper Setting |
| --- | --- |
| Primary codec onboard | Short Pins 1-2 |
| Primary codec on AMR slot | Open Pins 1-2 |

#### Jumper JP2A: CPU System Bus Selector

Use this 3-pin jumper to select if the processor runs on 133 MHz or 100 MHz system bus.

| Function | JP2A |
| --- | --- |
| 133 MHz system bus | Short Pins 1-2 |
| 100 MHz system bus | Short Pins 2-3 |

#### Jumper JP3: Keyboard Power On Selector

If you enable the keyboard power on feature, you can use hot keys on your keyboard as a power on/off switch for the system.

Note: The system must provide 1A on the +5VSB (+5V Standby) signal before using the Keyboard Power On function.

| Function | Jumper Setting |
| --- | --- |
| Disable Keyboard Power On | Short Pins 1-2 |
| Enable Keyboard Power On | Short Pins 2-3 |

#### Jumper JP5: Clear CMOS Memory

Use this jumper to clear the contents of the CMOS memory. You may need to clear the CMOS memory if the settings in the Setup Utility are incorrect and prevent your mainboard from operating. To clear the CMOS memory, disconnect all the power cables from the mainboard and then move the jumper cap into the CLEAR setting for a few seconds.

| Function | Jumper Setting |
| --- | --- |
| Normal Operation | Short Pins 1-2 |
| Clear CMOS Memory | Short Pins 2-3 |

#### Jumper JP6: Onboard LAN Selector

Use this 3-pin jumper to enable or disable the onboard network adapter.

| Function | Jumper Setting |
| --- | --- |
| Enable Onboard LAN | Short Pins 1-2 |
| Disable Onboard LAN | Short Pins 2-3 |

### Install the Mainboard

Install the mainboard in a system chassis (case). The board is an ATX size mainboard with a twin-tier of I/O ports. You can install this mainboard in an ATX case. Ensure that your case has an I/O cover plate that matches the ports on this mainboard.

Install the mainboard in a case. Follow the instructions provided by the case manufacturer using the hardware and internal mounting points on the chassis.

AUDIO1

SPK1

1

SW1

ATX1

SYSTEMFAN1

Connect the power connector from the power supply to the ATX1 connector on the mainboard.

If there is a cooling fan installed in the system chassis, connect the cable from the cooling fan to the SYSTEMFAN1 fan power connector on the mainboard.

Connect the cable from the PC speaker to the SPK1 header on the mainboard.

Connect the case switches and indicator LEDs to the SW1 header. If there are a headphone jack or/and a microphone jack on the front panel, connect the cables to the AUDIO1 header on the mainboard. See the illustrations below for the guide to the SW1 and AUDIO1 headers pin assignments.

HDD LED P1-3

AUDIO1

MIC 1

MIC-P 3

FPOUT-R 5

NC 7

FPOUT-L 9

2 GND

4 VCC

6 GND

8 Key

10 GND

1  2

Power/ACPI LED P2-4

Reset Switch P5-7

Power Button P6-8

SW1

### Install the Extension Brackets

The extension brackets are used to connect features on the mainboard to external connectors that can be attached to the system chassis. Follow the steps below to install the extension brackets.

Note: All the ribbon cables used on the extension brackets have a red stripe on the Pin-1 side of the cable.

#### LAN Adapter Extension Bracket

This bracket supports an RJ45 network connector and connects to the built in LAN header on the mainboard.

1

LAN Header

LAN Extension Bracket

- On the mainboard, locate the LAN header for this bracket.

- Plug the cable from the bracket into the LAN header.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

### Optional Extension Brackets

For this mainboard, you can also obtain a USB module extension bracket. Install them by following the steps below.

Note: All the ribbon cables used on the extension brackets have a red stripe on the Pin-1 side of the cable.

#### Extended USB Module

This module bracket has two USB ports for more USB devices (USB port 3-4).

USB2 Header

1

- Locate the USB2 header on the mainboard.

- Plug the bracket cable onto the USB2 header.

- In the system chassis, remove a slot cover from one of the expansion slots and install the extension bracket in the opening. Use the screw that held the slot cover in place to secure the extension bracket to the chassis.

### Install Other Devices

Install and connect any other devices in the system following the steps below.

IDE1

1

1

IDE2

FLOPPY1

1

#### Floppy Disk Drive

The mainboard ships with a floppy disk drive cable that can support one or two drives. Drives can be 3.5” or 5.25” wide, with capacities of 360K, 720K, 1.2MB, 1.44MB, or 2.88MB.

Install your drives and connect power from the system power supply. Use the cable provided to connect the drives to the floppy disk drive connector FLOPPY1.

#### IDE Devices

IDE devices include hard disk drives, high-density diskette drives, and CD-ROM or DVD-ROM drives, among others.

The mainboard ships with an IDE cable that can support one or two IDE devices. If you connect two devices to a single cable, you must configure one of the drives as Master and one of the drives as Slave. The documentation of the IDE device will tell you how to configure the device as a Master or Slave device. The Master device connects to the end of the cable.

Install the device(s) and connect power from the system power supply. Use the cable provided to connect the device(s) to the Primary IDE channel connector IDE1 on the mainboard.

If you want to install more IDE devices, you can purchase a second IDE cable and connect one or two devices to the Secondary IDE channel connector IDE2 on the mainboard. If you have two devices on the cable, one must be Master and one must be Slave.

#### Internal Sound Connections

If you have installed a CD-ROM drive or DVD-ROM drive, you can connect the drive audio cable to the onboard sound system.

On the mainboard, locate the two 4-pin connectors CD1 and CD2. There are two kinds of connector because different brands of CD-ROM drive have different kinds of audio cable connectors. Connect the cable to the appropriate connector.

1

+

+

1

CD1

CD2

J15

IR1

FIR 1

+5V 3

IRTX 5

2 Key

4 GND

6 IRRX

#### Infrared Port

You can connect an infrared port to the mainboard. You can purchase this option from third-party vendors.

- Locate the infrared port IR1 header on the mainboard.

- If you are adding an infrared port, connect the ribbon cable from the port to the IR1 header and then secure the port to an appropriate place in your system chassis.

#### Onboard LAN LED Connections

If you have a set indicator LEDs for the onboard LAN communication, you can connect the LED cable to the header J15.

Pins 1-2 are for Link LED. Pins 3-4 are for 10/100 Mbps mode LED, the onboard LAN run in 100 Mbps mode when the LED lit.

### Expansion Slots

This mainboard has four 32-bit PCI slots, one AGP slot and one AMR slot.

PCI1

PCI3

AMR1

AGP1

PCI2

PCI4

Follow the steps below to install a PCI/AGP/AMR expansion card.

- Locate the AMR, AGP or PCI slots on the mainboard.

- Remove the slot cover for this slot from the system chassis.

- Insert the expansion card edge connector into the slot and press it firmly down into it so that it is fully inserted.

- Secure the expansion card bracket to the system chassis using the screw that held the slot cover in place.

#### AMR Slot

The AMR (Audio Modem Riser) slot is an industry standard slot that allows for the installation of a special audio/modem riser card. Different territories have different regulations regarding the specifications of a modem card. You can purchase an AMR card that is approved in your area and install it directly into the AMR slot.

#### Wake On LAN (WOL)

If you have installed a LAN adapter expansion card, connect the card to the Wake On LAN connector WOL1. This allows incoming traffic to resume the system from a software power down. You need to enable this feature in the BIOS setup utility.

WOL1 Header

WOM1 Header

#### Wake On Modem

If you have installed a fax/modem card, connect the fax/modem to the Wake On Modem connector WOM1. You can then use the setup utility to program your computer to resume from a power saving mode whenever there is an incoming call to the fax/modem.

# Chapter 3

## BIOS Setup Utility

### Introduction

The BIOS Setup Utility records settings and information about your computer such as the date and time, the kind of hardware installed, and various configuration settings. Your computer uses this information to initialize all the components when booting up and functions as the basis for coordination between system components.

If the Setup Utility configuration is incorrect, it may cause the system to malfunction. It can even stop your computer from booting properly. If this happens, you can use the clear CMOS jumper to clear the CMOS memory used to store the configuration information.

You can run the setup utility and manually make changes to the configuration. You might need to do this to configure some of the hardware that you install on or connect to the mainboard, such as the CPU, system memory, disk drives, etc.

### Running the Setup Utility

Each time your computer starts, before the operating system loads, a message appears on the screen that prompts you to “Hit <DEL> if you want to run SETUP”. When you see this message, press the Delete key and the Main menu page of the Setup Utility appears on your monitor.

AMIBIOS SIMPLE SETUP UTILITY – VERSION 1.21.01 ©2000 American Megatrends, Inc. All Rights Reserved

Standard CMOS Setup Features Setup Advanced Setup CPU PnP Setup Power Management Setup Hardware Monitor PCI / Plug and Play Setup Change Password Load Optimal Settings Exit Load Best Performance Settings ESC: Quit  : Select Item (Shift)F2 : Change Color F5 : Old Values F6: Optimal values F7: Best performance values F10: Save&Exit

You can use the cursor arrow keys to highlight any of the options on the main menu page. Press Enter to select the highlighted option. To leave the setup utility, press the Escape key. To cycle through the Setup Utility’s optional color schemes hold down the Shift key and press F2.

Some of the options on the main menu page lead to tables of items with installed values. In these pages, use the cursor arrow keys to highlight the items, and then use the PgUp and PgDn keys to cycle through the alternate values for each of the items. Other options on the main menu page lead to dialog boxes which require you to answer Yes or No by hitting the Y or N keys.

If you have already made changes to the setup utility, press F10 to save those changes and exit the utility. Press F5 to reset the changes to the original values. Press F6 to install the setup utility with a set of default values. Press F7 to install the setup utility with a set of high-performance values.

### Standard CMOS Setup Page

Use this page to set basic information such as the date, the time, the IDE devices, and the diskette drives. If you press the F3 key, the system will automatically detect and configure the hard disks on the IDE channels.

AMIBIOS SETUP – STANDARD CMOS SETUP ©2000 American Megatrends, Inc. All Rights Reserved

Date (mm/dd/yy) : Tue Mar 27, 2001 Time (hh/mm/ss) : 14:26:53 LBA Blk PIO 32Bit Type Size Cyln Head WPcom Sec Mode Mode Mode Mode Pri Master : Auto On Pri Slave : Auto On Sec Master : Auto On Sec Slave : Auto On Floppy Drive A : 1.44MB 3 1/2” Floppy Drive B : Not Installed

Month : Jan – Dec ESC : Exit Day : 01 – 31  : Select Item Year : 1901 – 2099 PU/PD/+/- : Modify (Shift)F2 : Color F3 : Detect All HDD

| Date & Time | Use these items to set the system date and time |
| --- | --- |
| Pri Master Pri Slave Sec Master Sec Slave | Use these items to configure devices connected to the Primary and Secondary IDE channels. To configure an IDE hard disk drive, choose Auto. If the Auto setting fails to find a hard disk drive, set it to User, and then fill in the hard disk characteristics (Size, Cyls, etc.) manually. If you have a CD-ROM drive, select the setting CDROM. If you have an ATAPI device with removable media (e.g. a ZIP drive or an LS-120) select Floptical. |
| Floppy Drive A Floppy Drive B | Use these items to set the size and capacity of the floppy diskette drive(s) installed in the system. |

### Advanced Setup Page

Use this page to set more advanced information about your system. Take some care with this page. Making changes can affect the operation of your computer.

| AMIBIOS SETUP – ADVANCED SETUP (C) 2000 American Megatrends, Inc. All Rights Reserved |  |
| --- | --- |
| Trend ChipAway Virus Disabled Quick Boot Enabled 1st Boot Device IDE-0 2nd Boot Device Floppy 3rd Boot Device CDROM Try Other Boot Devices Yes S.M.A.R.T. for Hard Disks Disabled PS/2 Mouse Support Enabled Boot To OS/2 > 64MB No L1 Cache Enabled L2 Cache Enabled DRAM Time/Freq. by SPD Enabled DDR/SDR CAS Select 2.5/3 Dram Performance Normal AGP Delay Offset Auto AGP Driving Strength Auto |  |
|  | ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values |

| Trend ChipAway Virus | This mainboard has built-in virus protection in the BIOS. Use this item to enable or disable the built-in virus protection. If the OS (like Windows 98SE Japanese Version) needs to run EMM386.EXE and this item is enable, please add the NOEMS parameter (e.g.: EMM386 NOEMS) on it. |
| --- | --- |
| Quick Boot | If you enable this item, the system starts up more quickly be elimination some of the power on test routines. |
| 1st Boot Device 2nd Boot Device 3rd Boot Device | Use these items to determine the device order the computer uses to look for an operating system to load at start-up time. |
| Try Other Boot Device | If you enable this item, the system will also search for other boot devices if it fails to find an operating system from the first two locations. |
| S.M.A.R.T. for Hard Disks | Enable this item if any IDE hard disks support the S.M.A.R.T. (Self-Monitoring, Analysis and Reporting Technology) feature. |
| PS/2 Mouse Support | If this item is set to Enabled, the onboard PS/2 Mouse port will work. Setting this to Disable turns off the port. |
| Boot to OS/2 > 64MB | Enable this item if you are booting the OS/2 operating system and you have more than 64MB of system memory installed. |
| L1/L2 Cache | Leave these items enabled since all the processors that can be installed on this board have internal cache memory. |
| DRAM Time/ Freq. by SPD | This item allows you to enable or disable the DRAM timing/Frequency defined by the Serial Presence Detect electrical. |
| DDR/SDR CAS Select | If the item DRAM Time/Freq. by SPD is disabled, you can use this item to determine the operation of the DDR/SDRAM memory CAS (column address strobe). We recommend that you leave this item at the default value. |
| Dram Performance | If the item DRAM Time/Freq. by SPD is disabled, you can use this item to select the performance of the installed SDRAM. We recommend that you leave this item at the default value. |
| AGP Delay Offset | This item can be used to offset driving current on the delay of AGP cards. Some AGP cards need delaying than normal driving current in order to operate. We recommend that you set this item to Auto by default. |
| AGP Driving Strength | This item can be used to signal driving current on AGP cards to auto or Manual. Some AGP cards need stronger than normal driving current in order to operate. We recommend that you set this item to Auto by default. |

### Power Management Setup Page

This page sets some of the parameters for system power management operation.

| AMIBIOS SETUP – POWER MANAGEMENT SETUP (C) 2000 American Megatrends, Inc. All Rights Reserved |  |
| --- | --- |
| Power Management Disabled Ring On Power On Disabled Resume On PME# Disabled RTC Alarm Power On Disabled RTC Alarm Date Every Day RTC Alarm Hour 12 RTC Alarm Minute 30 RTC Alarm Second 30 KeyBoard PowerOn Disabled HotKey Select N/A |  |
|  | ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values |

| Power Management | Use this item to select a power management scheme. Both APM and ACPI are supported. |
| --- | --- |
| Ring On Power On | The system can be turned off with a software command. If you enable this item, the system can automatically resume if there is an incoming call on the Fax/Modem. You must use an ATX power supply in order to use this feature. |
| Resume On PME# | The system can be turned off with a software command. If you enable this item, the system can automatically resume if there is traffic on the onboard network adapter or PCI LAN/Modem card. You must use an ATX power supply in order to use this feature. |
| RTC Alarm Power On / Date / Hour / Minute / Second | The system can be turned off with a software command. If you enable this item, the system can automatically resume at a fixed time based on the system’s RTC (realtime clock). Use the items below this one to set the date and time of the wake-up alarm. You must use an ATX power supply in order to use this feature. |
| KeyBoard Power On | If you enable this item, you can turn the system on and off by pressing hot keys on the keyboard. You must enable the Keyboard Power On jumper and use an ATX power supply in order to use this feature. |
| HotKey Select | If the item KeyBoard PowerOn is enabled, you can use this item to select the hot keys to power on the system. |

### PCI / Plug and Play Setup Page

This page sets some of the parameters for devices installed on the PCI bus and devices that use the system plug and play capability.

| AMIBIOS SETUP – PCI / PLUG AND PLAY SETUP (C) 2000 American Megatrends, Inc. All Rights Reserved |  |
| --- | --- |
| Plug and Play Aware O/S Yes Primary Graphics Adapter PCI PCI VGA Palette Snoop Disabled Allocate IRQ for PCI VGA Yes |  |
|  | ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values |

| Plug and Play Aware O/S | Enable this item if you are using an O/S that supports Plug and Play such as Windows 95 or 98. |
| --- | --- |
| Primary Graphics Adapter | This item indicates if the primary graphics adapter uses the PCI or the AGP bus. The default PCI setting still lets the onboard display work and allows the use of a second display card installed in a PCI slot. |
| PCI VGA Palette Snoop | When this item is enabled, multiple VGA devices operating on different buses can handle data from the CPU on each set of palette registers on every video device. |
| Allocate IRQ for PCI VGA | If this item is enabled, an IRQ will be assigned to the PCI VGA graphics system. You set this value to No to free up an IRQ. |

### Load Optimal Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the Setup Utility loads a set of fail-safe default values. These default values are not very demanding and they should allow your system to function with most kinds of hardware and memory chips.

### Load Best Performance Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the Setup Utility loads a set of best-performance default values. These default values are quite demanding and your system might not function properly if you are using slower memory chips or other low-performance components.

### Features Setup Page

This page sets some of the parameters for peripheral devices connected to the system.

| AMIBIOS SETUP – PERIPHERAL SETUP (C) 2000 American Megatrends, Inc. All Rights Reserved |  |
| --- | --- |
| OnBoard FDC Enabled OnBoard Serial Port1 3F8h/COM1 OnBoard Serial Port2 2F8h/COM2 OnBoard Ir Port Disabled Ir Port Mode N/A Ir Port IRQ N/A Ir Port DMA N/A Ir Transceiver Type N/A Ir Half-Duplex TimeOut N/A OnBoard Parallel Port 378h Parallel Port Mode Normal EPP Version N/A Parallel Port IRQ 7 Parallel Port DMA N/A OnBoard IDE Both Ultra DMA Support Disabled OnBoard AC’97 Audio Enabled OnBoard AC’97 Modem Auto OnBoard USB Function Enabled USB Function for DOS Disabled |  |
|  | ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values |

| OnBoard FDC | Use this item to enable or disable the onboard floppy disk drive interface. |
| --- | --- |
| OnBoard Serial Port1/2 | Use these items to enable or disable the onboard COM1/2 serial port, and to assign a port address. |
| OnBoard Ir Port, IR items | Use this item to enable or disable the Infrared port, and to assign a port address. If you select a specific address, the resources are assigned to the IR port, and you can use the five items below to determine the operation of the IR port |
| Onboard Parallel Port | Use this item to enable or disable the onboard LPT1 parallel port, and to assign a port address. The Auto setting will detect and available address. |
| Parallel Port Mode | Use this item to set the parallel port mode. You can select SPP (Standard Parallel Port), ECP (Extended Capabilities Port), EPP (Enhanced Parallel Port), or ECP + EPP. |
| EPP Version | Use this item to determine the EPP version, if you have set the Parallel Port Mode to EPP |
| Parallel Port IRQ | Use this item to assign either IRQ 5 or 7 to the parallel port. |
| Parallel Port DMA | Use this item to assign a DMA channel to the parallel port. The options are 0, 1 and 3. |
| Onboard IDE | Use this item to enable or disable either or both of the onboard Primary and Secondary IDE channels. |
| Ultra DMA Support | Use this item to set Ultra DMA for IDE devices on the IDE channels. You must enable this or UDMA devices will not work at their intended speed. |
| Onboard AC’97 Audio | This item enables or disables the onboard AC’97 audio chip. |
| Onboard AC’97 Modem | This item enables or disables the onboard AC’97 modem chip. |
| Onboard USB Function | Enable this item if you plan to use the USB ports on this mainboard. |
| USB Function for DOS | Enable this item if you plan to use the USB ports on this mainboard in a DOS environment. |

### CPU PnP Setup Page

This page lets you manually configure the mainboard for the CPU. The system will automatically detect the kind of CPU that you have installed and make the appropriate adjustments to the items on this page.

AMIBIOS SETUP – CPU PnP SETUP ©2000 American Megatrends, Inc. All Rights Reserved

CPU BRAND AMD K7 CPU Type Athlon CPU Speed 933 MHz CPU Ratio 7.0x ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| CPU BRAND/Type/ Ratio | These items show the kind, core voltage, ratio and frequency of CPU that has installed in your system. |
| --- | --- |
| CPU Speed | Use this item to set the CPU speed that has installed in your system. |

### Hardware Monitor Page

This page sets some of the parameters for the hardware monitoring function of this mainboard.

| AMIBIOS SETUP – HARDWARE MONITOR (C) 2000 American Megatrends, Inc. All Rights Reserved |  |
| --- | --- |
| --- Hardware Monitor --- System Temperature CPU Temperature 30°C/86°F System Fan CPU Fan 5V 5.000 V 3.3Vin 3.300 V 12Vin 12.000 V Vcore |  |
|  | ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values |

| System / CPU Temperature | These items display CPU and system temperature measurement. |
| --- | --- |
| FANs & Voltage Measurements | These items indicate cooling fan speeds in RPM and the various system voltage measurements. |

### Change Password

If you highlight this item and press Enter, a dialog box appears which lets you enter a Supervisor password. You can enter no more than six letters or numbers. Press Enter after you have typed in the password. A second dialog box asks you to retype the password for confirmation. Press Enter after you have retyped it correctly. The password is then required to access the Setup Utility or for that and at start-up, depending on the setting of the Password Check item in Advanced Setup.

#### Change or Remove the Password

Highlight this item, press Enter and type in the current password. At the next dialog box, type in the new password, or just press Enter to disable password protection.

### Exit

Highlight this item and press Enter to save the changes that you have made in the Setup Utility configuration and exit the program. When the Save and Exit dialog box appears, press Y to save and exit, or press N to exit without saving.

# Chapter 4

## Software & Applications

### Introduction

The support software CD-ROM that is included in the mainboard package contains all the drivers and utility programs needed to properly run our products. Below you can find a brief description of each software program, and the location for your mainboard version. More information on some programs is available in a README file, located in the same directory as the software.

If the operating system used in your system is Windows 98, it will automatically install all the drivers and utilities for your board. See the Auto-Installing under Windows 98 section.

### Installing Support Software

The software on the support CD-ROM is for Windows 95/NT/2000 and Windows 98. The installation procedure differs depending on which Operating System you have, but the automatic installation is now for Win98 only.

#### Installing under Windows 95/NT/2000

To install support software for Windows 95/NT/2000 follow this general procedure:

- Insert the support CD-ROM disc in the CD-ROM drive.

(The system might get an error message from the PnP function. Don’t care the message. You don’t really need that file to install the drivers)

- Use My Computer or Windows Explorer to look at the directory structure. You must use the Open command in the right-button menu. Double-clicking on the drive icon will result in an error message because the disc’s AutoRun feature doesn’t work in Windows 95/NT/2000.

- Execute the EXE file name given in the description below.

Note: The correct path name for each software driver is provided, where D: identifies the CD-ROM drive letter – modify if necessary.

#### AGP Driver

The AGP Drivers allows the system to properly manage the AGP slot on the mainboard. Find the driver here:

- D:\VGA\AGP\ALi1647\

#### Audio Driver

The Audio driver allows the system to use the onboard audio circuitry. Find the driver and audio application here:

- D:\SOUND\Driver\ALi\

#### Network Adapter Driver

Find the network interface driver here:

- D:\LAN\Realtek\

#### AMR Modem Driver

Find the driver here:

- D:\Modem\Driver\AMR\Ali1535\

#### USB Driver

The USB Driver allows the system to recognize the USB ports on the mainboard. You need to install this driver if you are running Windows 95. Windows 95 OSR2 does not require this driver.

This driver is available for:

- Win95 – D:\USB\EUSBSUPP\USBSUPP.EXE

- Win95 (Chinese) – D:\USB\CUSBSUPP\CUSBSUPP.EXE

#### 3Deep Software

Find the software here:

- D:\3Deep\3Deep 3.3\Setup.EXE

#### BIOS Update Utility

The BIOS Update utility allows you to update the BIOS file on the mainboard to a newer version. You can download the latest version of the BIOS setup available for your mainboard from the website.

- D:\UTILITY\AMINFxxx.EXE

#### PC-Cillin Software

The PC-cillin software program provides anti-virus protection for your system. Find this program here:

- D:\PC-CILLIN\

### Auto-installing under Windows 98

The support software CD-ROM disc loads automatically under Windows 98. When you insert the CD-ROM disc in the system CD-ROM drive the Autorun feature will automatically bring up the install screen. The screen has three buttons on it, Setup, Browse CD and Exit. See the following screen illustration.

When you click on the Setup button the software installation program will run and you can select what kind of installation you want to do, as explained later in this section.

The Browse CD button is the standard Windows command that allows you to examine the contents of the disc using the Windows 98 file browsing interface.

The Exit button closes the Auto Setup window. To run the program again, reinsert the CD-ROM disc in the drive or click on AutoRun in the context sensitive menu for the CD-ROM drive icon in a file browser window.

#### Installing Software with Auto Setup

To install support software for the system board follow this procedure:

- Click on the Setup button. The install program will load and display the following screen. Click the Next button.

- Select the items that you want to setup by clicking on it (the default options are recommended). Click the Next button to proceed.

- The support software will automatically install.

Once any of the installation procedures start, software is automatically installed in sequence. You will need to follow the onscreen instructions, confirm commands and allow the computer to restart as few times as is needed to complete installing whatever software you selected to install. When the process is finished, all the support software will be installed and working.

There are some utilities that you have to manually install if you need, check to the above section.