Hardware FAQ for M285 / M288

1.1 Question:
What are the available processors supported by M285 / M288

Answer:
Currently, the supported processors are listed as below,

- Pentium-4 CPU at 2.0/2.2/2.4+GHz

- Mobile Pentium-4 CPU at 1.6/1.8+GHz

1.2. Question:
The Brightness ( Fn+F8, F9 ) and contrast ( Fn+F10, F11 ) keys do not work?

Answer:

Usually, the brightness adjustment is so trivial that human eyes cannot tell the difference. It is easier to see the changing of brightness in Windows environment rather than DOS environment. Please also note that the contrast control keys ( Fn+F10, F11 ) do not work with TFT but DSTN/HPA notebooks. For a better sense of brightness adjustment, please change the background to white color if possible.

1.3 Question:
How to erase the password if the password is forgotten?

Answer:

You could adjust DIP SWITCH 5 to clear CMOS ( RTC ) data. The DIP SW5 position under Memory socket door. You can use small flat screw driver to adjust position#4 switch to ON and waiting for 3 to 5 minutes, then, please switch back to OFF position in the DIP SW5..

Warning!! Before clear CMOS ( RTC ) data, you must remove the AC adapter and Battery first.

1.4. Question:
The battery life becomes shorter, How can I rebuild it ?

Answer:

Battery life will vary from user to user depending on how the computer is being used. To help increase your battery life, you can configure the power management features within the Power Management properties of Windows 98 Second Edition.

To access Power Management properties click on the Start button, go to Settings and choose Control Panel. Next, click on the Power Management Icon. You can customize these settings as you choose. You should be aware that any power management features will be of benefit only if there are periods of inactivity which allow the settings to engage. Please note that the battery in the A360 takes about 2.5 hours to fully charge with the machine turned OFF.

1.5 Question:
How do I do the battery learning cycle progress?

Answer:

Cycling your battery every month or so will extend the life expectancy of your A360 battery.
Important! Before beginning this process make sure your AC adapter is plugged in. The best way to do this is to turn the unit ON and press the F2 key when prompted to enter Setup. Go to the Power Management Setup menu and press Enter. Arrow down to Battery Auto Calibration, and change it to enabled. By pressing the Pg Dn key, you will get a message stating. “Please make sure that the AC adapter & battery are present. Do you wish to do battery auto-learning? (Y/N)” Press Y and then Enter. Once calibration is completed the computer will prompt you to shutdown and restart.

1.6 Question:
What is the function of PHDISK.EXE program and how to use it?

Answer:

The PHDISK utility of the notebook allows you to create a Suspend-To-Disk (STD) partition or file that is used to save the opened files when you activate STD mode and power off the computer. If you want to make use of the STD feature, you need first to run the PHDISK utility.

You could see a message appear in a red column during Power On Self Test before you execute the PHDISK.EXE. The red column displays ?Save To Disk file not found. Save to Disk feature is disabled. Run Phdisk for information.

File: creat new. Partition: consult manual. Hit any key to exit. ?The above message will disappear after you have execute the Phdisk.exe.

The Phdisk program has support FAT 16 and FAT 32 format also. To know the more detail information about PHDISK, you could execute Phdisk /? on the command from the directory where the PHDISK command is located.

1.7 Question:
Where & How to adjust the CPU jumper switch setting?

Answer:

The M285 / M288 features Intel Pentium 4 SFF Processors. It is located on the middle-right side of the system motherboard.

SW5

Switch 5 Jumper Setting

### Keyboard Type Select ( SW5 )

| K/B Type | Pos #1 | POS #2 |
| --- | --- | --- |
| US KEYBOARD | OFF | OFF |
| RESERVE | OFF | ON |
| JP KEYBOARD | ON | OFF |
| UK KEYBOARD | ON | ON |

### Logo Select ( SW5 )

| Logo | Pos #3 |
| --- | --- |
| RESERVE | OFF |
| RESERVE | ON |

### Password Override ( CMOS / RTC Data ) Jumper Setting

| ##### Password Override | ##### Pos#4 |
| --- | --- |
| ##### Password Normal | ##### OFF |
| ##### Clear Password | ##### ON |

- Before doing password override, take off AC adapter and battery first.

### Main Board ID Select

| Main Board ID | Pos #5 | POS #6 |
| --- | --- | --- |
| PCMCIA USE INTA# | OFF | OFF |
| RESERVE | OFF | ON |
| PCMCIA USE INTB# | ON | OFF |
| RESERVE | ON | ON |

### CD-ROM Master / Slave Select

| CD-ROM | Pos #7 |
| --- | --- |
| RESERVE | OFF |
| RESERVE | ON |

### CMOS Clear Select

| CD-ROM | Pos #8 |
| --- | --- |
| Normal | OFF |
| Clear CMOS | ON |

- Pos # 8 is N/A Pin for reserve

## SW6 / CPU VID SELECT  (Only for M288 Model)

SW6

Switch 6 Jumper Setting

| SW6 | 6 |
| --- | --- |
| Desktops CPU | OFF |
| Mobile CPU | ON |

|  |  | Processor Pins |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PIN5 | PIN4 | PIN3 | PIN2 | PIN1 | Vcc_max |
| OFF | OFF | OFF | OFF | OFF | VRM output off |
| OFF | OFF | OFF | OFF | ON | 1.100 |
| OFF | OFF | OFF | ON | OFF | 1.125 |
| OFF | OFF | OFF | ON | ON | 1.150 |
| OFF | OFF | ON | OFF | OFF | 1.175 |
| OFF | OFF | ON | OFF | ON | 1.200 |
| OFF | OFF | ON | ON | OFF | 1.225 |
| OFF | OFF | ON | ON | ON | 1.250 |
| OFF | ON | OFF | OFF | OFF | 1.275 |
| OFF | ON | OFF | OFF | ON | 1.300 |
| OFF | ON | OFF | ON | OFF | 1.325 |
| OFF | ON | OFF | ON | ON | 1.350 |
| OFF | ON | ON | OFF | OFF | 1.375 |
| OFF | ON | ON | OFF | ON | 1.400 |
| OFF | ON | ON | ON | OFF | 1.425 |
| OFF | ON | ON | ON | ON | 1.450 |
| ON | OFF | OFF | OFF | OFF | 1.475 |
| ON | OFF | OFF | OFF | ON | 1.500 |
| ON | OFF | OFF | ON | OFF | 1.525 |
| ON | OFF | OFF | ON | ON | 1.550 |
| ON | OFF | ON | OFF | OFF | 1.575 |
| ON | OFF | ON | OFF | ON | 1.600 |
| ON | OFF | ON | ON | OFF | 1.625 |
| ON | OFF | ON | ON | ON | 1.650 |
| ON | ON | OFF | OFF | OFF | 1.675 |
| ON | ON | OFF | OFF | ON | 1.700 |
| ON | ON | OFF | ON | OFF | 1.725 |
| ON | ON | OFF | ON | ON | 1.750 |
| ON | ON | ON | OFF | OFF | 1.775 |
| ON | ON | ON | OFF | ON | 1.800 |
| ON | ON | ON | ON | OFF | 1.825 |
| ON | ON | ON | ON | ON | 1.850 |

1.8 Question:
How come of my Power Saving Function key can not work with windows 98 ACPI environment?

Answer:
In the ACPI environment, Power Saving Function was controlled by OS. That's why the Power Saving Function key can not work.

1.9 Question:
How come of my Power saving Function key cannot work with windows 2000 environment?

Answer:
In the ACPI environment, Power Saving Function was controlled by OS and the Windows 2000 was an ACPI OS. That's why the Power Saving Function key can not work.

1.10 Question:

Why the system detected 56MB during POST only? I have use 64MB RAM module into system.

Answer:

Due to the reason that the VGA memory share with system memory ( Shared Memory Architecture, SMA ), if the frame buffer under BIOS setting is set as 8MB, you can only see 56MB.

1.11 Question:

What is Card Bus?

Answer:

CardBus devices combine the best features of 16-bit PC Cards with the performance of 32-bit PCI adapters. The CardBus specification provides a standard interface for peripherals. CardBus delivers increased throughput and functionality through 32-bit data transfers, bus mastering, and other PCI features, while maintaining backward compatibility with 16-bit PC Card adapters. The following are the CardBus key features:

- True 32-bit I/O with high speed 33MHz bus, compared with an 8.33MHz bus for 16-bit I/O.

- Bus mastering allows peer-to-peer CardBus data transfers.

- Burst-mode data transfers.

- Low power consumption using 3.3-volt CMOS devices.

- Hot Insertion and automatic configuration.

- Support for 16-bit PC Card and 32-bit CardBus devices in the same slot.

- True processor independence.

1.12 Question

How can I have the CD audio play from AUDIO DJ panel?

Answer :

To playing the Audio CD from AUDIO DJ Panel, make sure that you have switch to On/Off position, then the tiny LCD will show the CD-Track number from Audio CD. Press the AUDIO DJ Panel key to control the playing.

Take note that the AUDIO DJ panel will not function under Windows environment.