# M285 / M288 Software FAQ

1.1 Question:
How to check the BIOS version?

Answer:
You could get the BIOS version by pressing F2 function key to entering setup utility during Power On Self Test. The BIOS version will display at the bottom of the main item selected. The format should be : 1.0A-1.10-0812, the 1.0A is BIOS version that you want.

1.2 Question: 
How to install Windows ME?

Answer:
The easiest way to install Windows ME is to boot from Windows ME start-up disk. With Windows ME start-up disk, you don't need to install CD-ROM driver since the start-up disk can support virtually all CD-ROM device. Insert Windows ME Installation CD into CD-ROM drive and run setup.exe
If you don't have Windows ME start-up disk, you need to install CD-ROM / DVD-ROM driver under DOS. Then, insert Windows ME Installation CD into CD-ROM drive and run setup.exe

1.3 Question:
How to install other devices for Windows XP in this computer?

Answer:

### Installing Windows XP from Optical Devices

To install Windows XP directly from your CD-ROM or DVD-ROM, please go to Boot menu of BIOS setup menu. Use arrow key to select "CD-ROM Drive", then use "+" or "-" to move it to the top. Go to Exit menu and select “Exit Saving Changes”.

To install Windows XP directly from your CD-ROM or DVD-ROM, insert Windows XP installation CD into CD-ROM drive and boot on the notebook. Follows the screen instructions on the screen.

1.4 Question: 
How to install the VGA Driver for Windows XP?

Answer:

Your notebook computer uses the high-performance SiS 650 VGA controller, which is an AGP 4X video local bus, 2D/3D Graphic Engine. Following is the procedure for installing the VGA Driver for Windows XP:

- Click the Start button, and then point to Run. The Run dialog box appears.

- Click the Browse button and specify the directory as. 
"E:\Drivers\WinXP\VGA\Setup.exe"

- Implement the setup program to install this driver. The Welcome dialog box appears.

- Click Next to process the further step continuously when screen displays this command.

- Press Finish to restart your system.

1.5  Question: 
How to install the Audio Driver for Windows XP?

Answer:

- Boot Windows from your hard disk and insert the disc containing the Audio driver.

- Click the Start button, then click Run. In the Run dialog box, click the Browse button and navigate into the directory where the new Audio driver is. Then, run “setup.exe”.

- The Welcome message box appears. Click Next to continue with the next step.

- With “Upgrade Drivers” selected, click Next to begin install the Audio driver.

- With “Yes, I want to restart my computer now” selected, click Finish to complete the audio installation.

1.6   Question:
How to install the Modem driver for the Windows XP?

Answer:

Your notebook computer may come with an optional internal modem. The internal modem is a 56Kps V.90 Askey Data Fax modem.

- Click the Start button, then point to Settings, and click Control Panel.

- Double-click on the System icon and click on the Device Manager folder tab.

- Under the Other Devices line, you will find the PCI Communication Device, click Remove, OK, then Refresh buttons to appear the New Hardware Found Message Box.

- In the Add New Hardware Wizard message box which shows searching PCI Communication Device driver. Click Next to proceed to the next step.

- Select “Search for the best driver for your device”, and click Next.

- Tick on “Specify a location box”. Then, click Browse button and navigate to the modem driver location as “E:\driver\winme\ modem”. Click Next to begin searching the driver.

- The Add New Hardware will found LT Win Modem. Click Next to continue install the driver.

- Click Finish button to finish install modem driver.

1.7  Question: 
How to install LAN driver for the Windows XP?

Answer:
Following is the procedure for install the Realtek 8100BL Fast Ethernet LAN driver:

- Boot Windows XP from your hard disk and insert the disc containing the LAN driver for Windows XP.

- Click the Start button, then click Settings, and Control Panel. Double click System and click Device Manager tab. Under Other devices, you'll see PCI Ethernet Controller. Select it and click Remove button.

- Click Refresh button. The Add New Hardware Wizard will detect PCI Ethernet Controller. Click Next to search for the driver. Click Next to continue.

- Tick Specify a location and click Browse button. Then, navigate to “d:\lan” and click Next.

- Click Next to accept the updated driver for the Realtek 8100BL Fast Ethernet LAN driver. Click Next to continue with LAN driver installation.

- Insert the disk labeled “Windows XP CD-ROM”, and then click OK.

- Type “D:\WinXP\Setup”, then click “OK”.

- Click Finish to complete installation.

Restart Computer to finish setting up LAN.

1.8   Question:
How to install the EZ-button device for the Windows XP

Answer:

- Boot Windows from your hard disk and insert the disc containing the Internet/e-mail button driver.

- Click the Start button, then click Run. In the Run dialog box, click Browse button and navigate to the directory as "E:\driver\EZbutton\setup.exe".

- The Welcome dialog will appear. Click Next to continue with the installation.

- If you wish to install the driver in a different directory, click Browse. Otherwise, click Next to continue with the next step.

- The Select Program Folder dialog box will appear. To setup icon on a different folder, you may type a new folder name or select one from the existing Folder list. Click Next to continue.

- Click Finish to complete installation.

1.9   Question:
How to update the latest BIOS version for M285 / M288

Answer:
Warning!! Due to the reason that the incorrect BIOS version may cause encounter unexpected symptom and possible make system can not bootable!
You could get the latest BIOS version via our FTP site; FTP://pcg.fic.com.tw/NBTECH/M285 , Here is the procedure to upgrade the BIOS as below :

- Unzip the attached file from FTP site and save all of them to a formatted 1.44MB floppy diskette.

- Power on the computer and enter the pure DOS environment . ( Press F8 before enter to OS and choose "Safe mode command prompt only"  )

- Insert the formatted 1.44MB floppy diskette which containing the attached file.
Please execute a:\phlash BIOS.ROM /s then the system will flash the BIOS automatically.

1.10   Question:
How to enhance and improve the system performance?

Answer:
You can enhance Hard Disk and CD-ROM drive performance. You can see the improvement when running Winbench99.
You will want to enable DMA or Ultra DMA support on IDE devices to improve performance, because this feature enable supporting hardware to copy data directly into memory rather than having the CPU copy very byte. This capability greatly lowers the CPU utilization of the system on I/O interfaces. However, you will want to disable this feature if you frequently encounter data corruption during this feature enabled. To enable DMA or Ultra DMA setting:

- Click Start button -> Setting -> Control Panel. Double click System and Click the Device Manager tab. Navigate to Disk drives and double click Generic IDE Disk Type01.

2.    Click the Settings tab and tick on DMA.  Click OK and restart the system. This DMA feature will improve Hard Disk performance under Win95/98. For Windows NT, you do not need to do the above steps.

( The CD-ROM steps are same as above but navigate to CD-ROM and double click on your CD-ROM title. )

1.11   Question:
How can I do for extend the battery charge life?

Answer:

Battery life will vary from user depending on how the computer is being used. To follows below steps to extend the battery life correctly.

- Click on Start Button, Go to Settings and then select Control Panel.

- Go to Power Options Icon and then you can customize these setting as you want. You should be aware that any power management features will be benefit only if there are periods of inactivity, which allow the settings to engage.

1.12   Question:

How do I cycle my battery?

Answer:

To keep the better battery performance, we did suggest the user to cycling the battery every month. Follows below steps to execute the battery learning cycle correctly.

- Connected the AC adapter and fully charge the battery until the charge LED ( Orange Color ) becomes off.

- Entering the BIOS setup utility by pressing F2 function key during POST screen.

- Go to Exit menu and then select Battery Re-flash item and it will announce the user to remove the AC Adapter and continue.

- The battery starts to drain and shows decreasing capacity status on the screen until notebook turns off.

- When the notebook is shut down automatically, plug the AC adapter back on the notebook and let it charge until the Power LED turns GREEN.

1.13   Question:

What is different about Windows XP?

Answer:

Windows XP operating system including the follows item:

- Digital Media

- Home Networking

- Online Experience

- Critical File Protection

- Revering Your Computer Back to Normal

- Improved Help and Support

- Support for UPnP (Universal Plug and Play) Devices

1.14   Question:

How do I use the Revert Your Computer feature?

Answer:

You can access the System Restore Utility through Start, Programs, Accessories, System Tools. This features allows you to undo changes made to your computer that might prevent your computer from working properly. Windows will automatically make backups of your configuration files from time to time, and these can be restored using the System Restore Program. Please take note that this program does NOT backup document files. You must be still back there up separately.

1.15   Question:

How does Critical File Protection work and how does it affect me?

Answer:

Critical File Protection works by preserving copies of system files when you install new software. If you install a software package that damages or replaces a system file, this feature allows you to recover these files.