5AMVP3

MAIN BOARD

ATX  FORM  FACTOR

USER'S   MANUAL

(VER.2.0)

COPYRIGHT

THIS MANUAL AND THE INFORMATION CONTAINED HEREIN ARE PROTECTED BY COPYRIGHT. ALL RIGHTS ARE RESERVED.

WARNING AND DISCLAIMER

THIS MANUAL  IS DESIGNED TO PROVIDE INFORMATION  ABOUT  THE PENTIUM® AGP MAIN  BOARD.  EVERY EFFORT HAS BEEN MADE TO MAKE THIS MANUAL AS ACCURATE AS POSSIBLE, BUT NO WARRANTY OR FITNESS IS IMPLIED.    ALL THE INFORMATION IS PROVIDED ON AN 'AS IS' BASIS. THE AUTHOR AND HIS CORRESPONDING PUBLISHING COMPANY SHALL  HAVE NEITHER LIABILITY NOR RESPONSIBILITY TO  ANY  PERSON  OR ENTITY WITH  RESPECT TO ANY LOSS OR DAMAGES ARISING  FROM THE INFORMATION  CONTAINED IN THIS MANUAL  OR FROM THE USE OF THE SYSTEM BOARD THAT ACCOMPANIES IT. INFORMATION CONTAINED IN THIS MANUAL IS SUBJECT TO CHANGE WITHOUT NOTICE. THE MANUFACTURER OF THE SYSTEM BOARD WILL NOT BE HELD RESPONSIBLE FOR TECHNICAL OR EDITORIAL  OMISSIONS  MADE HEREIN, NOR FOR THE INCIDENTAL OR CONSEQUENTIAL DAMAGES RESULTING FROM  ITS FURNISHING, PERFORMANCE, FUNCTIONALITY OR USE. SUBSEQUENT CHANGES TO THIS MANUAL WILL BE INCORPORATED INTO THE NEXT EDITION. WE WELCOME ANY SUGGESTION REGARDING THIS MANUAL OR OUR COMPUTER PRODUCTS.

TRADEMARKS

ALL TRADEMARKS ARE THE PROPERTY OF THEIR RESPECTIVE OWNERS.

TABLE OF CONTENTS

CHAPTER 1  INTRODUCTION.......…………………………..…..1

1-1  OVERVIEW .........................................................................….1

1-2  MAIN FEATURES.....................……..........................................2

1-3  UNPACKING ........................................................…...………….3

1-4  NOTICE FOR CD DRIVER INSTALLATION………………......……..3

CHAPTER 2  MAIN BOARD SPECIFICATION.……………….…4

2-1  MAIN BOARD LAYOUT.......................................................……4

2-2  JUMPER SETTINGS.................................................................5

2-3  SIMM MODULE INSTALLATION PROCEDURES..............……..14

2-4  DIMM MODULE INSTALLATION PROCEDURES............……….15

2-5  MEMORY CHART......…..........................................................16

### CHAPTER 3 BIOS SETUP.……………….......................…....18

3-1  STANDARD CMOS SETUP.....................…........................…...19

3-2  BIOS FEATURES SETUP..........................…..............……...….21

3-3  CHIPSET FEATURES SETUP.......................…….....………...… 25

3-4  POWER MANAGEMENT SETUP..................……………...…..….27

3-5  PNP/PCI CONFIGURATION...............................….….……..…..30

3-6 LOAD SETUP DEFAULTS...................................…………………31

3-7 INTEGRATED PERIPHERALS.........................................………32

3-8 SUPERVISOR PASSWORD ..................................................….35

3-9 IDE HDD AUTO DETECTION.............................................……35

3-10 SAVE & EXIT SETUP.......................................................……38

3-11 EXIT WITHOUT SAVING...................................…............……38

3-12 I/O & MEMORY MAP............................................……...........39

3-13 TIME & DMA CHANNELS MAP..................................…..……..41

3-14 INTERRUPT MAP.....................................................…....…….41

3-15 RTC & CMOS RM MAP.....................................................……42

3-16 CD DRIVER SETUP AFTER INSTALLATING WINDOWS 98……43

CHAPTER 1   INTRODUCTION

1-1 OVERVIEW

5AMVP3 MAIN BOARD COMBINES THE ADVANCED CAPABILITIES OF THE VIA® APOLLO MVP3 CHIPSETS WITH A HIGH PERFORMANCE CONCURRENT PCI LOCAL BUS ARCHITECTURE TO PROVIDE THE IDEAL PLATFORM FOR UNLEASHING THE UNSURPASSED SPEED AND POWER OF THE INTEL® PENTIUM PROCESSOR, CYRIX 6X86M1/6X86LM1/ 6X86MX/ 6X86MII AND AMD K5/K6/K6-2 3D PROCESSORS, AND CAN BE EASILY UPGRADED FOR 321 PIN ZIF SOCKET.  TO BE MORE SPECIFIC, THIS MAIN BOARD OFFERS 100MHZ CPU/DRAM BUS FREQUENCIES AND SUPPORT PC100/66MHZ SDRAM OPTION WITH 100MHZ CPU.

THE PROCESSOR'S ADVANCED PERFORMANCE IS COMPLEMENTED BY A SECOND LEVEL WRITE BACK PB-SRAM UP TO 1MB AND MAIN MEMORY UP TO 1GB RAM.  THE MAIN MEMORY IS INITIALED USING THE BOARD'S FOUR 72-PIN SIMM SOCKETS  AND  TWO 168-PIN DIMM SOCKETS THAT ACCEPT EITHER THE NEW HIGH PERFORMANCE EDO, BEDO, OR FAST PAGE MODE DRAM.

VIA® MVP3 CHIPSET INTEGRATES A FULL SET OF I/O FEATURES ON BOARD, INCLUDING TWO 16550 UART COMPATIBLE SERIAL PORTS, ONE EPP/ECP CAPABLE PORT, ONE FLOPPY DISK CONTROLLER, AND ONE INFRARED COMMUNICATION CONTROLLER.   ONE CHIP BUILT IN ENHANCED IDE CONTROLLER PROVIDES CONVENIENT, SPEEDY PCI BUS MASTER CONNECTION CAPABLE OF FOUR IDE DEVICES, INCLUDING HARD DISK AND CD-ROM.

VIA®  BUILDS ALL PRODUCTS TO EXACTING STANDARDS, USING THE HIGHEST QUALITY COMPONENTS AVAILABLE.  WE ARE PROUD TO PROVIDE THIS SYSTEM BOARD AND HOPE IT BRINGS YOU YEARS OF RELIABLE SERVICE.

1-2 MAIN FEATURES

| # ITEM | DESCRIPTION |
| --- | --- |
| # CPU | SUPPORTS CPU IN A 321 PIN ZIF SOCKET - INTEL® PENTIUMP54C /P55C CPU - CYRIX6X86/6X86L/6X86MX/6X86MII 3. AMD K5/K6/K6-2 3D PROCESSORS |
| # CHIPSET | VIA® APOLLO MVP3 CHIPSET |
| # CACHE | 512K; 1MB |
| # MEMORY | 2DIMM/ 4SIMM 1. DIMM: A. SYNCHRONOUS DRAM WITH 168-PIN DIMM MODULES OF 2, 8,16, 32, 64MB, 128MB FOR 66MHZ B. EDO RAM WITH 168-PIN DIMM MODULES OF 8,16, 32, 64MB, 128MB FOR 66MHZ(5V OR 3.3V) C. 100MHZ FREQ. CPU: PC-100 100 MHZ DIMM 2. SIMM: TAKE UP TO 256MB RAM IN ONE BANK, USING 72-PIN SIMM MODULES OF 1, 2, 4, 8, 16, 32, 64 OR 128MB WITH SUPPORTS FOR EDO, BEDO OR FAST PAGE MODE MEMORY |
| # AGP SLOT | AGP COMPLIANT WITH 1X OR 2X MODE UP TO 133MHZ |
| # EXP. SLOT | 3*16-BIT ISA SLOTS AND 4*32-BIT PCI EXPANSION SLOTS |
| # I/O | INTEGRATED ITE IT8661F/RF MULTI I/O CHIPSET THAT OFFERS TWO 16550 UART COMPATIBLE SERIAL PORTS, ONE EPP/ECP CAPABLE PORT, ONE I.R. PORT, AND ONE FLOPPY DISK DRIVE CONNECTOR |
| # FLASH ROM | ## 128KB |
| # SIZE | 21.00 X 30.50 CM |
| # SPECIAL FEATURE | ATX POWER CONNECTOR |

NOTE1: 5AMVP3 IS ESPECIALLY DESIGNED FOR AMD® K6-PR300 3.45V V I/O(66MHZ

FREQ.) AND K6-2 3D CPU WITH 100 MHZ FREQ.

NOTE2: INTEL® AGP CARD ONLY SUPPORTS INTEL® SERIES CHIPSETS, LIKE EX/LX/

BX CHIPSETS.  IT  DOES NOT SUPPORT VIA® , SIS® AND ALI® CHIPSETS AT

PRESENT.

NOTE3:  IT IS NOT SUGGESTED TO OVER-SPEED CPU‘S FREQUENCY, WHICH IS

BEYOND  SPECIFICATION.

1-3 UNPACKING

THE MAIN BOARD PACKAGE CONTAINS:

* 5AMVP3 MAIN BOARD

* MANUAL

* CABLES

* DRIVER & UTILITY / CD

IF ANY OF THESE ITEMS IS MISSING OR DAMAGED,CONTACT THE DEALER FROM WHOM YOU PURCHASED.  LEAVE THE 5AMVP3  IN ITS ORIGINAL PACKING UNTIL YOU ARE READY TO INSTALL IT.

1-4 NOTICE FOR CD DRIVER INSTALLATION

THIS CD CONTAINS DRIVERS AS BELOW:

- MAIN BOARDS: I440BX®, I440EX®, I440LX®, I430TX®, VIA®  VPX, VP3, & MVP3 MAIN BOARDS

- AGP CARDS:  S- 6326 AND T985

- SOUND: ESS-SOLO-1 SOUND DRIVER

- GL518SM: CPU VOLTAGE /TEMPERATURE AND FAN SPEED DETECTION SOFTWARE

E.   PCCILLIN: ANTI- VIRUS PROTECTION SOFTWARE

PLEASE READ “INDEX” BEFORE INSTALLING REQUIRED DRIVERS.  “INDEX” OFFERS ALL THE INFORMATION ON THE BELOW FILES. “INDEX” FILE IS HTML FORMAT.

## CHAPTER 2  MAIN BOARD SPECIFICATION

2-1 MAIN BOARD LAYOUT

Socket 7

CN2

USB

CN1 KB

CN3 MOUSE

SIMM1

SIMM2

SIMM3

SIMM4

PCI1

PCI4

PCI3

PCI2

JP11

1

1

CN5

IR

JP18

JP1

JP2

JP3

CN6

ATX POWER

JP14

JP12

JP13

BIOS

DIMM1

AGP

SL2

SL1

I/O

DIMM2

SOFT-PWR

/SUSPEND

HDD-LED

COM2 COM1

##### LPT

JP5

JP10

JP9

JP8

JP15

SL3

CPU FAN

VIA

VT82C

598MVP3

1

JP7

JP6

JP4

VT82C586B

512K

512K

IDE1

CN7 FDC

JP17

1

IDE2

1

JP19

1

TB-LED

KEY-LOCK

RESET

SPEAKER

PW-LED

2-2 JUMPER SETTINGS

INTEL® PENTIUM PROCESSOR INSTALLATION (PLEASE REFER JP19 TO  PAGE   11.)

| CPU CLOCK | SYS. CLOCK | JP1 | JP2 | JP3 | JP6 | JP7 | JP8 | JP9 | JP10 | JP17 | JP18 | RATIO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P54C 100 MHZ 3.3V | 66 MHZ | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2, 5-6, 7-8 | OPEN | 1.5X |
| P54C 133 MHZ 3.3V | 66 MHZ | 2-3 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2, 5-6, 7-8 | OPEN | 2X |
| P54C 166 MHZ 3.3V | 66 MHZ | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2, 5-6, 7-8 | OPEN | 2.5X |
| P55C 166 MHZ 2.8/3.3V | 66 MHZ | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 7-8 | 1-2, 3-4, 5-6 | 2.5X |
| P54C 200 MHZ 3.3V | 66 MHZ | 1-2 | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2, 5-6, 7-8 | OPEN | 3X |
| P55C 200 MHZ 2.8/3.3V | 66 MHZ | 1-2 | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 7-8 | 1-2, 3-4, 5-6 | 3X |
| P55C 233 MHZ 2.8/3.3V | 66 MHZ | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 7-8 | 1-2, 3-4, 5-6 | 3.5X |

AMD K5/K6/K6-2 3D PROCESSOR INSTALLATION (PLEASE REFER JP19 TO  PAGE   11.)

| CPU CLOCK | SYS. CLOCK | JP1 | JP2 | JP3 | JP6 | JP7 | JP8 | JP9 | JP10 | JP17 | JP18 | RATIO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K5-PR100/PR133 3.52V | 66 MHZ | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2,3-4,5-6,7-8 | OPEN | 1.5X |
| K5-PR166 3.52V | 66 MHZ | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2,3-4,5-6,7-8 | OPEN | 2.5X |
| K6-PR166 2.9/3.3V | 66 MHZ | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2,7-8 | 1-2,3-4, 5-6 | 2.5X |
| K6-PR200 2.9/3.3V | 66 MHZ | 1-2 | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2,7-8 | 1-2,3-4, 5-6 | 3X |
| K6-PR233 3.2/3.3V | 66 MHZ | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 5-6,7-8 | 1-2,3-4, 5-6 | 3.5X |
| K6-PR233 3.3/3.3V | 66 MHZ | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2,5-6,7-8 | 1-2,3-4,5-6 | 3.5X |
| K6-PR266 2.2/3.3V | 66 MHZ | 2-3 | 1-2 | 2-3 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 3-4 | 1-2,3-4,5-6 | 4X |
| K6-PR300 2.2/3.45V | 66 MHZ | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 3-4 | 1-2,3-4,5-6 | 4.5X |
| K6-2 250 2.2V/3.3V | 100MHZ | 2-3 | 2-3 | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 | 2-3 | 3-4 | 1-2, 3-4,5-6 | 2.5X |
| K6-2 266 2.2V/3.3V | 66 MHZ | 2-3 | 1-2 | 2-3 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 3-4 | 1-2,3-4,5-6 | 4X |
| K6-2 300 2.2V/3.3V | 100MHZ | 1-2 | 2-3 | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 | 2-3 | 3-4 | 1-2,3-4,5-6 | 3X |
| K6-2 333 2.2V/3.3V | 95 MHZ | 1-2 | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 2-3 | 3-4 | 1-2,3-4,5-6 | 3.5X |
| K6-2 350 2.2/3.3V | 100 MHZ | 1-2 | 1-2 | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 | 2-3 | 3-4 | 1-2,3-4,5-6 | 3.5X |

CYRIX 6X86 PROCESSOR INSTALLATION (PLEASE REFER JP19 TO  PAGE   11.)

| CPU CLOCK | SYS. CLOCK | JP1 | JP2 | JP3 | JP6 | JP7 | JP8 | JP9 | JP10 | JP17 | JP18 | RATIO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6X86L 166 2.8/3.3V | 66 MHZ | 2-3 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 7-8 | 1-2,3-4, 5-6 | 2X |
| 6X86L 200 2.8/3.3V | 75 MHZ | 2-3 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 2-3 | 1-2 | 7-8 | 1-2,3-4, 5-6 | 2X |
| 6X86MX 166 2.9/3.3V | 66 MHZ | 2-3 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2,7-8 | 1-2,3-4, 5-6 | 2X |
| 6X86MX 200 2.9/3.3V | 75 MHZ | 2-3 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 2-3 | 1-2 | 1-2,7-8 | 1-2,3-4, 5-6 | 2X |
| 6X86MX 233 2.9/3.3V | 75 MHZ | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 | 2-3 | 1-2 | 1-2, 7-8 | 1-2,3-4, 5-6 | 2.5X |
| 6X86MX 266 2.7/3.3V | 83 MHZ | 2-3 | 2-3 | 1-2 | 1-2 | 2-3 | 2-3 | 2-3 | 1-2 | 1-2,3-4,5-6 | 1-2,3-4,5-6 | 2.5X |
| 6X86MII 300 2.9/3.3V | 66MHZ | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2, 7-8 | 1-2, 3-4, 5-6 | 3.5X |

*CYRIX® MX PR-266 IS FOR REFERENCE ONLY.

### JP1-JP3  - CPU/BUS RATIO

Socket 7

JP1-JP3  - CPU/BUS RATIO

JP1

JP2

JP3

1

1

1

| RATIO | JP1 | JP2 | JP3 | RATIO | JP1 | JP2 | JP3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JP1 JP2 JP3 1 1 1 JP1 JP2 JP3 1 1 1 1.5X | 1-2 | 1-2 | 1-2 | 2X | 2-3 | 1-2 | ###### 1-2 |
| JP1 JP2 JP3 1 1 1 JP1 JP2 JP3 1 1 1 2.5X | 2-3 | 2-3 | ###### 1-2 | 3X | 1-2 | 2-3 | ###### 1-2 |
| JP1 JP2 JP3 1 1 1 JP1 JP2 JP3 1 1 1 3.5X | 1-2 | 1-2 | ###### 1-2 | 4X | 2-3 | 1-2 | ###### 2-3 |
| JP1 JP2 JP3 1 1 1 4.5X | 2-3 | 2-3 | ###### 2-3 |  |  |  |  |

JP4&JP5: SDRAM CLOCK FREQUENCY

Socket 7

JP4JP5SDRAM CLOCK CLK2-31-2SAME WITH CPU CLK1-22-366MHZSAME WITH CPU CLK

66MHZ

JP5

JP10

JP9

JP8

JP7

JP6

JP4

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

JP5

JP10

JP9

JP8

JP7

JP6

JP4

###### JP6  - JP10 - CPU FREQUENCY SELECTOR

Socket 7

PLEASE NOTE “CPU FREQUENCY SELECTOR” ARE JP6-JP10.  JP4 & JP5 ARE NOT INCLUDED.

JP5

JP10

JP9

JP8

JP7

JP6

JP4

1

1

1

1

1

1

1

1

| JUMPER | 66.6MHZ | 75MHZ | 83.3MHZ | 95 MHZ | 100MHZ |
| --- | --- | --- | --- | --- | --- |
| JP6 | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 |
| JP7 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |
| JP8 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 |
| JP9 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 |
| JP10 | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 |
| JP7 JP6 JP4 1 1 1 1 1 1 JP7 JP6 JP4 1 1 1 JP7 JP6 JP4 1 1 1 1 1 1 1 JP5 JP10 JP9 JP8 1 1 1 1 JP5 JP10 JP9 JP8 1 1 1 1 1 1 1 1 JP7 JP6 JP4 1 1 1 JP5 JP10 JP9 JP8 1 1 1 1 DEMO. JP7 JP6 JP4 JP5 JP10 JP9 JP8 JP5 JP10 JP9 JP8 |  |  |  |  |  |

###### JP11 - DIMM VOLTAGE SELECTOR

JP11DIMM VOLTAGE SELECOR       1-2, 3-43.3V DIMM   5-6, 7-85V    DIMM

Socket 7

3.3V DIMM            5V DIMM

8    7     7

2     1

8     7

2     1

1

JP12: FLASH ROM VOLTAGE SELECTOR

JP12FLASH ROM VOLTAGE SELECTOR1-25V (SST®,WINBOND®)2-312V (INTEL®,MXIC®)

Socket 7

1

1

5V                               12V

JP13: CASE CONNECTOR

Socket 7

1

JP13: CASE CONNECTOR

RESET

zzz

PW-LED

TB-LED

SPEAKER

KEYLOCK

SOFT-PWR

SUSPEND

HDD-LED

JP14: CMOS STATUS

JP14CMOS STATUS1-2NORMAL2-3CLEAR CMOS

Socket 7

NORMAL          CLEAR CMOS

1

1

1

###### JP15: WAKE ON LAN

Socket 7

JP15: WAKE ON LAN

1

Socket 7

JP17 - CPU VOLTAGE SELECTOR

JP17 - CPU VOLTAGE SELECTOR

1      2

7      8

1

| JP17 | VCORE | JP17 | VCORE | JP17 | VCORE | JP17 | VCORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 2 1-2 1 2 7 8 1 2 1 2 7 8 7 8 7 8 | 2.1V | 3-4 | 2.2V | 1-2, 3-4, 5-6 | 2.7V | 7-8 | 2.8V |
| 7 8 1 2 1 2 7 8 1 2 1-2, 7-8 1 2 7 8 7 8 | 2.9V | 5-6, 7-8 | 3.2V | 1-2, 5-6, 7-8 | 3.3V | 1-2, 3-4, 5-6, 7-8 | 3.5V |

JP19VIO1-2, 5-6, 7-83.3V1-2, 3-4, 5-6, 7-83.45V

3.3V                        3.45V

JP19: VIO VOLTAGE SELECTOR

Socket 7

1

1

1

1

1

CN1 : PS/2 MOUSE CONNECTOR & KEYBOARD CONNECTOR

###### CN2: USB(UNIVERSL SERIAL BUS) CONNECTOR

COM1/COM2/LPT:  SERIAL PORT 1 CONNECTOR, SERIAL PORT2 CONNECTOR, PRINTER PORT CONNECTOR

USB

LPT1

COM1

COM2

PS/2 MOUSE

KEYBOARD

Socket 7

1

###### CN8: I.R (INFRA-RED )CONNECTOR

Socket 7

CN8: I.R(INFRA-RED) CONNECTOR

PINSIGNAL NAME    1IRRX   2GROUND   3IRTX   4+5V DC   5IRRXH   6+5V DC   7GROUND

1

CN6- ATX POWER SUPPLY CONNECTOR

Socket 7

CN6: ATX POWER CONNECTOR

| PIN | DESCRIPTION | PIN | DESCRIPTION |
| --- | --- | --- | --- |
| 1 | +3.3V DC | 11 | +3.3V DC |
| 2 | +3.3V DC | 12 | -12V DC |
| 3 | GROUND | 13 | GROUND |
| 4 | +5V DC | 14 | SOFT-POWER |
| 5 | +5V DC | 15 | GROUND |
| 6 | +5V DC | 16 | GROUND |
| 7 | GROUND | 17 | GROUND |
| 8 | POWER GOOD | 18 | -5V DC |
| 9 | +5V STANDBY | 19 | +5V DC |
| 10 | +12V DC | 20 | +5V DC |

CN7: FLOPPY DISK CONNECTOR

IDE1: PRIMARY HDD CONNECTOR

###### IDE2: SECONDARY HDD CONNECTOR

CN7: FLOPPY DISK CONNECTOR

IDE1: PRIMARY HDD CONNECTOR

IDE2: SECONDARY HDD CONNECTOR

2

1

2

1

2

1

CPU FAN

Socket 7

CPU FAN

1

2-3 SIMM MODULE INSTALLATION PROCEDURES

THE SIMM MEMORY MODULES WILL ONLY FIT IN ONE ORIENTATION AS SHOWN BECAUSE OF A "PLASTIC SAFETY TAB" ON ONE END OF THE SIMM SLOTS WHICH REQUIRES THE "NOTCHED END" OF THE SIMM MEMORY MODULES.

PRESS THE MEMORY MODULE FIRMLY INTO PLACE STARTING AT A 45 DEGREE ANGLE MAKING SURE THAT ALL THE CONTACTS ARE ALIGNED WITH THE SOCKET.

NOTCHED END

2-4 DIMM MEMORY INSTALLATION PROCEDURES

INSERT THE MODULE AS SHOWN.   DUE TO DIFFERENT NUMBER OF PINS ON EITHER SIDE OF THE BREAKS, THE MODULE WILL ONLY FIT IN THE ORIENTATION AS SHOWN.    DRAM SIMM MODULES HAVE THE SAME PIN CONTACT ON BOTH SIDES.   SDRAM DIMM MODULES HAVE DIFFERENT PIN CONTACTSS ON EACH SIDE AND THEREFORE HAVE A HIGHER PIN DENSITY.

EDO DIMM SUPPORTS EITHER 3.3V OR 5V.    PC-100 100 MHZ DIMM IS REQUIRED WHEN THE SYSTEM USES 100 MHZ CPU.

2-5 MEMORY CHART

THIS MAIN BOARD CONTAINS 4 BY 72-PIN SIMM SOCKETS  (BANK0: SIMM1,SIMM2 & BANK 1: SIMM3/SIMM4 ) OR  2 BY 168-PIN DIMM SOCKET (DIMM 1 & DIMM2).  SIMM MODULE SOCKETS ARE DIVIDED IN TWO BANKS : SIMM1, SIMM2 IN ONE BANK AND SIMM3, SIMM4 IN ANOTHER BANK.  MEMORY CAN BE INSTALLED IN A VARIETY OF CONFIGURATION.  PLEASE REFERS TO THE FOLLOWING CHART:

| TOTAL MEMORY | BANK 0 (SIMM1,SIMM2) |  | BANK 1 (SIMM3,SIMM4) | BANK1 (DIMM1) | BANK2 (DIMM2) |
| --- | --- | --- | --- | --- | --- |
| 8MB |  | 4MB & 4MB |  |  |  |
| 8MB |  |  | 4MB & 4MB |  |  |
| 8MB |  |  |  | 4MB | 4MB |
| 8MB |  |  |  | 8MB |  |
| 8MB |  |  |  |  | 8MB |
| 12MB |  |  |  | 4MB | 8MB |
| 12MB |  |  |  | 8MB | 4MB |
| 16MB |  | 4MB & 4MB | 4MB & 4MB |  |  |
| 16MB |  | 8MB & 8MB |  |  |  |
| 16MB |  |  | 8MB & 8MB |  |  |
| 16MB |  |  |  | 8MB | 8MB |
| 16MB |  |  |  | 16MB |  |
| 16MB |  |  |  |  | 16MB |
| 24MB |  | 8MB & 8MB | 4MB & 4MB |  |  |
| 24MB |  | 4MB & 4MB | 8MB & 8MB |  |  |
| 24MB |  |  |  | 16MB | 8MB |
| 24MB |  |  |  | 8MB | 16MB |
| 32MB |  | 8MB & 8MB | 8MB & 8MB |  |  |
| 32MB |  |  |  | 16MB | 16MB |
| 32MB |  |  |  | 32MB |  |
| 32MB |  |  |  |  | 32MB |
| 32MB |  | 16MB&16MB |  |  |  |
| 32MB |  |  | 16MB&16MB |  |  |
| 40MB |  |  |  | 32MB | 8MB |
| 40MB |  |  |  | 8MB | 32MB |
| 40MB |  | 4MB&4MB | 16MB&16MB |  |  |
| 40MB |  | 16MB&16MB | 4MB&4MB |  |  |
| 48MB |  |  |  | 16MB | 32MB |
| 48MB |  |  |  | 32MB | 16MB |
| 48MB |  | 8MB&8MB | 16MB&16MB |  |  |
| 48MB |  | 16MB & 16MB | 8MB & 8MB |  |  |
| 64MB |  | 16MB & 16MB | 16MB & 16MB |  |  |
| 64MB |  |  |  | 32MB | 32MB |
| 64MB |  | 32 MB & 32MB |  |  |  |
| 64MB |  |  | 32MB & 32MB |  |  |
| 64MB |  |  |  | 64MB |  |
| 64MB |  |  |  |  | 64MB |
| 72MB |  | 4MB & 4MB | 32MB & 32MB |  |  |
| 72MB |  | 32MB & 32MB | 4MB & 4MB |  |  |
| 72MB |  |  |  | 64MB | 8MB |
| 72MB |  |  |  | 8MB | 64MB |
| 80MB |  | 8MB & 8MB | 32MB & 32MB |  |  |
| 80MB |  | 32MB & 32MB | 8MB & 8MB |  |  |
| 80MB |  |  |  | 16MB | 64MB |
| 80MB |  |  |  | 64MB | 16MB |
| 96MB |  | 16MB & 16MB | 32MB & 32MB |  |  |
| 96MB |  | 32MB & 32MB | 16MB & 16MB |  |  |
| 96MB |  |  |  | 64MB | 32MB |
| 96MB |  |  |  | 32MB | 64MB |
| 128MB |  | 32MB & 32MB | 32MB & 32MB |  |  |
| 128MB |  | 64MB & 64MB |  |  |  |
| 128MB |  |  | 64MB & 64MB |  |  |
| 128MB |  |  |  | 64MB | 64MB |
| 128MB |  |  |  | 128MB |  |
| 128MB |  |  |  |  | 128MB |
| 256MB |  |  |  | 128MB | 128MB |
| 256MB |  | 64MB & 64MB | 64MB & 64MB |  |  |
| NOTE1: PLEASE NOTE THAT SIMM 3 / SIMM 4 AND DIMM 1 BELONG TO THE SAME BANK, SO DO NOT INSERT MODULES AT THE SAME BANK AT THE SAME TIME. NOTE2: IT IS NOT SUGGESTED TO MIX UP SIMM MODULES AND DIMM MODULES IN THE SAME SYSTEM, OR THE SYSTEM MAY GET UNSTABLE. NOTE3: EDO DIMM SUPPORTS EITHER 3.3V OR 5V. WITH 100 MHZ CPU, IT IS REQUIRED TO USE PC-100 100 MHZ DIMM . NOTE4: THE MAXIMUM MEORY FOR DRAM IS UP TO 2GB, AND SDRAM IS UP TO 1GB. |  |  |  |  |  |

CHAPTER 3   BIOS SETUP

AWARD'S ROM BIOS PROVIDES A BUILT-IN SETUP PROGRAM WHICH ALLOWS USER TO MODIFY THE BASIC SYSTEM CONFIGURATION AND HARDWARE PARAMETERS.  THE MODIFIED DATA WILL BE STORED  IN A BATTERY-BACKED CMOS RAM SO DATA WILL BE RETAINED EVEN WHEN THE POWER IS TURNED OFF.   IN GENERAL, THE INFORMATION SAVED  IN THE CMOS RAM STAY UNCHANGED UNLESS THERE IS CONFIG. CHANGE IN THE SYSTEM, SUCH AS HARDISK DRIVER REPLACEMENT OR CHANGES OF NEW EQUIPMENT.

IT IS POSSIBLE THAT CMOS HAS A BATTERY FAILURE WHICH CAUSES DATA  LOSS IN CMOS RAM.  IF SO,  RE-ENTERING SYSTEM CONFIG. PARAMETERS BECOMES NECESSARY.

TO ENTER SETUP PROGRAM

POWER ON THE COMPUTER AND PRESS <DEL> KEY IMMEDIATELY, WHICH BRINGS YOU INTO BIOS CMOS SETUP UTILITY.

THE MENU DISPLAYS ALL THE MAJOR SELECTION ITEMS AND ALLOWS USER TO SELECT ANY OF SHOWN ITEMS.  THE SELECTION IS MADE BY MOVING CURSOR (PRESS ANY DIRECTION KEY) TO THE ITEM AND PRESS "ENTER" KEY.  AN ON LINE HELPS MESSAGE IS DISPLAYED AT THE BOTTOM OF THE SCREEN AS CURSOR IS MOVING TO VARIOUS ITEMS WHICH PROVIDES USER BETTER UNDERSTANDING OF EACH FUNCTION.  WHEN A SELECTION IS MADE, THE MENU OF SELECTED ITEM WILL APPEAR.  THUS,  THE USER CAN MODIFY ASSOCIATED CONFIGURATION PARAMETERS.

3-1 STANDARD CMOS SETUP

THE STANDARD CMOS SETUP SCREEN IS DISPLAYED AS ABOVE.   SYSTEM BIOS AUTOMATICALLY DETECTS MEMORY SIZE.  NO CHANGES ARE NECESSARY SINCE IT HAS A FEW ITEMS FOR SETTING.   EACH ITEM MAY HAVE ONE OR MORE OPTION SETTINGS.  IT ALLOWS YOU TO CHANGE THE SYSTEM DATE AND TIME, IDE HARD DISK, FLOPPY DISK DRIVE TYPES FOR DRIVE A: AND B: BOOT UP VIDEO DISPLAY MODE, AND POST ERROR HANDLING SELECTION.  USE THE ARROW KEYS TO HIGHLIGHT THE ITEM AND THEN USE THE <PGUP> OR <PGDN> KEYS TO SELECT THE VALUE YOU WANT IN EACH ITEM.

### HARD DISK CONFIGURATIONS

TYPE  :

SELECT FROM "1" TO "45" TO FILL IN REMAINING FIELDS WITH REDEFINED VALUES OF DISK DRIVES.   SELECT "USER" TO FILL IN THE REMAINING FIELDS.  SELECT "AUTO" TO DETECT THE  HDD TYPE AUTOMATICALLY.

SIZE  :

THE HARD DISK SIZE.  THE UNIT IS MEGA BYTE

CYLS :

THE CYLINDER NUMBER OF THE HARD DISK

HEAD :

THE READ/WRITE HEAD NUMBER OF HARD DISK.  THE  RANGE IS

FROM "1" TO "16"

PRECCOMP :

THE CYLINDER NUMBER AT WHICH THE DISK DRIVE CHANGES

THE WRITTEN TIMING

LANDZ :

THE CYLINDER NUMBER THAT THE DISK DRIVE HEADS (READ/WRITE) ARE SEATED WHEN  THE DISK DRIVE IS PARKED

SECTOR :

THE SECTOR NUMBER OF EACH TRACK DEFINED ON THE HARD DISK. THE RANGE IS FROM "1" TO "64."

MODE :

SELECT "AUTO" TO DETECT THE MODE TYPE AUTOMATICALLY.  IF YOUR HARD DISK SUPPORTS THE LBA MODE, SELECT "LBA" OR "LARGE". HOWEVER, IF YOUR HARD DISK CYLINDER IS MORE THAN 1024 AND DOES NOT SUPPORT THE LBA FUNCTION, YOU HAVE TO SET AT "LARGE". SELECT "NORMAL" IF YOUR HARD DISK SUPPORTING  CYLINDER IS BELOW 1024.

3-2 BIOS FEATURES SETUP

SELECTING THE "BIOS FEATURE SETUP" OPTION IN THE CMOS SETUP UTILITY MENU ALLOWS USER TO CHANGE SYSTEM RELATED PARAMETERS IN THE DISPLAY MENU. THIS MENU SHOWS ALL OF THE MANUFACTURER'S DEFAULT VALUES OF SV-P55V. AGAIN, USER CAN MOVE THE CURSOR BY PRESSING DIRECTION KEYS AND <PGDN> OR <PGUP>  KEY TO MODIFY THE PARAMETERS. PRESSING [F1] KEY TO DISPLAY HELP MESSAGE OF THE SELECTED ITEM.  THE SETUP PROGRAM ALSO PROVIDES 2 CONVENIENT WAYS TO LOAD THE DEFAULT PARAMETER DATA FROM CMOS [F7] AREA IF SHOWN DATA IS CORRUPTED.  THIS PROVIDES THE SYSTEM WITH A CAPABILITY TO RECOVER FROM ANY POSSIBLE ERROR.

VIRUS WARNING:

WHEN ENABLED, IT ASSIGNS THE BIOS TO MONITOR THE MASTER BOOT SECTOR AND THE DOS BOOT SECTOR OF THE FIRST HARD DISK DRIVE.  THE OPTIONS ARE:  ENABLED, DISABLED (DEFAULT).

CPU INTERNAL CACHE:

WHEN ENABLED,  IT IMPROVES THE SYSTEM PERFORMANCE.  DISABLE THIS ITEM WHEN TESTING OR TROUBLE-SHOOTING.  THE OPTIONS ARE: ENABLED (DEFAULT), DISABLED

EXTERNAL CACHE:

WHEN ENABLED, IT SUPPORTS AN OPTIONAL CACHE SRAM.  THE OPTIONS ARE: ENABLED (DEFAULT), DISABLED.

QUICK POWER ON SELF TEST:

WHEN ENABLED, IT ALLOWS THE BIOS TO BYPASS THE EXTENSIVE MEMORY TEST.  THE OPTIONS ARE : ENABLED, DISABLED (DEFAULT).

BOOT SEQUENCE:

ALLOWS THE SYSTEM BIOS TO TRY FIRST TO BOOT THE OPERATING SYSTEM FROM THE SELECTED DISK DRIVE.  THE OPTIONS ARE : A, C, SCSI (DEFAULT); C, A, SCSI; C, CDROM, A;  CDROM, C, A;  D, A, SCSI; E, A, SCSI;  F, A, SCSI; SCSI, A, C;  SCSI, C, A; C ONLY; LS/ZIP, C.

SWAP FLOPPY DRIVE:

WHEN ENABLED,  IT ALLOWS YOU TO SWITCH THE ORDER IN WHICH THE OPERATING SYSTEM ACCESSES THE FLOPPY DRIVES DURING BOOT UP.  THE OPTIONS ARE : ENABLED, DISABLED (DEFAULT)

BOOT UP FLOPPY SEEK:

WHEN ENABLED, IT ASSIGNS THE BIOS TO PERFORM FLOPPY DISKETTE DRIVE TESTS BY ISSUING THE TIME-CONSUMING SEEK COMMANDS.  THE OPTIONS ARE : ENABLED (DEFAULT), DISABLED

BOOT UP NUMLOCK STATUS:

WHEN SET TO ON, IT ALLOWS THE BIOS TO AUTOMATICALLY ENABLE THE NUM LOCK FUNCTION WHEN THE SYSTEM BOOTS UP. THE OPTIONS ARE: ON (DEFAULT), OFF.

PORT 92H FAST A20:

WHEN ENABLED, IT ALLOWS THE A20G BUS LINE SIGNAL GENERATED FROM THE CHIPSET 82C586 PC/AT TO DIRECTLY PASS TO PORT 92H, INSTEAD OF THE KEYBOARD CONTROLLER. IT WILL SPEED UP THE SYSTEM PERFORMANCE.  THE OPTIONS ARE FAST, NORMAL (DEFAULT).

TYPEMATIC RATE SETTING:

THE TERM "TYPEMATIC" MEANS THAT WHEN A KEYBOARD KEY IS HELD DOWN, THE CHARACTER IS REPEATEDLY ENTERED UNTIL THE KEY IS RELEASED.  WHEN THIS ITEM IS ENABLED,  YOU MAY CHANGE THE TYPEMATIC REPEAT RATE.  THE OPTIONS ARE : DISABLED (DEFAULT), ENABLED

TYPEMATIC RATE (CHARS/SEC):

IT SETS THE RATE OF A CHARACTER REPEAT WHEN THE KEY IS HELD DOWN.  THE OPTIONS ARE: 6 (DEFAULT),  8, 10, 12, 15, 20, 24, 30.

TYPEMATIC DELAY (MSEC):

IT SETS THE DELAY TIME BEFORE A CHARACTER IS REPEATED.  THE OPTIONS ARE: 250 (DEFAULT), 500, 750, 1000 MILLISECOND.

SECURITY OPTION:

ALLOWS YOU TO SET THE SECURITY LEVEL OF THE SYSTEM.  THE OPTIONS ARE : SETUP (DEFAULT), SYSTEM.

PCI/VGA PAIETTE SNOOP:

WHEN ENABLED, IT ALLOWS YOU TO INSTALL AN ENHANCED GRAPHICS ADAPTER CARD.  IF YOUR GRAPHICS ADAPTER CARD DOES NOT SUPPORT THE PALETTE SNOOP FUNCTION,  PLEASE SET AT  DISABLE TO AVOID SYSTEM MALFUNCTION. THE OPTIONS ARE: ENABLED, DISABLED (DEFAULT).

VIDEO BIOS SHADOW:

WHEN ENABLED,  IT ALLOWS THE BIOS TO COPY THE VIDEO ROM CODE OF THE ADD-ON VIDEO CARDS TO THE SYSTEM MEMORY FOR FASTER ACCESS.  THE OPTIONS ARE : ENABLED (DEFAULT), DISABLED.

C8000-CBFFF TO DC000-DFFFF SHADOW:

WHEN ENABLED, IT ALLOWS THE BIOS TO COPY THE BIOS ROM CODE OF  THE ADD-ON CARD TO SYSTEM MEMORY FOR FASTER ACCESS. IT MAY IMPROVE THE PERFORMANCE OF THE ADD-ON CARD.  SOME ADD-ON CARDS WILL NOT FUNCTION PROPERLY IF IT’S BIOS ROM CODE IS SHADOWED. TO USE THESE OPTIONS CORRECTLY, YOU NEED TO KNOW THE MEMORY ADDRESS RANGE USED BY THE  BIOS ROM OF EACH ADD-ON CARD. THE OPTIONS ARE: ENABLED, DISABLED (DEFAULT).

3-3 CHIPSET FEATURES SETUP

ONCHIP USB                             : ENABLED

USB KEYBOARD SUPPORT  : ENABLED

VIDEO BIOS CACHEABLE:

WHEN ENABLED, IT ALLOWS THE SYSTEM TO USE THE VIDEO BIOS CODES C0000H-C7FFFH FROM CACHE, INSTEAD OF THE SLOWER DRAMS OR ROMS. VIDEO BIOS MUST BE SHADOWED FIRST.  THE OPTIONS ARE: ENABLED (DEFAULT), DISABLED.

SYSTEM BIOS CACHEABLE:

WHEN ENABLED, IT ALLOWS THE ROM AREA E0000H-FFFFFH TO BE CACHEABLE WHEN CACHE CONTROLLER IS ACTIVATED.  THE OPTIONS ARE: ENABLED (DEFAULT), DISABLED.

MEMORY HOLE AT 15MB ADDR.:

WHEN ENABLED, THE MEMORY HOLE AT THE 15MB ADDRESS WILL BE RELOCATED TO THE 15~16MB ADDRESS RANGE OF THE ISA CYCLE WHEN THE PROCESSOR ACCESSES THE 15~16MB ADDRESS AREA.  WHEN DISABLED, THE MEMORY HOLE AT THE 15MB ADDRESS WILL BE TREATED AS A DRAM CYCLE WHEN THE PROCESSOR ACCESSES THE 15~16MB ADDRESS.  THE OPTIONS ARE: ENABLED, DISABLED (DEFAULT).

SUSTAINED 3T WRITE:

THE CACHE ARCHITECTURE ADOPTS WRITE THROUGH.  WHEN WRITE THROUGH IS ENABLED, THE PERFORMANCE IS BETTER UNDER MOST APPLICATION ENVIRONMENT BECAUSE THE VP3 FIFO QUEUE IS DEEP.  THE OPTIONS ARE: ENABLED, DISABLED (DEFAULT).

ONCHIP USB :

DEFAULT IS DISABLE.  IF THE SYSTEM USES RELEVENT USB DEVICES,  FIRST ENABLE “ASSIGN IRQ FOR USB”(REFER TO PAGE 30 &31).  THEN GO TO CHIPSET FEATURES SETPUP, AND CHOOSE “ENABLE” IN “ON CHIP USB.”  THE OTHER LINE “USB KEYBOARD SUPPORT”  WILL APPEAR AS BELOW.

3-4 POWER MANAGEMENT SETUP

POWER MANAGEMENT:

WHEN ENABLE, IT ALLOWS YOU TO USE POWER MANAGEMENT FEATURES.

PM CONTROL BY APM:

THE OPTION "NO" ALLOWS THE BIOS TO IGNORE THE APM (ADVANCED POWER MANAGEMENT) SPECIFICATION. SELECTING "YES" ALLOWS THE BIOS TO WAIT FOR APM'S PROMPT BEFORE IT ENTERS DOZE MODE, STANDBY MODE, OR SUSPEND MODE.  IF APM IS INSTALLED, IT WILL PROMPT BIOS TO SET THE SYSTEM INTO THE POWER SAVING MODE AFTER ALL TASKS ARE DONE.

VIDEO OFF OPTION:

THIS FEATURE PROVIDES THE SELECTIONS OF THE VIDEO DISPLAY POWER SAVING MODE.  THE OPTION "SUSPEND -> OFF" ALLOWS THE DISPLAY BLANKS IF THE SYSTEM ENTERS SUSPEND MODE.  THE OPTION "ALL MODES -> OFF" ALLOWS THE VIDEO DISPLAY BANKS IF THE SYSTEM ENTERS DOZE MODE OR SUSPEND MODE.  THE OPTION "ALWAYS ON" ALLOWS THE VIDEO DISPLAY TO STAY IN STANDBY MODE EVEN THE SYSTEM ENTERS DOZE OR SUSPEND MODE.

VIDEO OFF METHOD:

THE OPTION "V/H SYNC+ BLANK" ALLOWS THE BIOS TO BLANK OFF SCREEN DISPLAY BY TURNING OFF THE V-SYNC SIGNALS SENT FROM ADD-ON VGA CARD.  "DPMS SUPPORTED" ALLOWS THE BIOS TO BLANK OFF SCREEN DISPLAY BY YOUR ADD-ON VGA CARD WHICH SUPPORTS DPMS.  (DISPLAY POWER MANAGEMENT SIGNALING FUNCTION). "BLANK SCREEN" ALLOWS THE BIOS TO BLANK SCREEN DISPLAY BY TURNING OFF THE RED-GREEN-BLUE SIGNALS.

MODEM USE IRQ:

WHEN THE SYSTEM IS IN GREEN FUNCTION, MODEM WAKES UP THE SYSTEM THROUGH IRQ.

HDD POWER DOWN:

SELECTING "DISABLED" WILL TURN OFF THE HARD DISK DRIVE (HDD) MOTOR.  SELECTING "1MIN ...15MIN" ALLOWS YOU TO DEFINE THE HDD IDLE TIME BEFORE THE HDD ENTERS POWER SAVING MODE.  THE OPTION "WHEN SUSPEND" LETS THE BIOS TURN THE HDD MOTOR OFF WHEN THE SYSTEM IS IN SUSPEND MODE.  THE OPTIONS "1MIN ... 15MIN" AND "WHEN SUSPEND" WILL NOT WORK CONCURRENTLY. WHEN HDD IS IN POWER SAVING MODE, ANY ACCESS TO THE HDD WILL WAKE THE HDD UP.

DOZE MODE:

WHEN DISABLED, THE SYSTEM WILL NOT ENTER DOZE MODE. THE SPECIFIED TIME OPTION DEFINE THE IDLE TIME THE SYSTEM TAKES BEFORE IT ENTERS DOZE MODE.

SUSPEND MODE:

WHEN DISABLED, THE SYSTEM WILL NOT ENTER SUSPEND MODE. THE SPECIFIED TIME OPTION DEFINES THE IDLE TIME THE SYSTEM TAKES BEFORE IT ENTERS SUSPEND MODE.

VGA:

SELECTING "ON" WILL ENABLE THE POWER MANAGEMENT TIMERS WHEN A "NO ACTIVITY" EVENTS IS

DETECTED IN THE VGA. SELECTING "OFF" TO DISABLE THE PM TIMER EVEN IF A "NO ACTIVITY" EVENT IS DETECTED.

LPT & COM:

SELECTING "LPT & COM" WILL ENABLE THE POWER MANAGEMENT TIMERS WHEN A "NO ACTIVITY"

EVENT IS DETECTED IN THE LPT AND COM PORTS. SELECTING "LPT" ("COM") WILL ENABLE THE POWER MANAGEMENT TIMERS WHEN A "NO ACTIVITY" EVENT IS DETECTED IN THE LPT (COM) PORTS.

SELECTING "NONE" TO DISABLE THE PM TIMER EVEN IF A "NO ACTIVITY" EVENT IS DETECTED.

HDD & FDD:

SELECTING "ON" WILL ENABLE THE POWER MANAGEMENT TIMERS WHEN A "NO ACTIVITY" EVENT IS DETECTED IN THE HARD DISK DRIVE AND FLOPPY DISK DRIVE. SELECTING "OFF" TO DISABLE THE PM TIMER EVENT IF A "NO ACTIVITY" EVENT IS DETECTED.

DMA/MASTER:

WHEN THE MASTER IS WORKING, THE SYSTEM WILL NOT HAVE SMI SIGNAL UNTIL THE MASTER IS FINISHED.

PRIMARY INTR:

WHEN ENABLED, YOU CAN CHOOSE ANY IRQ#.

IRQ#:

WHEN SET AT "PRIMARY" THE PROCESSOR WILL POWER DOWN ONLY AFTER THE BIOS DETECTS A "NO IRQ ACTIVITY" DURING THE TIME SPECIFIED BY THE SUSPEND TIME. IF SET AT "SECONDARY EVENT"

THE SYSTEM WILL DISTINGUISH WHETHER AN INTERRUPT ACCESSES AN I/O ADDRESS OR NOT. IF IT DOES, THE SYSTEM ENTERS THE STANDBY MODE. IF NOT, THE SYSTEM ENTERS THE DREAMING MODE; THAT IS THE SYSTEM GOES BACK FULL-ON STATUS BUT LEAVES THE MONITOR BLANK. FOR INSTANCE, IF THE SYSTEM CONNECTS TO A LAN AND RECEIVES AN INTERRUPT FROM ITS FILE SERVER, THE SYSTEM WILL ENTER THE DREAMING MODE TO EXECUTE THE CORRESPONDING CALLING ROUTINE.

3-5 PNP/PCI CONFIGURATION

PCI IRQ ACTIVED BY:

IF YOUR IDE CARDS IS TRIGGERED BY EDGE, SET IT AT "EDGE".  THE OPTIONS ARE: LEVEL, EDGE (DEFAULT).

CPU TO PCI WRITE BUFFER:

WHEN ENABLED, IT ALLOWS DATA AND ADDRESS ACCESS TO THE INTERNAL BUFFER OF 82C586B,  SO THE PROCESSOR CAN BE RELEASED FROM THE WAITING STATE.  THE OPTIONS ARE: ENABLED (DEFAULT), DISABLED.

PCI DYNAMIC BURSTING:

WHEN ENABLED, THE PCI CONTROLLER ALLOWS BURSTING PCI TO TRANSFER IF THE CONSECUTIVE PCI CYCLES COME WITH THE ADDRESS FALLING IN THE SAME 1KB SPACE.   THIS IMPROVES THE PCI BUS THROUGH-PUT.  THE OPTIONS ARE: ENABLED (DEFAULT), DISABLED.

ASSIGN IRQ FOR USB & ASSIGN IRQ FOR USB

DEFAULT VALUE FOR “ASSIGN IRQ FOR USB” IS DISABLE AND “ASSIGN IRQ FOR VGA” ENABLE.  FOR VGA CARDS OF GOOD QUALITY,  IRQ ADDRESS IS REQUIRED, BUT FOR SOME LOW-END VGA CARDS, NO IRQ IS NEEDED.  THEREFORE, PLEASE CONSIDER VGA CARD’S FEATURE BEFORE SETTING “ASSIGN IRQ FOR USB” AND “ASSIGN IRQ FOR VGA.”

3-6 LOAD SETUP DEFAULTS

SELECTING THIS FIELD LOADS THE FACTORY DEFAULTS FOR BIOS AND CHIPSET FEATURES WHICH THE SYSTEM AUTOMATICALLY DETECTS.

3-7 INTEGRATED PERIPHERALS

ONCHIP IDE FIRST CHANNEL:

WHEN ENABLED,  IT ALLOWS THE IDE DRIVER TO USE THE FIRST CHANNEL OF THE PRIMARY IDE.

ONCHIP IDE SECOND CHANNEL:

WHEN ENABLED, IT ALLOWS THE IDE DRIVE TO USE THE SECOND CHANNEL OF THE PRIMARY IDE.

IDE PRIMARY SLAVE PIO:

THE DEFAULT VALUE IS AUTO.

AUTO:  BIOS WILL AUTOMATICALLY DETECT THE ONBOARD PRIMARY

SLAVE PCI IDE HDD  ACCESSING MODE.7

MODE 0-4 : MANUALLY SET THE IDE ACCESSING MODE.

IDE SECONDARY MASTER PIO:

THE DEFAULT VALUE IS AUTO.

AUTO    :  BIOS WILL AUTOMATICALLY DETECT THE ONBOARD

SECONDARY MASTER PCI IDE HDD ACCESSING  MODE.

MODE 0-4 :  MANUALLY SETS THE IDE ACCESSING MODE.

IDE PRIMARY MASTER PIO:

THE DEFAULT VALUE IS AUTO.

IDE SECONDARY SLAVE PIO:

THE DEFAULT VALUE IS AUTO.

ONBOARD FDC CONTROLLER:

THE DEFAULT VALUE IS ENABLED.

ENABLED  : ENABLED THE ONBOARD I/O CHIP'S FLOPPY DRIVE INTERFACE CONTROLLER. DISABLED : DISABLED THE ONBOARD I/O CHIP'S FLOPPY DRIVE INTERFACE CONTROLLER.  WHEN USE ON-CARD ISA FDC'S CONTROLLER.

ONBOARD SERIAL PORT 1:

THE FIELD ALLOWS THE USER TO SELECT THE SERIAL PORT.  THE DEFAULT VALUE IS AUTO.

COM1: ENABLE ONBOARD SERIAL PORT1 AND ADDRESS IS COM1/3F8H

COM2: ENABLE ONBOARD SERIAL PORT1 AND ADDRESS IS COM2/2F8H

COM3: ENABLE ONBOARD SERIAL PORT1 AND ADDRESS IS COM3/3E8H

COM4: ENABLE ONBOARD SERIAL PORT1 AND ADDRESS IS COM4/2E8H

DISABLED: DISABLE ONBOARD I/O CHIP'S SERIAL PORT 1.

AUTO : BIOS WILL AUTOMATICALLY DETECT THE ONBOARD SERIAL PORT.

ONBOARD SERIAL PORT 2:

THE FIELD ALLOWS THE USER TO SELECT THE SERIAL PORT.  THE DEFAULT VALUE IS AUTO.

COM1: ENABLE ONBOARD SERIAL PORT1 AND ADDRESS IS COM1/3F8H

COM2: ENABLE ONBOARD SERIAL PORT1 AND ADDRESS IS COM2/2F8H

COM3: ENABLE ONBOARD SERIAL PORT1 AND ADDRESS IS COM3/3E8H

COM4: ENABLE ONBOARD SERIAL PORT1 AND ADDRESS IS COM4/2E8H

DISABLED: DISABLE ONBOARD I/O CHIP'S SERIAL PORT 1.

AUTO : BIOS WILL AUTOMATICALLY DETECT THE ONBOARD SERIAL PORT.

ONBOARD PARALLEL PORT:

THE FIELD ALLOWS THE USER TO SELECT THE LPT PORT.  THE DEFAULT VALUE IS 378H/IRQ7.

378H : ENABLE ONBOARD LPT PORT AND ADDRESS IS 378H AND IRQ7

278H : ENABLE ONBOARD LPT PORT AND ADDRESS IS 278H AND IRQ5

3BCH : ENABLE ONBOARD LPT PORT AND ADDRESS IS 3BCH AND IRQ7

DISABLED : DISABLE ONBOARD I/O CHIP'S LPT PORT

3-8 SUPERVISOR PASSWORD

1. IF CMOS IS CORRUPTED OR THE OPTION  IS  NOT USED, A DEFAULT PASSWORD STORED IN THE ROM WILL BE USED.  THE SCREEN WILL DISPLAY THE FOLLOWING MESSAGE:

ENTER PASSWORD

PRESS THE [ENTER] KEY TO CONTINUE

AFTER PROPER PASSWORD IS GIVEN.

2. IF CMOS IS CORRUPTED OR THE OPTION IS USED EARLIER,  BUT THE USER WISHES TO CHANGE DEFAULT PASSWORD, THE SETUP UTILITY WILL DISPLAY A MESSAGE AND ASK FOR A CONFIRMATION.

CONFIRM PASSWORD:

3.AFTER PRESSING THE [ENTER] KEY (ROM PASSWORD IF THE OPTION WAS NOT USED) OR CURRENT PASSWORD (USER-DEFINED PASSWORD), THE USER CAN CHANGE THE PASSWORD AND STORE NEW ONE IN CMOS RAM. A MAXIMUM OF 8 CHARACTERS CAN BE ENTERED.

3-9 IDE HDD AUTO DETECTION

THE "IDE HDD AUTO DETECTION" UTILITY IS A VERY USEFUL TOOL ESPECIALLY WHEN YOU DO NOT KNOW WHICH KIND OF HARD DISK TYPE YOU ARE USING.  YOU CAN USE THIS UTILITY TO DETECT THE CORRECT DISK TYPE INSTALLED IN THE SYSTEM AUTOMATICALLY.   YET,  NOW YOU CAN SET HARD DISK TYPE TO AUTO IN THE STANDARD CMOS SETUP.  YOU DON'T NEED THE"IDE HDD AUTO DETECTION" UTILITY.  THE BIOS WILL AUTO-DETECT THE HARD DISK SIZE AND MODEL ON DISPLAY DURING POST.

ROM PCI/ISA BIOS CMOS SETUP UTILITY AWARD SOFTWARE, INC.

HARD DISK TYPE SIZE CYLS HEADS PRECOMP LANDZONE SECTORS MODE PRIMARY MASTER 343 665 16 65535 664 63 NORMAL PRIMARY SLAVE SECONDARY MASTER SECONDARY SLAVE

NOTE: HDD MODES:

THE AWARD BIOS SUPPORTS 3 HDD MODES:  NORMAL, LBA&LARGE.

NORMAL MODE:

GENERIC ACCESS MODE EITHER  IN BIOS OR IDE CONTROLLER WILL MAKE ANY TRANSFORMATION DURING ACCESSING.  THE MAXIMUM NUMBER OF CYLINDERS, HEAD & SECTORS FOR NORMAL MODE ARE 1024, 16 & 63.

NO.CYCLINDER     (1024)

X NO.HEAD            (16)

X NO.SECTOR           (63)

X NO.PER SECTOR      (512)

-------------------------------

528 MEGABYTES

IF USER SETS IN NORMAL MODE, THE MAXIMUM ACCESSIBLE HDD SIZE WILL BE 528 MEGABYTE EVEN THOUGH ITS PHYSICAL SIZE MAY BE GREATER THAN THAT !

LBA (LOGICAL BLOCK ADDRESSING) MODE:

A NEW HDD ACCESSING METHOD TO OVERCOME THE 528 MEGABYTE BOTTLENECK.  THE NUMBER OF CYLINDERS, HEAD & SECTORS SHOWN IN SETUP MAY NOT BE THE NUMBER PHYSICALLY CONTAINED IN THE HDD. DURING HDD ACCESSING, THE IDE CONTROLLER WILL TRANSFORM THE LOGIC ADDRESS DESCRIBED BY SECTOR, HEAD & CYLINDER INTO ITS OWN PHYSICAL ADDRESS INSIDE THE HDD.  THE MAXIMUM HDD SIZE SUPPORTED BY LBA MODE IS 8.4 GIGABYTES WHICH IS OBTAINED BY THE FOLLOWING FORMULA:

NO.CYLINDER      (1024)

X NO.HEAD            (255)

X NO.SECTOR           (63)

X BYTES.PER SECTOR  (512)

--------------------------------

8.4 GIGABYTES

LARGE MODE:

EXTENDED HDD ACCESS MODE SUPPORTED BY AWARD SOFTWARE. SOME IDE HDDS CONTAIN MORE THAN 1024 CYLINDER WITHOUT LBA SUPPORT (IN SOME CASES, USER DOES NOT WANT LBA). THE AWARD BIOS PROVIDES ANOTHER ALTERNATIVE TO SUPPORT THESE KINDS OF LARGE MODE:

CYLS.     HEADS        SECTOR      MODE

1120           16                   59             NORMAL

560           32                   59             LARGE

BIOS TRICKS DOS (OR OTHER OS) THAT THE NUMBER OF CYLINDERS IS LESS THAN 1024 BY DIVIDING IT INTO 2.  AT THE SAME TIME, THE NUMBER OF HEADS IS MULTIPLIED BY 2.  AVERSE TRANSFORMATION PROCESS WILL BE MADE INSIDE INT 12H IN ORDER TO ACCESS THE RIGHT HDD ADDESS THE RIGHT HDD ADDRESS !

NO.CYLINDER       (1024)

X NO.HEAD                (32)

X NO.SECTOR              (63)

X BYTES.PER SECTOR  (512)

------------------------------

1 GIGABYTES

NOTE: TO SUPPORT LBA OR LARGE MODE OF HDDS, THERE MUST BE SOME SOFTWARES INVOLVED. ALL THESE SOFTWARES ARE LOCATED IN THE AWARD HDD SERVICE ROUTINE (INT 13H). IT MAY FAIL TO ACCESS A HDD WITH LBA (LARGE) MODE SELECTED IF YOU ARE RUNNINGING UNDER A OPERATIONAL SYSTEM WHICH REPLACES THE WHOLE INT 13H. UNIX OPERATIONL SYSTEMS DO NOT SUPPORT EITHER OR LARGE AND MUST UTILIZE THE STANDARD MODE. UNIX CAN SUPPORT DRIVES LARGER THAN 528MB.

3-10 SAVE & EXIT SETUP

AFTER YOU HAVE MADE CHANGES UNDER SETUP, PRESS <ESC> TO RETURN TO THE MAIN MENU.  MOVE CURSOR TO "SAVE AND EXIT SETUP," OR PRESS "F10" AND THEN PRESS "Y" TO CHANGE THE CMOS  SETUP.  IF YOU DIO NOT CHANGE ANYTHING, PRESS <ESC> AGAIN OR MOVE CURSOR TO "EXIT WITHOUT SAVING" AND PRESS "Y" TO RETAIN THE SETUP SETTINGS.  THE FOLLOWING MESSAGE WILL APPEAR AT THE CENTER OF THE SCREEN TO ALLOW YOU TO SAVE DATA TO CMOS AND EXIT THE SETUP UTILITY.

SAVE TO CMOS AND EXIT (Y/N)?

3-11 EXIT WITHOUT SAVING

THE "EXIT WITHOUT SAVING" OPTION WILL BRING YOU BACK TO NORMAL BOOT UP PROCEDURE WITHOUT SAVING ANY INTO CMOS RAM.  ALL OF THE OLD DATA IN THE CMOS WILL NOT BE DESTROYED.  IF YOU SELECT THIS FEATURE, THE FOLLOWING MESSAGE WILL APPEAR AT THE CENTER OF THE SCREEN TO ALLOW YOU TO EXIT THE SETUP UTILITY WITHOUT SAVING CMOS MODIFICATIONS:

QUIT WITHOUT SAVING(Y/N)?

3-12 I/O & MEMORY MAP

MEMORY MAP

| ADDRESS RANGE | SIZE | DESCRIPTION |
| --- | --- | --- |
| 00000-7FFFF | 512K | CONVENTIONAL MEMORY |
| 80000-9FBFF | 127K | EXTENDED CONVENTIONAL MEMORY |
| 9FC00-9FFFF | 1K | EXTENDED BIOS DATA AREA IF PS/2 MOUSE IS INSTALLED |
| A0000-C7FFF | 160K | AVAILABLE FOR HI DOS MEMORY |
| C8000-DFFFF | 96K | AVAILABLE FOR HI DOS MEMORY AND ADAPTER ROMS |
| E0000-EEFFF | 60K | AVAILABLE FOR UMB |
| EF000-EFFFF | 4K | VIDEO SERVICE ROUTINE FOR MONOCHROME & CGA ADAPTER |
| F0000-F7FFF | 32K | BIOS CMOS SETUP UTILITY |
| F8000-FCFFF | 20K | BIOS RUNTIME SERVICE ROUTINE (2) |
| FD000-FDFFF | 4K | PLUG AND PLAY ESCD DATA AREA |
| FE000-FFFFF | 8K | BIOS RUNTIME SERVICE ROUTINE (1) |

I/O MAP

| 000-01F | DMA CONTROLLER (MASTER) |
| --- | --- |
| 020-021 | INTERRUPT CONTROLLER (MASTER) |
| 022-023 | CHIPSET CONTROL REGISTERS. I/O POSTS |
| 040-05F | TIMER CONTROL REGISTERS |
| 060-06F | KEYBOARD INTERFACE CONTROLLER (8042) |
| 070- 07F | RTC PORTS & CMOS I/O PORTS |
| 080-09F | DMA REGISTER |
| 0A0-0BF | INTERRUPT CONTROLLER (SLAVE) |
| 0C0-0DF | DMA CONTROLLER (SLAVE) |
| 0F0-0FF | MATH COPROCESSOR |
| 1F0-1FB | HARD DISK CONTROLLER |
| 278-27F | PARALLEL PORT 2 |
| 2B0-2DF | GRAPHICS ADAPTER CONTROLLER |
| 2F8-2FF | SERIAL PORT 2 |
| 360-36F | NETWORK PORTS |
| 378-37F | PARALLEL PORT 1 |
| 3B0-3BF | MONOCHROME & PARALLEL PORT ADAPTER |
| 3C0-3CF | EGA ADAPTER |
| 3D0-CDF | CGA ADAPTER |
| 3F0-3F7 | FLOPPY DISK CONTROLLER |
| 3F8-3FF | SERIAL PORT-1 |

3-13 TIME & DMA CHANNELS MAP

TIME MAP:

TIMER CHANNEL 0 SYSTEM TIMER INTERRUPT                                                          TIMER CHANNEL 1 DRAM REFRESH REQUEST                                                             TIMER CHANNEL 2 SPEAKER TONE GENERATOR

DMA CHANNELS:

DMA CHANNEL 0 AVAILABLE

DMA CHANNEL 1 ONBOARD ECP (OPTION)                                                                  DMA CHANNEL 2 FLOPPY DISK (SMC CHIP)                                                                 DMA CHANNEL 3 ONBOARD ECP (DEFAULT)                                                              DMA CHANNEL 4 CASCADE FOR DMA CONTROLLER 1                                            DMA CHANNEL 5 AVAILABLE

DMA CHANNEL 6 AVAILABLE

DMA CHANNEL 7 AVAILABLE

3-14 INTERRUPT MAP

NIMI:     NON-MASKABLE INTERRUPT

IRQ(H/W): 0 SYSTEM TIMER INTERRUPT FROM TIMER 0

1 KEYBOARD OUTPUT BUFFER FULL

2 CASCADE FOR IRQ8-15

3 SERIAL PORT2

4 SERIAL PORT1

5 PARALLEL PORT 2

6 FLOPPY DISK (SMC CHIP)

7 PARALLEL PORT 1

8 RTC CLOCK

9 AVAILABLE

10 AVAILABLE

11 AVAILABLE

12 PS/2 MOUSE

13 MATH COPROCESSOR

14 ONBOARD HARD DISK (IDE1) CHANNEL

15 ONBOARD HARD DISK (IDE2) CHANNEL

3-15 RTC & CMOS RAM MAP

RTC & CMOS:00     SECONDS

01     SECOND ALARM

02     MINUTES

03     MINUTES ALARM

04     HOURS

05     HOURS ALARM

06     DAY OF WEEK

07     DAY OF MONTH

08     MONTH

09     YEAR

0A     STATUS REGISTER A

0B     STATUS REGISTER B

0C     STATUS REGISTER C

0D     STATUS REGISTER D

0E     DIAGNOSTIC STATUS BYTE

0F     SHUTDOWN BYTE

10     FLOPPY DISK DRIVE TYPE BYTE

12     HARD DISK TYPE BYTE

13     RESERVE

14     EQUIPMENT TYPE

15     BASE MEMORY LOW BYTE

16     BASE MEMORY HIGH BYTE

17     EXTENSION MEMORY LOW BYTE

18     EXTENSION MEMORY HIGH BYTE

19-2D

2E-2F

- RESERVED FOR EXTENSION MEMORY LOW BYTE

31   RESERVED FOR EXTENSION MEMORY HIGH BYTE                              32   DATE CENTURY BYTE

33   INFORMATION FLAG

34-3F  RESERVE

40-7F  RESERVED FOR CHIPSET SETTING DATA

3-16 CD DRIVER SETUP AFTER INSTALLATING WINDOWS 98

### AFTER SETTING WINDOWS 98, IT IS REQUIRED TO INSTALL DRIVERS FOR 586XIRQ AND VXD-V26 AS BELOW.   FIRST,  PERFORM “SETUP” IN 586XIRQ, AND THEN FOLLOWS “SETUP” IN VXD-V26.

STEP 1

STEP3:

STEP 2

Vxd-v26

*IF THE CD DRIVER IS BEFORE/INCLUDES VER.1.6, PLEASE DOWNLOAD THE LASTEST VERSION FROM OUR WEBSITE :

WWW.LUCKY-STAR.COM.TW