RELIABILITY TEST REPORT

# PRODUCT: S58M/VT7100

Issue Date: October 2, 2000

PREPARED BY:

Steven H.C. Li n

Engineer:

Reliability Engineering Department

APPROVED BY:	REVIEWED BY:

Homer Yu 	Glavin Hsu

Director:	Technical Manager

Component & Reliability Engineering	Reliability Engineering Department

Center

# 0. Conclusion

I. The test results are listed as follows:

1. Acceptance Criteria of C3 Entry

2. Voltage Dips & Short Interruptions Immunity Test

3. Electrical Fast Transient/Burst Immunity Test

4. Surge Immunity Test

5. Electrostatic Discharge Susceptibility Test

6. DC Margin Test

7. Line Voltage and Frequency Test

8. Power ON/OFF Characteristics

9. Temperature/Thermal Profile Test

10. Operating Temperature and Humidity Test

11. Non-operating Temperature and Humidity Test

12. Operating Vibration Test t

13. Non-operating Vibration Test

14. Acoustic Noise Measurement

15. Transportation Drop Test

16. AC Power Consumption Measurement	Note

Note: The SKU B system add on LEADTEK Winfast Geforce 256 32MB SDR+TV Out + DVI VGA card, so power consumption to high, can’t under EPA SPEC. 30W.

II. Details of Some Definitions:

1. The following definitions of the Performance Criteria are defined by Acer Inc. according to IEC-1000-4-11 Basic Standard. On the basis of the operating conditions and functional specifications of the equipment under test, the test results shall be classified as in the following, Unless, different specifications are given by product specifications.

| Performance Criteria | Error Level | Description |
| --- | --- | --- |
| A | No Error | Normal performance within the specification limits. |
| B | Soft Error | Temporary degradation or loss of function or performance which is self-recoverable. |
| C | Re-start Error | Temporary degradation or loss of function or performance which requires operator intervention or system reset. |
| D | Damage Error | Degradation or loss of function which is not recoverable due to damage of equipment (components) or software, or loss of data. |

2. Test Software:

Scorpion 1

 HDD R/W and Compare repeatedly

 FDD R/W and Compare repeatedly

 CD-ROM/DVD-ROM Play Movie CD

Scorpion 2

 QAPluse  or HDD13.EXE (for Acoustic noise test only)

III. Test Configuration List:

| Brand / model | SKU A (505) | SKU B (506) |
| --- | --- | --- |
| Main Board | S58M-1 | S58M-1 |
| CPU | P iii 550 MHz/100MHz | P iii 733MHz/133MHz |
| Heat-sink |  |  |
| Memory | MICRON 64MB 0.18u / PC-100 | MICRON 128MB 0.18u /PC-133 |
| System chipset | Intel F82815 | Intel F82815 |
| HDD | Seagate U10 15.3GB (ST315323A) | Seagate Cuda II 15.3GB (ST315320A) |
| FDD | Panasonic JU-256A-046P C | Panasonic JU-256A-046P C |
| Daughter Board | S58M USB/Audio board |  |
| CD-ROM | AOPEN 48X CD-948E/AKU F |  |
| Modem card | Askey 56K Modem 1456VQH20E | Askey 56K Modem 1456VQH20E |
| Key Board | API 6511-UA41/E | API 6511-UA41/E |
| CDRW |  | SONY CRX140E (8X4X32) |
| DVD-ROM |  | HITACHI 8X GD-7000 |
| VGA card |  | LEADTEK Winfast Geforce 256 32MB SDR+TV Out + DVI |
| LAN card |  | 3com 3C905C-TXM |
| Mouse | Primax USB 3D, MOSXU | Primax USB 3D, MOSXK |
| SPS | DELTA 5V/3A DPS-200PB-82 | Hipower 200W |
| Housing | H80 | H80 |

| Brand / model | SKU C (507) | SKU D |
| --- | --- | --- |
| Main Board | S58M-1 | S58M-1 |
| CPU | P iii 866 MHz/133MHz | P iii 500MHz |
| Heat-sink |  |  |
| Memory | Infineon 128MB 0.19u /PC-133 | Infineon 128MB 0.19u /PC-100 |
| System chipset | Intel F82815 | Intel F82815 |
| HDD | Seagate Cuda II ST320430A (20.4GB) | Seagate U10 20.4GB (ST320423A) |
| FDD | Panasonic JU-256A-046P C | Panasonic JU-256A-046P C |
| CD-ROM |  | AOPEN 48X CD-948E/AKU F |
| CDRW | LG 8X4X32X VED-8080 IBE |  |
| DVD-ROM | HITACHI 8X GD-7000 |  |
| VGA card | LEADTEK Winfast 3D S325 (M64) 32MB | ATI RAGE 128/XPERT99 |
| LAN | NIC-559A | ALN 325 W/8139B-F |
| Modem card | Ambit T62M154.00 | Ambit T62M154.01 |
| Daughter Board | S58M USB/Audio board |  |
| Key Board | API 6511-UA41/E | API 6511-UA41/E |
| Mouse | Logitech U48A | Primax 2D, MUSXJ |
| SPS | DELTA 5V/3A DPS-200PB-82 | Hipower 200W |
| Housing | H80 | H80 |

# 1. Acceptance Criteria of C3 Entry

- (a)	 Test Equipment:

1. Blank FDD

2. VCD

3. QAFE test program

- (b) Test Criteria:

1. Function check:

No any major problem allowed in each function check; each minor problem should be scheduled to

be cleared in limited time period.

2. Appearance/mechanical function check:

No major mechanical function problem (e.g. Floppy disk locked in FDD drive, CDROM tray

can not be ejected, etc.) , no obvious assembly gap (0.8mm), no uniformity problem, no unit

wobbles (0.5mm)

Test Result:

1. Function check

|  | Test Item | Results |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SKU A | SKU B | SKU C | SKU D |
| 1 | Power on, Enter and Exit Windows 98 and power off for 5 times | PASS | PASS | PASS | PASS |
| 2 | Check device manager of system property in control panel | PASS | PASS | PASS | PASS |
| 3 | QAFE quick check (use battery only) | PASS | PASS | PASS | PASS |
| 4 | Scorpion I test program long run test | PASS | PASS | PASS | PASS |

Table 1-1

2. Appearance/mechanical function check:

|  | Test Item | Results |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SKU A | SKU B | SKU C | SKU D |
| 1 | Check all doors and covers mechanical function | PASS | PASS | PASS | PASS |
| 2 | Check all switches and buttons mechanical function | PASS | PASS | PASS | PASS |
| 3 | Check if there are units wobbles (moving distance is more than 0.5mm) | PASS | PASS | PASS | PASS |
| 4 | Check if the assembly gap is more than 0.8mm | PASS | PASS | PASS | PASS |
| 5 | Check if there are uniformity problems | PASS | PASS | PASS | PASS |

Table 1-2

# 2. Voltage Dips & Short Interruptions Immunity Test

(a) Test Equipment:

1. Pacific Power Source 140TMX

2. CHROMA programmable AC Source, Model: 6120

3. Other relevant equipment

(b) Test Specifications:

UT: The lower and upper limit of input AC voltage of the SPS (e.g. if input range of the adapter is 100Vac to 240Vac, then UT = 100Vac and 240Vac)

| Condition | Test level (% of UT) | Duration (ms) | Number of disturbance | Polarity | Time interval between shoots |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 5000 | 3 | 90 and 0, 180 | Depends on the time to enter windows environment |
| 2 | 40 | 100 | 3 | 90 and 0, 180 | Depends on the time to enter windows environment |
| 3 | 70 | 10 | 6 | 90 and 0, 180 | 10s |

- Table 2-1

- Note: UT stands for the test nominal voltage.

(e) Test Criteria:

1. Adapter only

| Condition | Performance criterion (permitted) |
| --- | --- |
| 1 | C |
| 2 | C |
| 3 | B |

- Table 2-2

Test Result:

A. UT: the lower limit of input voltage of the SPS

| Test level % UT | Voltage dip and short interruptions % UT | Duration | Performance Criteria | Phase | Result |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SKU B | SKU C |
| 0 | 100 | 5000 ms | C | 0 | Pass | Pass |
|  |  |  |  | 90 | Pass | Pass |
|  |  |  |  | 180 | Pass | Pass |
| 40 | 60 | 100 ms | C | 0 | Pass | Pass |
|  |  |  |  | 90 | Pass | Pass |
|  |  |  |  | 180 | Pass | Pass |
| 70 | 30 | 10 ms | B | 0 | Pass | Pass |
|  |  |  |  | 90 | Pass | Pass |
|  |  |  |  | 180 | Pass | Pass |

Table 2-3

B. UT: the upper limit of input voltage of the SPS

| Test level % UT | Voltage dip and short interruptions % UT |  |  | Duration | Performance Criteria | Phase | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SKU B | SKU C |
| 0 |  |  | 100 | 5000 ms | C | 0 | Pass | Pass |
|  |  |  |  |  |  | 90 | Pass | Pass |
|  |  |  |  |  |  | 180 | Pass | Pass |
| 40 |  | 60 |  | 100 ms | C | 0 | Pass | Pass |
|  |  |  |  |  |  | 90 | Pass | Pass |
|  |  |  |  |  |  | 180 | Pass | Pass |
| 70 |  | 30 |  | 10 ms | B | 0 | Pass | Pass |
|  |  |  |  |  |  | 90 | Pass | Pass |
|  |  |  |  |  |  | 180 | Pass | Pass |

Table 2-4

# 3. Electrical Fast Transient/Burst Immunity Test

(a) Test Specifications:

.1. Input AC power ports

- Rise time of one pulse			: 5ns

- Impulse duration			: 50ns

- Repetition frequency of the impulses	: 5kHz

- Charge voltage			: +/- 1kV

- Duration of the test for each condition	: 1 minutes

- Ports to be tested			: L & N

.2. Ports for signal lines and control lines (if modem or network adapter module is built in the system)

- Rise time of one pulse			: 5ns

- Impulse duration			: 50ns

- Repetition frequency of the impulses	: 5kHz

- Charge voltage			: +/- 0.5kV

- Duration of the test for each condition	: 1 minutes

Note: Capacitive coupling clamp used

(b) Test Criteria:

1.Test on AC power port:

| Error Level / Testing voltage | 500V | 750V | 1000V | 1250V |
| --- | --- | --- | --- | --- |
| Performance criteria permitted | A | A | B | Margin Test for reference |

Table 3-1

2.Test on signal line or control line:

| Error Level / Testing voltage | 250V | 500V | 750V |
| --- | --- | --- | --- |
| Performance criteria permitted | A | B | Margin Test for reference |

Table 3-2

Test Results

- Input AC power ports

| Order | Test Point |  | Burst applied |  | Performance during test |  | Results |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SKU B |  | SKU C |
| 1 | L |  | + 500 V |  | A |  |  | Pass | Pass |
| 2 | N |  | + 500 V |  | A |  |  | Pass | Pass |
| 3 | L |  | - 500 V |  | A |  |  | Pass | Pass |
| 4 | N |  | - 500 V |  | A |  |  | Pass | Pass |
| 5 | L |  | + 750 V |  | A |  |  | Pass | Pass |
| 6 | N |  | + 750 V |  | A |  |  | Pass | Pass |
| 7 | L |  | - 750 V |  | A |  |  | Pass | Pass |
| 8 | N |  | - 750 V |  | A |  |  | Pass | Pass |
| 9 | L |  | + 1000 V |  | B |  |  | Pass | Pass |
| 10 | N |  | + 1000 V |  | B |  |  | Pass | Pass |
| 11 | L |  | - 1000 V |  | B |  |  | Pass | Pass |
| 12 | N |  | - 1000 V |  | B |  |  | Pass | Pass |
| 13 | L (for reference) |  | + 1250 V |  | B |  |  | Pass | Pass |
| 14 | N (for reference) |  | + 1250 V |  | B |  |  | Pass | Pass |
| 15 |  | L (for reference) |  | - 1250 V | B | Pass |  |  | Pass |
| 16 |  | N (for reference) |  | - 1250 V | B | Pass |  |  | Pass |

Table 3-3

Note: Burst applied to L and N is asynchronous

2. Signal line or control line

| Order | Test Point | Burst applied | Performance during test | Results |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | SKU B | SKU C |
| 1 | Modem line | + 250 V | A | Pass | Pass |
| 2 | Modem line | - 250 V | A | Pass | Pass |
| 3 | Modem line | + 500 V | B | Pass | Pass |
| 4 | Modem line | - 500 V | B | Pass | Pass |
| 5 | Modem line (for reference) | + 750 V | B | Pass | Pass |
| 6 | Modem line (for reference) | - 750 V | B | Pass | Pass |
| 7 | Network line | + 250 V | A | Pass | Pass |
| 8 | Network line | - 250 V | A | Pass | Pass |
| 9 | Network line | + 500 V | B | Pass | Pass |
| 10 | Network line | - 500 V | B | Pass | Pass |
| 11 | Network line (for reference) | + 750 V | B | Pass | Pass |
| 12 | Network line (for reference) | - 750 V | B | Pass | Pass |

Table 3-4

# 4. Surge Immunity Test

(a) Test Specification

| Test Mode | Test Voltage | Polarity | Phase Shifting | No. of surges | Repetition Rate |
| --- | --- | --- | --- | --- | --- |
| Line to line (differential mode) | 1kV | Positive & negative | 0, 90 and 270 | 5 | 1 surge/minute |
| Line to ground (common mode) | 2kV | Positive & negative | 0, 90 and 270 | 5 | 1 surge/minute |

Table 4-1

Note: Lower voltage levels must pass the test too.

(b) Test Criteria:

1. AC power port: Line to Line

| Error Level \Testing voltage | 500V | 750V | 1000V | 1250V |
| --- | --- | --- | --- | --- |
| Performance criteria permitted | A | A | B | Margin Test for reference |

Table 4-2

2. AC power port: Line to ground

| Error Level \Testing voltage | 1000V | 1500V | 2000V | 2250V |
| --- | --- | --- | --- | --- |
| Performance criteria permitted | A | A | B | Margin Test for reference |

Table 3-3

Test Result:

1. Line to Line (No. of surges applied in each sub-item: 5)

| Item | Mode |  | Phase |  | Impulse Voltage |  | Performance | Result |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SKU B | SKU C |  |
| 1 | L to N | 0 |  | 500V |  | A |  | Pass |  | Pass |
| 2 | L to N | 90 |  | 500V |  | A |  | Pass |  | Pass |
| 3 | L to N | 270 |  | 500V |  | A |  | Pass |  | Pass |
| 4 | L to N | 0 |  | -500V |  | A |  | Pass |  | Pass |
| 5 | L to N | 90 |  | -500V |  | A |  | Pass |  | Pass |
| 6 | L to N | 270 |  | -500V |  | A |  | Pass |  | Pass |
| 7 | L to N | 0 |  | 750V |  | A |  | Pass |  | Pass |
| 8 | L to N | 90 |  | 750V |  | A |  | Pass |  | Pass |
| 9 | L to N | 270 |  | 750V |  | A |  | Pass |  | Pass |
| 10 | L to N | 0 |  | -750V |  | A |  | Pass |  | Pass |
| 11 | L to N | 90 |  | -750V |  | A |  | Pass |  | Pass |
| 12 | L to N | 270 |  | -750V |  | A |  | Pass |  | Pass |
| 13 | L to N | 0 |  | 1000V |  | B |  | Pass |  | Pass |
| 14 | L to N | 90 |  | 1000V |  | B |  | Pass |  | Pass |
| 15 | L to N | 270 |  | 1000V |  | B |  | Pass |  | Pass |
| 16 | L to N | 0 |  | -1000V |  | B |  | Pass |  | Pass |
| 17 | L to N | 90 |  | -1000V |  | B |  | Pass |  | Pass |
| 18 | L to N | 270 |  | -1000V |  | B |  | Pass |  | Pass |
| 19 | L to N | 0 |  | 1250V |  | B |  | Pass |  | Pass |
| 20 | L to N | 90 |  | 1250V |  | B |  | Pass |  | Pass |
| 21 | L to N | 270 |  | 1250V |  | B |  | Pass |  | Pass |
| 22 | L to N | 0 |  | -1250V |  | B |  | Pass |  | Pass |
| 23 | L to N | 90 |  | -1250V |  | B |  | Pass |  | Pass |
| 24 | L to N | 270 |  | -1250V |  | B |  | Pass |  | Pass |

Table 4-5

2. Line to PE: (No. of surges applied in each sub-item: 5; in case there is safety ground of the system)

| Item |  | Mode |  | Phase |  | Impulse Voltage |  | Performance |  | Result |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | SKU B |  | SKU C |
| 1 | L to PE |  | 0 |  | 1000V |  | A |  | Pass |  | Pass |  |
| 2 | L to PE |  | 90 |  | 1000V |  | A |  | Pass |  | Pass |  |
| 3 | L to PE |  | 270 |  | 1000V |  | A |  | Pass |  | Pass |  |
| 4 | N to PE |  | 0 |  | 1000V |  | A |  | Pass |  | Pass |  |
| 5 | N to PE |  | 90 |  | 1000V |  | A |  | Pass |  | Pass |  |
| 6 | N to PE |  | 270 |  | 1000V |  | A |  | Pass |  | Pass |  |
| 7 | L to PE |  | 0 |  | -1000V |  | A |  | Pass |  | Pass |  |
| 8 | L to PE |  | 90 |  | -1000V |  | A |  | Pass |  | Pass |  |
| 9 | L to PE |  | 270 |  | -1000V |  | A |  | Pass |  | Pass |  |
| 10 | N to PE |  | 0 |  | -1000V |  | A |  | Pass |  | Pass |  |
| 11 | N to PE |  | 90 |  | -1000V |  | A |  | Pass |  | Pass |  |
| 12 | N to PE |  | 270 |  | -1000V |  | A |  | Pass |  | Pass |  |
| 13 | L to PE |  | 0 |  | 1500V |  | A |  | Pass |  | Pass |  |
| 14 | L to PE |  | 90 |  | 1500V |  | A |  | Pass |  | Pass |  |
| 15 | L to PE |  | 270 |  | 1500V |  | A |  | Pass |  | Pass |  |
| 16 | N to PE |  | 0 |  | 1500V |  | A |  | Pass |  | Pass |  |
| 17 | N to PE |  | 90 |  | 1500V |  | A |  | Pass |  | Pass |  |
| 18 | N to PE |  | 270 |  | 1500V |  | A |  | Pass |  | Pass |  |
| 19 | L to PE |  | 0 |  | -1500V |  | A |  | Pass |  | Pass |  |
| 20 | L to PE |  | 90 |  | -1500V |  | A |  | Pass |  | Pass |  |
| 21 | L to PE |  | 270 |  | -1500V |  | A |  | Pass |  | Pass |  |
| 22 | N to PE |  | 0 |  | -1500V |  | A |  | Pass |  | Pass |  |
| 23 | N to PE |  | 90 |  | -1500V |  | A |  | Pass |  | Pass |  |
| 24 | N to PE |  | 270 |  | -1500V |  | A |  | Pass |  | Pass |  |
| 25 | L to PE |  | 0 |  | 2000V |  | B |  | Pass |  | Pass |  |
| 26 | L to PE |  | 90 |  | 2000V |  | B |  | Pass |  | Pass |  |
| 27 | L to PE |  | 270 |  | 2000V |  | B |  | Pass |  | Pass |  |
| 28 | N to PE |  | 0 |  | 2000V |  | B |  | Pass |  | Pass |  |
| 29 | N to PE |  | 90 |  | 2000V |  | B |  | Pass |  | Pass |  |
| 30 | N to PE |  | 270 |  | 2000V |  | B |  | Pass |  | Pass |  |
| 31 | L to PE |  | 0 |  | -2000V |  | B |  | Pass |  | Pass |  |
| 32 | L to PE |  | 90 |  | -2000V |  | B |  | Pass |  | Pass |  |
| 33 | L to PE |  | 270 |  | -2000V |  | B |  | Pass |  | Pass |  |
| 34 | N to PE |  | 0 |  | -2000V |  | B |  | Pass |  | Pass |  |
| 35 | N to PE |  | 90 |  | -2000V |  | B |  | Pass |  | Pass |  |
| 36 | N to PE |  | 270 |  | -2000V |  | B |  | Pass |  | Pass |  |
| 37 | L to PE |  | 0 |  | 2250V |  | B |  | Pass |  | Pass |  |
| 38 | L to PE |  | 90 |  | 2250V |  | B |  | Pass |  | Pass |  |
| 39 | L to PE |  | 270 |  | 2250V |  | B |  | Pass |  | Pass |  |
| 40 | N to PE |  | 0 |  | 2250V |  | B |  | Pass |  | Pass |  |
| 41 | N to PE |  | 90 |  | 2250V |  | B |  | Pass |  | Pass |  |
| 42 | N to PE |  | 270 |  | 2250V |  | B |  | Pass |  | Pass |  |

| 43 | L to PE | 0 | -2250V | B | Pass | Pass |
| --- | --- | --- | --- | --- | --- | --- |
| 44 | L to PE | 90 | -2250V | B | Pass | Pass |
| 45 | L to PE | 270 | -2250V | B | Pass | Pass |
| 46 | N to PE | 0 | -2250V | B | Pass | Pass |
| 47 | N to PE | 90 | -2250V | B | Pass | Pass |
| 48 | N to PE | 270 | -2250V | B | Pass | Pass |

Table 4-6

# 5. Electrostatic Discharge Susceptibility Test

Test Specifications and Criteria

1.	Climatic conditions shall be within the following ranges: (IEC 1000-4-2)

- Ambient Temperature: 15C ~ 35C

- Relative Humidity: 30% ~ 60%

- Atmospheric Pressure: 86kPa to 106kPa

2.

|  |  | ESD Voltage Level |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acceptance Criteria | Air Discharge |  | Contact Discharge | Air Discharge | Contact Discharge | Air Discharge | Contact Discharge |
|  | 4 kV |  | 4 kV | 8 kV | 6 kV | 15 kV | 8 kV |
| B |  | Not Permitted |  | Permitted |  | Permitted |  |
| C |  | Not Permitted |  | Not permitted |  | Permitted |  |
| D |  | Not Permitted |  | Not Permitted |  | Not Permitted |  |
| Remark |  | Note (1), (2) |  |  |  | Note (3) |  |

Table 5-1

Note: 	(1): It is a minimum spec. of the add-on card which built in a PC system under an operation

mode.

(2): Following the spec also if the speakers with finger guard are exposed.

(3): It is a minimum spec. of an operating PC whose add-on card under the idle mode.

3. At least 10 single discharges shall be applied to a pre-selected point

4. Time interval between successive single discharge: 1 second (standard) or 3 seconds.

Fig-5-1 ESD test point of System

Test Report:

|  | ESD Voltage Level |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test results & Performance | Air Discharge |  |  |  |  |  |  |  |  |  |  |  |
| Criteria | Voltage Level |  | Performance Level |  | Voltage Level |  | Performance Level |  | Voltage Level |  | Performance Level |  |
| Test Location | +4kV |  | A/B/C/D |  | +8kV |  | A/B/C/D |  | +15kV |  | A/B/C/D |  |
| LED indicators |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Power Switch |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Aperture around CDROM |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Aperture around CDRW |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Aperture around DVD-ROM |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Aperture around FDD |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Upper case |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Aperture around USB ports |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Aperture around PS/2 ports |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Serial port connector hoods |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Screws of SPS |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Port Brackets |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Aperture around Audio input/output jacks |  | Pass |  | A |  | Pass |  | B |  | Pass |  |  |
| Parallel port connector hoods |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Game port Connector |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| VGA connector of VGA Card |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Port bracket of LAN card |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Outer screws |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Right and Left side door |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |
| Front side USB/ Audio Ports |  | Pass |  | A |  | Pass |  | B |  | Pass |  | C |

Table 5-2

|  |  | ESD Voltage Level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test results & Performance Criteria |  | Contact Discharge |  |  |  |  |  |  |  |  |  |
|  |  | Voltage Level |  | Performance Level |  | Voltage Level | Performance Level |  | Voltage Level |  | Performance Level |
| Test Location |  | +4kV |  | A/B/C/D |  | +6kV | A/B/C/D |  | +8kV |  | A/B/C/D |
| Upper case |  | Pass |  |  | A | Pass | B |  | Pass |  | C |
| Serial port connector hoods |  | Pass |  |  | A | Pass | B |  | Pass |  | C |
| Serial port connector hoods |  | Pass |  |  | A | Pass | B |  | Pass |  | C |
| Screws of SPS |  | Pass |  |  | A | Pass | B |  | Pass |  | C |
| Port Brackets |  | Pass |  |  | A | Pass | B |  | Pass |  | C |
| Parallel port connector hoods | Pass |  | A |  |  | Pass | B | Pass |  | C |  |
| Game port Connector | Pass |  | A |  |  | Pass | B | Pass |  | C |  |
| VGA connector of VGA Card | Pass |  | A |  |  | Pass | B | Pass |  | C |  |
| Port bracket of LAN card | Pass |  | A |  |  | Pass | B | Pass |  | C |  |

| Outer screws | Pass | A | Pass | B | Pass | C |
| --- | --- | --- | --- | --- | --- | --- |
| Right and Left side door | Pass | A | Pass | B | Pass | C |

Table 5-3

# 6. DC Margin Test

(a) Test Specifications

| DC Voltage | Requirement |
| --- | --- |
| 5V+/-5% & 3.3V+/-5% | No any error |

Table 6-1

Note:	(1) The test specifications of DC voltages depend on the component (DC/DC converter)

specifications of the product (Use Product External Specifications as basis).

(2) Temperature chamber shall be simulate in no air flow environment.

(b) Test Criteria:

DC power change (+/- 5%) with 25C environment: performance criteria A must be me

Test Result:

1. Function Test

(a) test environment

| DC Voltage |  | Scorpion Test Result |  |  |
| --- | --- | --- | --- | --- |
|  |  | SKU A |  | SKU B |
| 5V -5% & 3.3V +5% | Pass |  | Pass |  |
| 5V -5% & 3.3V -5% | Pass |  | Pass |  |
| 5V +5% & 3.3V +5% | Pass |  | Pass |  |
| 5V +5% & 3.3V -5% | Pass |  | Pass |  |

Table 6-2

Note: The DC voltage depends on the component’s specification.

2. Current consumption test:

The current consumption of the EUT(M.B. only) for each applied voltage are:

SKU A:

| Current Consumption Each Voltage Setting |  |  |  |
| --- | --- | --- | --- |
| Condition | + 5 V | + 3.3 V | + 12 V |
| Operating | 2.96~3.2A | 1.89~2.04A | 0.112~0.321A |
| Idle | 0.978~1.02A | 1.74A | 0.112~0.321A |

Table 6-3

SKU C:

| Current Consumption Each Voltage Setting |  |  |  |
| --- | --- | --- | --- |
| Condition | + 5 V | + 3.3 V | + 12 V |
| Operating | 4.21~5.2A | 2.07~2.31A | 0.112~0.321A |
| Idle | 1.678~4.182A | 1.98~2.07A | 0.112~0.321A |

Table 6-4

# 7. Line Voltage and Frequency Test

(a) Test Specifications

1. Function Test I

Voltage		100V/120V (-/+ 10%)			: 90~132V

200V/240V (-/+ 10%)			: 180~264V

Frequency		50Hz (-/+ 3Hz)				: 47~53Hz

60Hz (-/+ 3Hz)				: 57~63Hz

Voltage variation (frequency is fixed to 50Hz or 60Hz)		: 90V to 264V and 264V to 90V

Frequency variation (voltage is fixed to 115V or 230V)	: 47Hz to 63Hz and 63Hz to 47Hz

(b) Test Criteria:

Performance Criteria A must be met for both test conditions

| Condition | Voltage & Frequency | Time (minutes) | Condition | Voltage & Frequency | Time (minutes) |
| --- | --- | --- | --- | --- | --- |
| 1 | 90V / 47Hz | 15 | 9 | 180V / 47Hz | 15 |
| 2 | 90V / 53Hz | 15 | 10 | 180V / 53Hz | 15 |
| 3 | 90V / 57Hz | 15 | 11 | 180V / 57Hz | 15 |
| 4 | 90V / 63Hz | 15 | 12 | 180V / 63Hz | 15 |
| 5 | 132V / 47Hz | 15 | 13 | 264V / 47Hz | 15 |
| 6 | 132V / 53Hz | 15 | 14 | 264V / 53Hz | 15 |
| 7 | 132V / 57Hz | 15 | 15 | 264V / 57Hz | 15 |
| 8 | 132V / 63Hz | 15 | 16 | 264V / 63Hz | 15 |

Table 7-1

Test Results:

7-1. Fixed Frequency and Voltage test

| Frequency | 50Hz |  |  |  | 60Hz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 47 Hz |  | 53 Hz |  | 57 Hz |  | 63 Hz |  |
| Voltage ( V ) | SKU A | SKU B | SKU A | SKU B | SKU A | SKU B | SKU A | SKU B |
| 90 | Pass | Pass | Pass | Pass | Pass | Pass | Pass | Pass |
| 132 | Pass | Pass | Pass | Pass | Pass | Pass | Pass | Pass |
| 180 | Pass | Pass | Pass | Pass | Pass | Pass | Pass | Pass |
| 264 | Pass | Pass | Pass | Pass | Pass | Pass | Pass | Pass |

Table7-2

7-2. Variation Frequency and Voltage test

7-2-1. SKU A:

| Voltage |  |  | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency(Hz) | 90V~264V | 264V~90V | Voltage (V) | 47Hz~63Hz | 63Hz~47Hz |
| 50Hz | Pass | Pass | 115Vac | Pass | Pass |
| 60Hz | Pass | Pass | 230Vac | Pass | Pass |

Table7-3

7-2-2. SKU B:

| Voltage |  |  | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency(Hz) | 90V~264V | 264V~90V | Voltage (V) | 47Hz~63Hz | 63Hz~47Hz |
| 50Hz | Pass | Pass | 115Vac | Pass | Pass |
| 60Hz | Pass | Pass | 230Vac | Pass | Pass |

Table7-4

# 8. Power ON/OFF Characteristics

(f)	Test Specifications

(1) Basic Function Test:

System ON Time 		: Booting OK in MS-DOS mode

System OFF Time		: 20 seconds

ON/OFF Cycle		: 1000 times

2)	Margin Test:

System ON/OFF	time	: Less than 1 second

ON/OFF Cycle 		: 100 times

(g) Test Criteria:

1. Basic function Test:

(1) Successfully boot to MS-DOS mode at each power on/off cycle

(2) No unacceptable RTC data loss

(3) No physical damage after test

(4) Function check OK using QAFE "quick check"

2. Margin Test:

(1) No unacceptable RTC data loss

(2) No damage or safety problem after test

(3) Function check OK using QAFE "quick check"

Figure 8-1 Power ON and OFF test curve for basic function test

2.	Margin Test:

Figure 8-2 Power ON and OFF test curve for margin test

Test Results:

| POWER ON/OFF Test |  | Result |  |
| --- | --- | --- | --- |
|  |  | SKU C | SKU D |
| Function Test | Boot successfully each cycle | Pass | Pass |
|  | No RTC data loss | Pass | Pass |
|  | No damage problem after test | Pass | Pass |
|  | QAFE quick check | Pass | Pass |
| Margin Test | No RTC data loss | Pass | Pass |
|  | No damage problem after test | Pass | Pass |
|  | QAFE quick check | Pass | Pass |

Table 8-1

# 9. Temperature/Thermal Profile Test

(a) Field of Application:

1. New notebook system

2. CPU, Chipsets, VGA, Audio, Modem, etc. chip model change

3. Battery pack model change

4. CDROM/DVD-ROM model change

5. FDD/LS-120/ZIP model change

6. HDD model change

7. System fan model change

8. Heat plate (sink) thickness, shape and  model change

(b) Test Criteria:

1. Component on main board

(1) Major test points

| No. | Chip Name | Point | Tc(max)  C |
| --- | --- | --- | --- |
| 1 | CPU | Center of Surface | refer to databook |
| 2 | POWER SUPPLY CONTROLLERS | Center of Surface | 105 or refer to databook |
| 3 | VGA CHIP | Center of Surface | 75 or refer to databook |
| 4 | SOUND CHIP | Center of Surface | 75 or refer to databook |
| 5 | DRAM/SRAM | Center of Surface | 75 or refer to databook |
| 6 | CHIPSETS | Center of Surface | 75 or refer to databook |
| 7 | MODEM CHIP | Center of Surface | 75 or refer to databook |
| 8 | Other chips whose temperature is relative high according to the survey of IR camera. | Center of Surface | 75 or refer to databook |

Table 9-1

Test Result:

Ambient temperature in test room: 26.8C

Attach the picture of heat plate or heat pipes here

1.  Components on main board or other subsystem board using Tc(max) test criteria

SKU A:

| COMPONENTS ON MAIN BOARD OR SUBSYSTEM BOARDS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Component location number(Location number of board name) | Component Name (Vendor name; model number) | Tc measured (C) | 35C conversion temperature | Tc(max) (C) | Result |
| CPU | Piii 733/133MHz | 62.0 | 71.0 | 80 | Pass |
| Q1 of M.B. | 50N03LT | 73.6 | 82.6 | 125 | Pass |
| Q2of M.B. | 50N03LT | 63.92 | 72.92 | 125 | Pass |
| U5 of M.B. | L951XA | 49.68 | 58.68 | 75 | Pass |
| U13 of M.B. | SMSC 74210 | 46.16 | 55.16 | 75 | Pass |
| U10 of M.B. | FW82815 | 54.8 | 63.8 | 75 | Pass |
| U12 of M.B. | ST75185C | 48.96 | 57.96 | 75 | Pass |
| U19 of M.B. | INTEL 938SE | 39.20 | 48.20 | 75 | Pass |
| U17 of M.B. | AD1881 | 46.48 | 55.48 | 75 | Pass |
| VR1 of M.B. | 78M05 | 46.88 | 55.88 | 75 | Pass |
| Q13 of M.B. | PHD50N03 | 42.0 | 51.0 | 125 | Pass |
| Q9 of M.B. | PHD50N03 | 46.32 | 55.32 | 125 | Pass |

| COMPONENTS ON MAIN BOARD OR SUBSYSTEM BOARDS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Component location number(Location number of board name) | Component Name (Vendor name; model number) | Tc measured (C) | 35C conversion temperature | Tc(max) (C) | Result |
| Q5 of M.B. | PHD50N03 | 43.2 | 52.2 | 125 | Pass |
| Q8 of M.B. | PHD50N03 | 45.28 | 54.28 | 125 | Pass |
| Q4 of M.B. | EZ1117CM | 63.92 | 72.92 | 125 | Pass |
| U6 of M.B. | ICS9250B | 72.8 | 81.8 | 85 | Pass |

Table 9-2

SKU C:

| COMPONENTS ON MAIN BOARD OR SUBSYSTEM BOARDS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Component location number(Location number of board name) | Component Name (Vendor name; model number) | Tc measured (C) | 35C conversion temperature | Tc(max) (C) | Result |
| CPU | Piii 866/133MHz | 59.0 | 65.0 | 80 | Pass |
| U10 of M.B. | FW82815 | 53.2 | 58.2 | 75 | Pass |
| U7 of M.B. | SC1164CSW | 46.56 | 64.84 | 75 | Pass |
| U6 of M.B. | 9250BF-28 | 64.56 | 69.56 | 85 | Pass |
| Q1 of M.B. | 50N03LT | 63.84 | 68.84 | 125 | Pass |
| Q2 of M.B. | 50N03LT | 59.44 | 64.44 | 125 | Pass |
| L1 of M.B. | COIL | 51.44 | 56.44 | 125 | Pass |
| TC3 of M.B. | 1000u/16V | 48.08 | 53.08 | 105 | Pass |

Note: 
1.This table shows only the components whose temperature rises above 50C.
2. Tj = Tc  + Pd * JC  or  Tj = TA  + Pd * JA  (Tc - 5) + Pd * JA

# 10. Operating Temperature and Humidity Test

(a) Test Specifications

Low temperature			:  0C / humidity not controlled

High temperature & low humidity	: +40C / 20% R.H.

High temperature & high humidity	: +40C/ 80% R.H.

Notes:

1) Temperature and humidity gradient should be less than 20C /hour and 20% R.H. /hour,

respectively, to prevent condensation.

2) Above humidity is non-condensing

3) Provided that the maximum wet-bulb temperature is 32 C at high temperature and high

humidity condition.

Figure 10-1 Operating temperature/humidity cycling test curve

.

(b) Test Criteria:

1. Performance criteria A must be met during and after the test

2. During the test, temperature margin (5C) must be added if number of  test samples is less than

two.

Test Results:

| Test Item |  | Cold start test | On-Off Test at low temperature | On-Off Test at high temperature | Function Test | Physical Check |
| --- | --- | --- | --- | --- | --- | --- |
|  | SKU A | Pass | Pass | Pass | Pass | Pass |
| Operating | SKU B | Pass | Pass | Pass | Pass | Pass |
|  | SKU C | Pass | Pass | Pass | Pass | Pass |
|  | SKU D | Pass | Pass | Pass | Pass | Pass |

Table 10-1

# 11. Non-operating Temperature and Humidity Test

(a) Test Specifications

A. Un-packed

Low temperature				: -10C / humidity not controlled

High temperature & low humidity		: +60C / 20% R.H.

High temperature & high humidity		: +60C/ 80% R.H.

B. Storage Package

Low temperature				: -20C / humidity not controlled

High temperature & low humidity		: +60C / 20% R.H.

High temperature & high humidity		: +60C/ 90% R.H.

Notes:

1. Temperature gradient should be less than 20C /hour and 20%RH/hour to prevent condensation.

2. Above humidity is non-condensing

3. Provided that the maximum wet-bulb temperature is 32 C at high temperature and high humidity condition.

(b) Test Criteria:

A. Un-packed

1. Performance criteria A must be met after the test

2. Appearance and mechanical function

- No dent.

- No crack

- No mold speckle

- No unclear letter, broken line

- No easily removed button

- Input and output terminals' connection must be easily matched

- No units wobbles

- No migration materials in the set's plinths (e.g. foot of EUT dropped off)

- No letters or symbols on cabinet are missing

- No transfigure, deformity

- No stiffness or squeak for buttons when pressed

- No warp or rust for metal parts

- No slack (e.g. screw loose)

B. Storage Package

1. Performance criteria A must be met after the test

2. Appearance and mechanical function

- No scratch

- No dent.

- No crack

- No mold speckle

- No unclear letter, broken line

- No easily removed button

- No obvious fingerprint to degrade the quality image of the product.

- Input and output terminals' connection must be easily matched

- No units wobbles

- No migration materials in the set's plinths (e.g. foot of EUT dropped off)

- No letters or symbols on cabinet are missing

- No transfigure, deformity

- No stiffness or squeak for buttons when pressed

- No warp or rust for metal parts

- No slack (e.g. screw loose)

- No cracks or severe damage to cushion

- No server damage or crack to carton box

(c) Test Software

QAFE quick check

Figure 11-1 Non operating temperature/humidity test curve (unpacked)

Figure 11-2 Storage's package temperature/humidity test curve.

Test Results:

| Test Item |  | QAFE Quick Check | Appearance/mechanical check | Package Check |
| --- | --- | --- | --- | --- |
|  | SKU A | Pass | Pass | Pass |
|  | SKU B | Pass | Pass | Pass |
| Unpacked | SKU C | Pass | Pass | Pass |
|  | SKU D | Pass | Pass | Pass |
|  | SKU A | Pass | Pass | Pass |
|  | SKU B | Pass | Pass | Pass |
| Packed | SKU C | Pass | Pass | Pass |
|  | SKU D | Pass | Pass | Pass |

Table 11-2

# 12. Operating Vibration Test

(a) Test Specifications:

The specifications below are applicable to most products. Some products, due to their peculiar

design, may require different tests. These limits must be determined by the responsible design

engineer.

| Frequency Hertz | Displacement (peak-peak) Or Acceleration | Sweep Rate | Endurance Cycle |
| --- | --- | --- | --- |
| 5 ~ 16.11 | 0.38 mm (0.015”) | 1 Octave/Minute | 1 Cycles ( 2 sweeps ) |
| 16.11 ~ 250 | 0.2 G |  |  |

Table 12-1

(b) Test Criteria:

During and after the test:

1. Function:

Performance criteria A, B or minor C shall be met

2. Appearance & mechanical function

The EUT should not suffer permanent deformation or fracture. No fixed part or assembly

shall be loosen. No moving or movable part of an assembly should become free or sluggish

in the testing process. No movable part should shift in setting, position or adjustment.

(c) Supplementary Information:

1. Formula of Sine Mode Vibration:

G=0.0511*Af2 or 2.013*Df2

Where 	G: the acceleration (1G=9.8 m/s2)

A: inches (peak to peak)

f: frequency (Hz)

D: meters (peak to peak)

So, when vibration amplitude is 0.015" and vibration frequency is 20Hz, the calculated

acceleration is 0.0511*0.015*20*20 = 0.3066G; especially, when vibration amplitude is 0.015"

and vibration frequency is 25.6Hz, the calculated acceleration is 0.0511*0.015*25.6*25.6 =

0.50G

2. The number of octave for a sweep is given by:

N = Log2(f2/f1) = 3.322Log10(f2/f1)

where:	N = number of octaves

f1 = the lower frequency of sweep

f2 = the upper frequency of sweep

When 0<N<1 set N=1

So, if the logarithm sweep rate is one octave per minute, the time required to sweep from f1 to f2

or from f2 to f1 is "Log2(f2/f1)" minutes.

For the operating vibration, frequency is sweep from 5Hz to 250Hz and then from 250Hz to

5Hz in octave/minutes sweep rate, the test duration required for one cycle is Log2(250/5)*2 =

11.3 minutes.

3. Test axes: The EUT shall be excited along the three orthogonal axes, generally in x, y, z-axis

(refer to Figure 12-1).

4. Qualification Test: the qualified test is complete when all key component of the EUT pass the

overall test. When a failure occurs, the test should be stopped. The failure shall be analyzed and

then find an effective countermeasure, then the test can continue. Each component is considered

as qualified when it has been exposed to the overall test.

5. Mounting and fixturing: The product shall be mechanically connected to the vibration tester

either directly or by means of a fixture. The points/area of vibration input to the product shall to

be the same as during shipment.

6.  Single point & multi-point control method:

a) Single point method is achieved by using the signal the reference point in order to maintain

this point at the specified level. (Acceleration sensing point in control loop)

b) Multi-points is used when the deviation on the reference points are over ± 6dB from original

specified vibration level . This method can be used to control the overall EUT in the

Z

specified level. The control points must be described in the report.

Y

Figure 12-1 The three orthogonal axes of EUT Vibration Test

X

Test Results:

1. Attach the picture of mounting state here

2. Attach the acceleration curve of the component which causes "minor C" performance

3.

| Test Item | Function Test |  |  | Appearance & mechanical function Check |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SKU A | SKU B | SKU C | SKU A | SKU B | SKU C |
| X orthogonal axis | Pass | Pass | Pass | Pass | Pass | Pass |
| Y orthogonal axis | Pass | Pass | Pass | Pass | Pass | Pass |
| Z orthogonal axis | Pass | Pass | Pass | Pass | Pass | Pass |

Table 12-2

# 13. Non-operating Vibration Test

(a) Test Specifications:

1. packed/Sine Mode: (Test time: 53 minutes/axis)

| Frequency (Hertz) | Displacement (peak-peak) Or Acceleration | Sweep Rate | Endurance Cycle |
| --- | --- | --- | --- |
| 5 ~ 27.1 | 0.6G | 0.5 Octave/Minute | 2 Cycles (4 sweeps) |
| 27.1 ~ 50 | 0.016"(0.4mm) |  |  |
| 50 ~ 500 | 2.0G |  |  |

Table 13-1

(b) Test Criteria:

After the test:

1. Function:

Performance criteria A shall be met

2. Appearance & mechanical function

The EUT should not suffer permanent deformation or fracture. No fixed part or assembly

shall be loosen. No moving or movable part of an assembly should become free or sluggish

in the testing process. No movable part should shift in setting, position or adjustment.

Test Results:

1. packed vibration (Sine mode)

| Test Item | Function Test |  |  | Physical Check |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SKU A | SKU B | SKU C | SKU A | SKU B | SKU C |
| X orthogonal axis | Pass | Pass | Pass | Pass | Pass | Pass |
| Y orthogonal axis | Pass | Pass | Pass | Pass | Pass | Pass |
| Z orthogonal axis | Pass | Pass | Pass | Pass | Pass | Pass |

Table 13-2

# 14. Acoustic Noise Measurement

## Sound Pressure Level

(a) Specifications:

| Operator Position (dBA) | Idle mode |  38 | Bystander Position /LpA (dBA) | Idle mode |  38 |
| --- | --- | --- | --- | --- | --- |
|  | Operating (a)* |  42 |  | Operating (a)* |  40 |
|  | Operating (b)* |  46 |  | Operating (b)* |  44 |
|  | Operating (c)* |  46 |  | Operating (c)* |  44 |
|  | Operating (d)* |  46 |  | Operating (d)* |  44 |

Table 14-1 Acoustic Noise Specifications

(b) Test Criteria:

Meet the specifications

Attach Figure 14-1 & 14-2 here

Figure 14-1 Operator Position

Test Result:

Background Noise Level: 19.8dB

1. Operator Position

|  |  | Noise Level (dBA) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location | IDLE |  | FDD | HDD 15.3GB Seagate | CD-ROM AOPEN CD-940AKU |
| Measured | 33.7dB |  | 36.8 dB | 40.1dB | 43.4 dB |
| Spec. |  40 dB |  |  42dB |  46dB |  46 dB |
| Result | Pass |  | Pass | Pass | Pass |

Table 14-2

SKU C

|  |  | Noise Level (dBA) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location | IDLE |  | FDD | HDD 20.4GB Seagate | CD-RW Sony CRX-14E | DVD HITACHI GD-7000 |
| Measured | 35.5 dB |  | 37.5 dB | 41.7 dB | 41.5 dB | 42.4 dB |
| Spec. |  40 dB |  |  42 dB |  46 dB |  46 dB |  46 dB |
| Result | Pass |  | Pass | Pass | Pass | Pass |

Table 14-3

Figure 14-2 Bystander Position

2. Bystander Position

SKU A

|  | Noise Level ( dB ) /LpA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| location |  | IDLE | FDD | CD-ROM AOPEN CD-948E/AKU | HDD Seagate 15.3GB |
| Front |  | 30.8 dB | 34.5 dB | 38.4 dB | 37.1 dB |
| Left |  | 34.2 dB | 37.7 dB | 39.2 dB | 39.5 dB |
| Rear |  | 35.5 dB | 38.4 dB | 39.3 dB | 39.4 dB |
| Right |  | 35.2 dB | 38.8 dB | 38.3 dB | 40.0dB |
| /LpA |  | 34.27 dB | 37.64 dB | 38.82 dB | 39.13 dB |
| Spec. |  |  38 dB |  40 dB |  44 dB |  44 dB |
| Result |  | Pass | Pass | Pass | Pass |

Table 14-4

SKU C

|  | Noise Level ( dB ) /LpA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| location |  | IDLE | FDD | HDD Seagate 20.4GB | CD-RW SONY CRX-140E | DVD-ROM Hitachi GD-7000 |
| Front |  | 33.1 dB | 33.2 dB | 38.7 dB | 36.1 dB | 38.2 dB |
| Left |  | 36.8 dB | 38.0 dB | 42.3 dB | 39.3 dB | 38.8 dB |
| Rear |  | 37.9 dB | 37.7 dB | 41.3 dB | 38.4 dB | 40.0 dB |
| Right |  | 37.6 dB | 36.8 dB | 43.2 dB | 38.5 dB | 39.5 dB |
| /LpA |  | 36.71 dB | 36.78 dB | 41.67 dB | 38.23 dB | 39.18 dB |
| Spec. |  |  38 dB |  40 dB |  44 dB |  42 dB |  44 dB |
| Result |  | Pass | Pass | Pass | Pass | Pass |

Table 14-5

Note:

Lpi = Noise measured at direction I

n = Number of directions (Typically, n=4)

# 15. Transportation Drop Test

(a). Test Specifications

| Packaged Gross Weight |  | Drop Height |  |  |
| --- | --- | --- | --- | --- |
| KG | lb | cm | inch | No. of Drops |
| 0 ~ 9.1 | 0 ~ 20 | 76 | 30 | 10 |
| > 9.1 ~ 18.2 | > 20 ~ 40 | 61 | 24 | 10 |
| > 18.2 ~ 27.3 | > 40 ~ 60 | 46 | 18 | 10 |
| > 27.3 ~ 45.4 | > 60 ~ 100 | 31 | 12 | 10 |
| 10 drops : 1 corner , 3 edges and 6 surfaces |  |  |  |  |

Table 15-1

(b). Test Criteria:

1. Function: performance criteria A must be met after the test

2. Appearance & Mechanical Function after the test

- No scratch

- No dent.

- No crack

- No mold speckle

- No unclear letter, broken line

- No easily removed button

- No obvious fingerprint to degrade the quality image of the product.

- Input and output terminals' connection must be easily matched

- No units wobbles

- No migration materials in the set's plinths (e.g. foot of EUT dropped off)

- No letters or symbols on cabinet are missing

- No transfigure, deformity

- No stiffness or squeak for buttons when pressed

- No warp or rust for metal parts

- No slack (e.g. screw loose)

- No cracks to cushion

- No server damage to carton box

(c) Test Results:

| Drop Height : 61 cm | Function Test (QAFE) |  | Physical Check |  |
| --- | --- | --- | --- | --- |
|  | SKU B | SKU C | SKU B | SKU C |
| The Weakest corner of product | Pass | Pass | Pass | Pass |
| The shortest edge radiating from the corner 5-3-2 | Pass | Pass | Pass | Pass |
| The next shortest edge radiating from the corner 5-3-2 | Pass | Pass | Pass | Pass |
| The longest edge radiating from the corner 5-3-2 | Pass | Pass | Pass | Pass |
| Surface 2 | Pass | Pass | Pass | Pass |
| Surface 4 | Pass | Pass | Pass | Pass |
| Surface 5 | Pass | Pass | Pass | Pass |
| Surface 6 | Pass | Pass | Pass | Pass |
| Surface 3 | Pass | Pass | Pass | Pass |
| Surface 1 | Pass | Pass | Pass | Pass |

Table 15-2

| Check items | Result |  |
| --- | --- | --- |
|  | SKU B | SKU C |
| Off-track test | Pass | Pass |
| Package appearance | Pass | Pass |
| Cushion appearance | Pass | Pass |

Table 15-3

# 16. AC Power Consumption Measurement

(a) Referenced External Standard:

U.S. EPA energy star computers: U.S. EPA's Atmospheric Pollution Prevention Division

(b)	Test Specifications

A) Test conditions:

1.	Ambient Temperature			: 24 .3  C

2.	Input AC Voltage			: 115Vac, 60Hz and 230Vac, 50Hz

3.	Line Impedance				: less than 0.25 ohm

4. Total Harmonic Distortion (voltage)	: < 5%

B) Input AC power shall be less than 30 watts in low-power state (no battery pack installed).

(c) Test Criteria:

To Meet USA EPA energy star program for computers

a) Power management features must be activated before shipping with initial pre-set default.

b) Power management pre-set default times: 15 to 30 minutes

c) 30 Watt maximum in low-power state.

Test Result:

SKU A

| AC test condition Configuration | 115Vac, 60Hz |  | 230Vac, 50Hz |  |
| --- | --- | --- | --- | --- |
|  | (Wac) | BTU | (Wac) | BTU |
| Windows 95/98 low power state (within 30 minutes after system idle) | 55.8W | 190.3896 | 55.3W | 188.6836 |
| Scorpion I (system Operating condition) | 29.3W | 99.9716 | 29.1W | 99.2892 |

Table 16-1

SKU B

| AC test condition Configuration | 115Vac, 60Hz |  | 230Vac, 50Hz |  |
| --- | --- | --- | --- | --- |
|  | (Wac) | BTU | (Wac) | BTU |
| Scorpion I (system Operating condition) | 85.2W | 290.7024 | 84.2W | 287.2704 |
| Windows 98 low power state (within 30 minutes after system idle) | 33.8W | 115.3256 | 33.6W | 114.6432 |

Table 16-2

SKU C

| AC test condition Configuration | 115Vac, 60Hz |  | 230Vac, 50Hz |  |
| --- | --- | --- | --- | --- |
|  | (Wac) | BTU | (Wac) | BTU |
| Scorpion I (system Operating condition) | 69.3W | 236.4516 | 68.8W | 234.7456 |
| Windows 95/98 low power state (within 30 minutes after system idle) | 29.9W | 102.0188 | 29.8W | 101.6776 |

Table 16-3

Note:  1. Different test configurations including various system memory sizes, storage devices,

different Add on cards, CPUs, HDD/FDD/CD-ROM/DVD/CDRW operating status, aetc.

2. Sample 1 result is over EPA Spec. 30W